SA WG2 Meeting #S2-158	S2-2309354

21 - 25 August, 2023, Goteborg, Sweden

Title:	Reply LS to LS on Issues on QoS monitoring for satellite backhaul

Response to:	S2-2308298/C3-232469 (LS on Issues on QoS monitoring for satellite backhaul)

Release:	3GPP Rel-18

Work Item:	5GSATB

Source:	3GPP SA2

To:	3GPP CT3

Cc:

Contact Person:	Tianji Jiang

tianjijiang@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2309355 (CR 1150)

1. Overall Description:

3GPP SA2 thanks CT3 for your LS on Issues on QoS monitoring for satellite backhaul in C3-232469, discussing the support for QoS monitoring of dynamic Satellite Backhaul categories.

3GPP SA2 is responsible for the specification of QoS monitoring of dynamic Satellite Backhaul within 3GPP and these are the responses to your two questions:

Question 1:	Please clarify whether the PCF may determine to request QoS monitoring to get the report(s) for the packet delay based on the request received from the AF or based on local policy/configuration at the PCF (e.g., on the satellite backhaul category reported from the SMF)?

Answer 1:	Currently in TS23.501 and TS 23.503, the PCF generates the QoS monitoring policy based on local policy and AF request. So, the AF request is mandatory. And the answer for Q1 is: PCF may determine to request QoS monitoring to get the report(s) for the packet delay based on the request received from the AF, but cannot be only based on local policy/configuration at the PCF.

Question 2:	If the PCF triggers packet delay reporting for dynamic satellite backhaul categories based on local policy/configuration, does the report influence any policy decisions at the PCF?

Answer 2:	SA2 has discussion this case and please check the answer in the attached document.

3. Actions:

To CT3: SA2 respectfully asks CT3 to take the above information into consideration, and provide feedback if needed.

3. Date of Next TSG-SA WG2 Meetings:

SA5#159	9 – 13 October 2023	Xiamen, China

SA5#160	13 – 17 November 2023	Chicago, US