3GPP TSG-SA WG2 Meeting #158	S2-2310012

Gothenburg, Sweden, August 21 – 25, 2023	(revision of S2-2309995)

Title:	Reply LS on Issues on QoS monitoring for satellite backhaul

Response to:	S2-2308298 / C3-232469

Release:	Rel-18

Work Item:	5GSATB

Source:	SA2

To:	CT3

Cc:

Contact Person:

#### Name:	 Hannu Hietalahti

E-mail Address:	 hannu.hietalahti@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.501 CR 4861 and TS 23.503 CR 1150

1. Overall Description:

SA2 thanks CT3 for their LS and provides the following comments and answers.

In addition to the AF requested QoS monitoring, the PCF may trigger QoS monitoring based on local policy or configuration. If QoS monitoring is used, it is a PCF option whether and how to use the results in its policy decisions.

SA2 answers to the questions from CT3 as follows:

Question 1:	Please clarify whether the PCF may determine to request QoS monitoring to get the report(s) for the packet delay based on the request received from the AF or based on local policy/configuration at the PCF (e.g., on the satellite backhaul category reported from the SMF)?

SA2 answer: In addition to the AF requested QoS Monitoring as described in clause 6.1.3.21 and 6.1.3.22 of TS 23.503, the PCF may request QoS monitoring based on its own local policy or configuration. SA2 has clarified the TS 23.501 and TS 23.503 text in the attached CRs.

Question 2:	If the PCF triggers packet delay reporting for dynamic satellite backhaul categories based on local policy/configuration, does the report influence any policy decisions at the PCF?

SA2 answer: The PCF may use the QoS monitoring results as appropriate. It is up to PCF implementation and local configuration to determine what actions to take. This is not further specified by SA2.

2. Actions:

To CT3:

ACTION: 	SA2 kindly asks CT3 to take notice of this information and to align their specifications with it, as necessary.

3. Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#159 | Oct 9 – 13, 2023 | Xiamen | China |
| SA2 #160 | Nov 13 – 17, 2023 | Chicago | US |

See https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/