3GPP TSG-SA2 Meeting #159	S2-2310211

# October 09 – 13, 2023, Xiamen, China

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0350 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Layer-2 link management on UE-to-UE Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Editorial update of text in subclause 6.4.3.7.1. There are definitions of User Info IDs of several entities. At the end of each sentence, there is redundant text, ‘(i.e. User Info ID)’. The sentence starts with ‘User Info ID’, so it is not necessary. So, it is needed to be deleted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing redundant text ‘(i.e. User Info ID)’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are unnecessary texts in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

6.4.3.7.1	Common part for Layer-2 link management over PC5 reference point for 5G ProSe UE-to-UE Relay

For the 5G ProSe Communication via 5G ProSe UE-to-UE Relay as described in clause 6.7.1 and clause 6.7.2:

-	The Direct Communication Request message over the first hop PC5 reference point includes:

-	User Info ID of source 5G ProSe End UE: the identity of the source 5G ProSe End UE requesting relay operation.

-	User Info ID of 5G ProSe UE-to-UE Relay: the identity of the UE-to-UE Relay provided to the source 5G ProSe End UE during 5G ProSe UE-to-UE Relay Discovery procedure.

-	User Info ID of target 5G ProSe End UE: the identity of the target 5G ProSe End UE provided to the source 5G ProSe End UE during UE-to-UE Relay Discovery procedure.

-	(optional) Destination Layer-2 ID of target 5G ProSe End UE: the unicast destination Layer-2 ID of the target 5G ProSe End UE determined by the source 5G ProSe End UE as specified in clause 5.8.2.4.

-	ProSe Service Info: the information about the ProSe identifier(s) requesting Layer-2 link establishment.

-	RSC: the connectivity service provided by the 5G ProSe UE-to-UE Relay as requested by the source 5G ProSe End UE.

-	Security Information: the information for the establishment of security for the first hop PC5 link establishment.

NOTE 1:	The Security Information is defined by SA WG3.

The Direct Communication Request message over the second hop PC5 reference point includes:

-	User Info ID of source 5G ProSe End UE.

-	User Info ID of target 5G ProSe End UE.

-	User Info ID of 5G ProSe UE-to-UE Relay.

-	ProSe Service Info: the information about the ProSe identifier(s).

-	RSC: the connectivity service provided by the 5G ProSe UE-to-UE Relay as requested by the source 5G ProSe End UE.

-	Security Information: the information for the establishment of security for the second hop PC5 link establishment.

NOTE 2:	The Security Information is defined by SA WG3.

The Direct Communication Accept message over the second hop PC5 reference point includes:

-	User Info ID of target 5G ProSe End UE.

The Direct Communication Accept message over the first hop PC5 reference point includes:

-	User Info ID of target 5G ProSe End UE.

-	User Info ID of 5G ProSe UE-to-UE Relay.

The Link Modification Request message over the first hop PC5 reference point includes:

-	User Info ID of target 5G ProSe End UE: the identity of the target 5G ProSe End UE provided to the source 5G ProSe End UE during UE-to-UE Relay Discovery procedure.

-	(optional) Destination Layer-2 ID of target 5G ProSe End UE: the unicast destination Layer-2 ID of the target 5G ProSe End UE determined by the source 5G ProSe End UE as specified in clause 5.8.2.4.

The Link Modification Request message over the second hop PC5 reference point includes:

-	User Info ID of source 5G ProSe End UE.

-	User Info ID of target 5G ProSe End UE.

The Link Modification Accept message over the second hop PC5 reference point includes:

-	User Info ID of target 5G ProSe End UE.

The Link Modification Accept message over the first hop PC5 reference point includes:

-	User Info ID of target 5G ProSe End UE.

* * * End of Changes * * *