3GPP TSG-SA WG2 Meeting #159	S2-2310232

# Xiamen, China, October 09 – 13, 2023								          (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0356 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Layer-3 U2U Relay link identifier update procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the Link identifier update via Layer-3 UE-to-UE Relay procedure, the following parameters are not necessary: - "new IP address needed" indication is not necessary. The new IP address should be allocated using DHCP/SLACC mechanism but not relying on the Link identifier update procedure. - “its peer UEs information” is not necessary as the Relay UE has the information about which UE the source UE is communicating with. - "peer update" is not necessary as the new IP address can be used as a trigger to update to the target UE. The LIU over Layer-3 U2U Relay procedure should be updated to remove the unnecessary parameters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the Link identifier update over Layer-3 UE-to-UE Relay procedure by removing some unnecessary parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Link identifier update over Layer-3 UE-to-UE Relay procedure is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

6.7.1.2	Layer-2 link identifier update for PC5 communication via 5G ProSe Layer-3 UE-to-UE Relay

The Link Identifier Update procedure as defined in clause 6.4.3.2 is reused between source 5G ProSe Layer-3 End UE and 5G ProSe Layer-3 UE-to-UE Relay, and between 5G ProSe Layer -3 UE-to-UE Relay and target 5G ProSe Layer-3 End UE to perform a link identifier update. When the IP address/prefix is changed, the new IP address/prefix is exchanged between source 5G ProSe Layer-3 End UE and target 5G ProSe Layer-3 End UE as depicted in Figure 6.7.1.2-1.

Figure 6.7.1.2-1: Link Identifier Update and IP address/prefix exchange via 5G ProSe Layer-3 UE-to-UE Relay

0.	The source 5G ProSe Layer-3 End UE and the 5G ProSe Layer-3 UE-to-UE Relay, and the 5G ProSe Layer-3 UE-to-UE Relay and the target 5G ProSe Layer-3 End UE have a unicast link established as described in clause 6.7.1.1.

1.	The source 5G ProSe Layer-3 End UE decides to change its identifier(s) and sends a Link Identifier Update Request message to the 5G ProSe Layer-3 UE-to-UE Relay. The Link Identifier Update Request message includes the new identifier(s) of the source 5G ProSe Layer-3 End UE to use (including the new Layer-2 ID, Security Information, optionally the new Application Layer ID, and optionally the new IP address/prefix if IP communication is used).

2.	Upon reception of the Link Identifier Update Request message and if the IP address/prefix of the source 5G ProSe Layer-3 End UE is changed, the 5G ProSe Layer-3 UE-to-UE Relay sends a Relay Update Request message to the target 5G ProSe Layer-3 End UE. The Relay Update Request message includes the old and new IP address/prefix of the source 5G ProSe Layer-3 End UE.

3.	The target 5G ProSe Layer-3 End UE responds to the 5G ProSe Layer-3 UE-to-UE Relay with a Relay Update Response message. The target 5G ProSe Layer-3 End UE continues to receive traffic with the old IP address/prefix from the source 5G ProSe Layer-3 End UE until the target 5G ProSe Layer-3 End UE receives traffic with the new IP address/prefix from the source 5G ProSe Layer-3 End UE.

4.	The 5G ProSe Layer-3 UE-to-UE Relay responds with a Link Identifier Update Response message which includes new identifier(s) of the 5G ProSe Layer-3 UE-to-UE Relay to use (including the new Layer-2 ID, Security Information, optionally the new Application layer ID, and optionally the IP address/prefix if IP communication is used).

5.	The source 5G ProSe Layer-3 End UE responds with a Link Identifier Update Ack message. The Link Identifier Update Ack message includes the new identifier(s) from the 5G ProSe Layer-3 UE-to-UE Relay, as received on the Link Identifier Update Response message.

* * * * End of Changes * * * *