3GPP TSG-WG SA2 Meeting #159	S2-2310287

Xiamen, P.R. China, 09 - 13 October, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4949 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on AMF selection for MBSR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is proposed the following changes in clause 5.35A.1: - Correcting the descriptions when MBSR node does not operate as a MBSR. Based on the conclusion of SA2#157 meeting, the following is agreed: If the MBSR node does not operate as a MBSR, e.g. due to the MBSR authorization indication from AMF, it does not provide the indication when establishing new RRC connection. Editor's note: The detailed handling between NAS and AS needs further synch with RAN WGs. However, if the IAB-UE(MBSR) is de-registered due to MBSR authorization and it doesn’t provide MBSR indication in the RRC connection during the next initial registration procedure, the IAB-donor-CU cannot select an AMF that supports IAB-node with mobility because the IAB-donor-CU assumes the IAB-UE is accessing the core network as a normal UE. Then even the MBSR moves to an area where the MBSR can be authorized as a MBSR, the selected AMF as a legacy AMF which doesn’t support IAB mobility neither trigger Deregistration procedure nor UE Configuration Update procedure. In this case, the MBSR is still registered as a normal UE and will not receive the change of MBSR authorization from AMF. It is proposed to correct the above descriptions. - Replacing the terminology of “mobile IAB indication” in clause 5.35A.1 with MBSR indication, in order to align with others clauses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections on AMF selection for MBSR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AMF selection for MBSR is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.35A.1	General

The MBSR uses the IAB architecture as defined in clause 5.35, and operates as an IAB node (with an IAB-UE and gNB-DU) with mobility when integrated with the serving PLMN. The architecture described in clause 5.35 applies unless specific handling is specified in clause 5.35A. Additionally, the following limitations apply to the MBSR:

-	the MBSR has a single hop to the IAB-donor node;

-	NR Uu is used for the radio link between a MBSR and served UEs, and between MBSR and IAB-donor node.

Regulatory requirements (e.g. emergency services, priority services) are supported when UEs access 5GS via a MBSR. LCS framework as defined in TS 23.273 [87] is used for providing the location service to the served UEs, with additional enhancements described in clause 5.35A.5.

Roaming of the MBSR is supported, i.e. a MBSR can integrated with a VPLMN's IAB-donor node. The corresponding enhancements to support MBSR roaming are described in clause 5.35A.4.

CAG mechanism as defined in clause 5.30 can be used for the control of UE's access to the MBSR. Optional enhancements to the CAG mechanism for MBSR use are described in clause 5.35A.6.

For a MBSR node to operate as a MBSR, it provides a MBSR indication to the IAB-donor-CU to indicate that it operates as an IAB node with mobility when the RRC connection is established as defined in TS 38.331 [28]. When the MBSR indication is received, the IAB-donor-CU selects an AMF that supports IAB-node with mobility and includes the MBSR indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34] so that the AMF can perform MBSR authorization as described in clause 5.35A.4. If the MBSR node is not authorized, e.g. due to the MBSR authorization indication from AMF, it also provides the MBSR indication when establishing new RRC connection so that the AMF supporting IAB-node with mobility selected by the IAB-donor-CU can decide whether the MBSR authorization state changes for a registered MBSR node as described in clause 5.35A.4.

Editor's note:	The detailed handling between NAS and AS needs further synch with RAN WGs.

After the IAB-UE performs registration procedure in 5GS, further mobility procedure can be performed to change the IAB-donor-DU, the IAB-donor-CU as specified in TS 38.401 [42]. The mobility support of UEs served by the MBSR is specified in clause 5.35A.3

# * * * * End of changes * * * *