3GPP SA WG2 Meeting #159	S2-2310338

# Xiamen, China, October 9 - October 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4958 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for the Unavailability Period Feature over non-3GPP Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc., Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | SUECR |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support for the Unavailability Period feature relies on the AMF taking the Unavailability Period Duration and Start of Unavailability Period into account when determining Periodic Registration Update timer value. There is no Periodic Registration Update timer when the UE is registered over non-3GPP access, therefor the feature is not supported over non-3GPP access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new section under section 5.5.3 UE reachability, under Non-3GPP access specific aspects for support of the Unavailability Period feature. The new section refers to the existing section and adds that the non-3GPP Deregistration timer value in the UE may be set based on the Unavailability Period Duration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Unavailability Period feature would not be supported when the UE is registered over non-3GPP access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

#### 5.5.3.x	Support of Unavailability Period

Support for the Unavailability Period feature when the UE is connected over non-3GPP access is as described in clause 5.4.1.4 with the following additions.

- Since the AMF does not provide a Periodic Registration Update over non-3GPP access, the AMF may take the Unavailability Period Duration (if available) and Start of Unavailability Period into account when determining non-3GPP Deregistration timer value. The AMF may provide a non-3GPP Deregistration timer value that is longer than or equal to the combination of the Unavailability Period Duration and Start of Unavailability Period to avoid interfering with the UE dealing with the event that causes the unavailability.

*************************** End of Changes ****************************