SA WG2 Meeting #S2-159							                 S2-2310360

09 - 13 October, 2023, Xiamen, P.R. China

Title:	[Draft]LS reply on Issues on QoS Monitoring parameters for XR service

Response to:	LS on Issues on QoS Monitoring parameters for XR service (C3-233649/S2-2310102)

Release:	Release 18

Work Item:	XRM

Source:	SA2

To:	CT3

Cc:	CT4

Contact Person:

#### Name:	Xiaowan Ke

#### Tel. Number:

#### E-mail Address:	xiaowan(dot)ke(at)vivo(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR4736

- Overall description

SA2 thanks CT3’s LS on Issues on QoS Monitoring parameters for XR service (C3-233649/S2-2310102)and would like to provide the following replies to CT3’s questions:

Question 1:

If NG-RAN provides congestion measurements based on NG-RAN implementation, do the reporting period, and periodic and event triggered reporting frequencies apply to the monitoring and exposure of congestion information?

SA Answer:

Yes; as defined CR4736 (in clause 5.45 of TS 23.501), the QoS Monitoring request for QoS parameter (i.e. congestion information), provided by the SMF to the RAN, includes the following:

-	QoS parameters to be measured (i.e. congestion information);

-	Reporting frequency (event triggered, periodic):

-	if the reporting frequency is event triggered:

-	the corresponding reporting threshold to each QoS parameter;

-	minimum waiting time between subsequent reports;

-	the reporting period.

Question 2:

If the answer to question 1 is yes, for event triggered reporting, does it mean that the minimum and the maximum congestion reports provided by NG-RAN also need to be reported, if received within the minimum waiting time?

SA Answer:

SA2 thinks the average, minimum and maximum values are all valuable. But SA2 treats those as stage3 detail and leave it to CT3 decision.

Question 3:

For data rate monitoring, if the averaging window is shorter than the waiting time, does the UPF report the minimum and maximum monitored data rate or should the UPF report the average?

SA Answer:

SA2 thinks the average, minimum and maximum values are all valuable. But SA2 treats those as stage3 detail and leave it to CT3 decision.

Question 4:

Do the minimum and the maximum PDV measurement results also need to be reported by the PCF?

SA Answer:

SA2 thinks the average, minimum and maximum values are all valuable. But SA2 treats those as stage3 detail and leave it to CT3 decision.

Question 5:

For the Congestion information, Data Rate and PDV, does it mean that the measurement failure also needs to be reported if no measurement result is available?

SA Answer:

SA2 understands it is stage3 aspects and expects robustness definition for the measurement report. However, SA2 treats the following case as normal measurement result rather than measurement failure: no congestion and data rate=0.

Question 6:

If the answer to question 5 is yes, how does the UPF detect that no Congestion or Data Rate measurement result is available?

SA Answer:

Date rate is measured by PSA UPF on its own and so the PSA UPF is always aware of the data rate for the QoS flow through measurement, including data rate=0, i.e. no data.

The Congestion information depends on RAN reported congestion information and no congestion is a kind of measurement report.

Question 7:

Whether all TSC QoS requirements are applicable to multi-modal/XRM communications or not?

SA Answer:

No, only the UL and/or DL Periodicity and N6 Jitter Information associated with the DL Periodicity defined in clause 5.37.8 of TS 23.501 are applied to multi-modal/XRM communication. The TSCAI is only reused as the transmission format to provide the information defined in clause 5.37.8 of TS 23.501.

- Actions

To CT3:

ACTION: 	SA2 respectfully asks CT3 to take the above information into account.

3	Dates of next TSG SA WG 2 meetings

SA2#160	13th - 17th November 2023, 			Chicago, USA

SA2#161	26th February-1st March, 2024                            	Athens, GR