3GPP WG-SA2 Meeting #159	S2-231xxxx

Xiamen, China, Oct 9 – 13, 2023

Source: 	China Unicom

Title: 	Observations and gap analysis on Network Sharing Enhancements in R19

Agenda item:

Document for:

Introduction

Based on the offline discussion in the last SA2 meeting, some potential solutions and wayforward options were provided and recorded. This contribution provides observations and gap analysis as input from China Unciom’s perspective for furthering the discussions on Network Sharing Enhancements to progress the work and reach consensus.

The content of DP including:

- Clarify the basic requirements of Network Sharing in R19 considering the factual deployment.

- Gap analysis between the potential solutions and the requirements to get a specific solution for work further.

- Gap analysis between this provided solution and the existing specification for workload evaluation.

- Summarizes conclusions and proposals.

Basic requirements

The clarifications of the basic requirements when we evaluated the different network sharing mechanism to deploy are as follow:

- Currently, only 5G MOCN network sharing mechanism is supported in the existing 5G specification. However the challenge for the network operators is the maintenance generated by the “direct” interfaces (e.g., number of N2 and N3 interfaces) between the shared RAN and two or more core networks, if using 5G MOCN, especially for a large number of shared 5G base stations. For this reason, it is valuable to introduce a newly supported network sharing mechanism based on the operators' agreement.

- From operator’s perspective, the charging and accounting crossing PLMNs need to be supported flexibly using the new network sharing mechanism, for example we hope to charge between hosting operator and participating operator using the amount of data, this charging mode is more flexible than 5G MOCN, because the charging and accouting of 5G MOCN is always based on sharing agreements.

- Currently, GWCN is specified for 3G/4G in TS 23.251. For GWCN in EPS, the shared MME may be co-constructed by each participating operator based on the architecture figure. No clear which operator the shared MME belongs to. However considering the deployment case, we hope the shared NFs belong to one operator definitely considering the charging, management and division of responsibility. And the security aspect between hosting operator and participating operator also needs to be considered.

- And the scenario to be addressed is that some UEs of participating operator can be authorized to access the shared NR and the other UEs cannot, e.g., based on UE subscription. The potential requirement is that the charging using the new network sharing mechanism may be different based on the network sharing agreement. Because the 4G of participating operator has large coverage in the country, the UE can continue to use the 4G network even though the UE moves to the shared NR area. Therefore the shared 5G may be seemed as value-added business per UE. This is an optional functionality for extended commercial mode.

Above requirements are also specified in TS 22.261, and new network sharing mechanism is named as Indirect Network Sharing:

- Indirect Network Sharing: a type of NG-RAN Sharing in which the communication between the Shared NG-RAN and the Participating Operator’s core network is routed through the Hosting NG-RAN Operator’s core network.

- In case of Indirect Network Sharing, the communication between the Shared NR and the Participating Operator’s core network happens via a number of inter-operator interfaces that are independent of the actual number of base stations at the Hosting Operator.

- The network sharing partners can set a specific sharing allocation model for the network sharing method they are using. The collection of charging information associated with the sharing method that the UE accesses with can be possible. It is also necessary to understand the charging information between the networks of both parties, e.g., the number of the users, and how long users using a certain shared network method will take. This information is also needed when users use the participating operator's hosted services they have subscribed to and their flexible charging via Shared NR.

- Based on operator policy, the 5G system shall support a mechanism to enable an authorized UE with a subscription to a Participating Operator to select and access a Shared NR.

- During NG-RAN sharing, the security and privacy of shared networks, non-shared networks, and subscribers need to be maintained without negative effects. Especially in the case of Indirect Network Sharing, where the involvement of the core network of the hosting operator e.g. for signalling exchange between the users and the core network of the participating operator could cause exposure of the subscriber’s information to the hosting network, an extra scrutiny of the security mechanism is expected to avoid sharing the information that is not needed for the Indirect Network Sharing operation (e.g. network topology) and protect the information that is needed for the Indirect Network Sharing operation between the hosting operator and the participating operator.

The figure of Indirect Network Sharing defined in Annex I of TS 22.261 can be as the reference, the OP1 is hosting operator, OP2 and OP3 are participating operators:

Figure 1: Indirect Network Sharing scenario

Observation 1: The new network sharing mechanism in 5G should be specified to satisfy the above requirements and SA1 definition of Indirect Network Sharing.

Gap analysis between the potential solutions and the requirements

Based on the offline discussion in the last SA2 meeting, some use cases and potential solutions were provided and recorded. And the clarifications are given from China Unicom’s perspective as below:

- In the factual deployment, the GWCN network sharing mechanism specified in TS 23.251 is not used from our side. However the scenario of the co-existence between 5G MOCN and the new Indirect Network Sharing may happen in the certain deployment case. This use case was also specified in Figure I-1 of TS 22.261. The Hosting NR Operator 1, as illustrated below, can share its NR with the different participating operators using different network sharing mechanism (5G MOCN or Indirect Network Sharing). And the hosting NR also provides the access for its own UE.

Figure 2: The scenario of the co-existence between 5G MOCN and the Indirect Network Sharing

- The requirements include avoiding to maintain a large number of interfaces between Hosting Operator’s Shared NR and Participating Operator’s core network (e.g. N2/N3 interfaces).

- The potential solution provided in the last meeting to address the N2/N3 issue is designed as the GWCN approach for 5GS for N2 aggregation and ETSUN approach for N3 aggregation. The potential architecture is shown as below:

Figure 3: Architecture of the potential solution of new Network sharing mechanism

The gap analysis between the potential solutions and the requirements:

- Architecture aspect: considering the requirements as above, the network sharing scenario involves the core networks of hosting operator and participating operator, the original design of architecture is:

Figure 4: Original architecture of Indirect Network Sharing

As shown in the Figure 4 and comparing with the potential solution as shown in Figure 3, the NFs of core network of hosting operator 1 connects with the NFs of core network of participating operator 2/3 through SEPP and UPF supporting IPUPS considering the security of inter PLMN case. And the NSSF and NRF of hosting operator also need to be introduced to implement the network slice mapping function and NF selection function crossing PLMNs in this network sharing scenario for 5G. Considering the concern about sharing full core network of hosting operator, the suitable architecture of the new network sharing mechanism is provided as below:

Figure 5: Suitable architecture of Indirect Network Sharing considering the different requirements

For this architecture, the hosting operator involves the shared RAN, AMF, SMF, UPF, NSSF and NRF, and the main interfaces crossing PLMNs which need to be maintained are N9, N16 and N14 (if considering UE mobility).

- Network access control: Considering the specific PLMN IDs of participating operators broadcasted by shared NR,  the EHPLMNs or EPLMNs of participating operators broadcasted by shared NR are suitable. The scenario is shown as figure below:

Figure 6: The boundary scenario of Indirect Network Sharing

In the Indirect Network Sharing case, the reasonable priority of PLMN and RAT selection is NR of participating operator > shared NR of hosting operator > E-UTRAN of participating operator. And in the boundary scenario, i.e., the coverage of hosting operator’s shared 5G network may overlap with participating operator’s 5G network (③ in the figure), the UE of participating operator should access the subscribed network in the area ③. However if the shared NR broadcasts the same PLMN ID of the participating operator, the UE will detect two same PLMN IDs simultaneously, and the UE may mistakenly connect to the shared NR instead of the NR of participating operator. And considering the network name display requirements from CMCC, the EPLMN ID or EHPLMN ID (different from HPLMN ID) can be mapped to the different network name flexibly. Accordingly, the solution of broadcasting the EPLMN ID or EHPLMN ID of participating operator is more applicable in the Indirect Network Sharing case. The EPLMN ID and EHPLMN ID solution can reuse the network selection mechanism specified in TS 23.122. This is no UE impact.

NOTE:	In this boundary scenario, the signalling strength is also considered in network selection. The main requirement is that UE should access the 5G of participating operator in an acceptable range of signalling strength considering the charging aspect (whether to use hosting operator network), the UP/CP path and network sharing agreement.

During the registration procedure, the AMF in hosting operator can route the registration message to the participating operator based on UE SUPI/SUCI specified in clause 4.2.2.2 of TS 23.502. However the AMF in hosting operator needs to justify whether the UE is Indirect Network Sharing UE or not based on the selected PLMN ID.

- SMF selection: The impact of user experience (delay, data rate) needs to be minimized in the Indirect Network Sharing case, therefore the traffic related to Indirect Network Sharing should be routed to the subscribed DN nearby the shared area. After the AMF justified that the UE is Indirect Network Sharing UE, the AMF selects the H-SMF considering the UE location information via V-NRF. And the H-SMF registers to the H-NRF including the corresponding location information of shared area and selected PLMN ID. There are no impact to V-SMF selection.

- Charging aspect: Based on the flexible charging requirement of inter PLMN case, the SMF of hosting operator and SMF of participating operator need to collect the charging information related to the Indirect Network Sharing traffic. And the charging and accouting between the two PLMNs is different from ETSUN.

Observation 2: Considering architecture of Indirect Network Sharing provided in Figure 5, the NFs of core network of hosting operator connects with the NFs of core network of participating operator through SEPP and UPF supporting IPUPS considering the security of inter PLMN case, it is similar as HR roaming. The hosting operator involves the shared RAN, AMF, SMF, UPF, NSSF and NRF.

Observation 3: The shared NR broadcasts the multiple PLMN IDs (e.g., EPLMN IDs or EHPLMN IDs) of participating operators. AMF in hosting operator can justify whether the UE is Indirect Network Sharing UE or not based on the selected PLMN ID. The SMF selection and charging aspect are different with ETSUN.

Gap analysis between the provided solution and the existing specification

Based on the analysis as above, this clause gives the enhanced parts between the provided solution and the existing specification considering registration procedure, PDU session establishment procedure and mobility procedure.

- Registration procedure:

When the UE registers to the hosting operator, the AMF of hosting operator needs to justify whether the UE is Indirect Network Sharing UE or not based on the selected PLMN ID.

- PDU session establishment procedure:

When the UE triggers the PDU session establishment procedure in the shared network, the AMF of hosting operator needs to consider the UE location information (e.g., geographical area) to select H-SMF and supports the conversion from TAI to geographical area. In clause 4.3.2.2.3.3 of TS 23.502, the H-SMF selection mechanism in HR roaming case only considers the limited parameter including PLMN ID of the SUPI, DNN, HPLMN S-NSSAI, the hNRF and possibly an HPLMN NSI ID, it does not consider the UE location information.

The H-SMF registers to H-NRF with the UE location information (e.g., geographical area) reusing the existing field, e.g., servingScope or locality specified in clause 6.1.6.2.2 of TS 29.510.

- Mobility procedure:

The mobility registration update procedure specified in clause 4.23.3 of TS 23.502 considers the serving area of SMF to trigger the I-SMF insertion/change/removal. However the H-SMF registers to H-NRF with the location information of shared area in the Indirect Network Sharing case (the serving area of SMF may include the shared area), the AMF should consider the selected PLMN ID instead of the serving area of SMF to trigger the V-SMF insertion when the UE moves to shared area reusing this mobility registration update procedure.

Observation 4: Based on the provided architecture in clause 3 and the gap analysis in this clause, the work item or TEI19 of Indirect Network Sharing can start to define the architecture and solve the gap described as above.

Conclusions and Proposals

The observations and proposals from the above evaluation are provided as input to further the discussion on network sharing enhancements and reach consensus:

Observation 1: The new network sharing mechanism in 5G should be specified to satisfy the above requirements and SA1 definition of Indirect Network Sharing.

Observation 2: Considering architecture of Indirect Network Sharing provided in Figure 5, the NFs of core network of hosting operator connects with the NFs of core network of participating operator through SEPP and UPF supporting IPUPS considering the security of inter PLMN case, it is similar as HR roaming. The hosting operator involves the shared RAN, AMF, SMF, UPF, NSSF and NRF.

Observation 3: The shared NR broadcasts the multiple PLMN IDs (e.g., EPLMN IDs or EHPLMN IDs) of participating operators. AMF in hosting operator can justify whether the UE is Indirect Network Sharing UE or not based on the selected PLMN ID. The SMF selection and charging aspect are different with ETSUN.

Observation 4: Based on the provided architecture in clause 3 and the gap analysis in clause 4, the work item or TEI 19 of Indirect Network Sharing can start to define the architecture and solve the gap described as above.

Proposal: The work item or TEI19 of Network Sharing in SA2 R19 should start to complete the normative work to solve the gaps, the related enhancements are provided for further discussion:

- The definition and architecture of Indirect Network Sharing need to be specified in clause 5.18 of TS 23.501 below the 5G MOCN clause.

- Under the Indirect Network Sharing case, the EPLMN or EHPLMN broadcasting solution needs to be clarified.

- The AMF should support the conversion from TAI to geographical area of UE location information and justify whether the UE is Indirect Network Sharing UE or not based on the selected PLMN ID in clause 6.2.1 of TS 23.501.

- In clause 4.3.2.2.3.3 of TS 23.502, the H-SMF selection mechanism also considers the UE location information in the Indirect Network Sharing case;

- The mobility registration update procedure specified in clause 4.23.3 of TS 23.502 adds the trigger condition based on the selected PLMN ID.

NOTE 1:	There is no UE functional impact.

NOTE 2: 	The scope limits to NR with 5GC in this network sharing mechanism.