3GPP TSG-SA WG2 Meeting #159	S2-2310409

# Xiamen, China, 09 – 13 October 2023								  	    (Revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0363 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New IP address for peer remote UE in Link Identifier Update procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 6.7.1.2, the link identifier update procedure may be initiated for updating UE1’s IP address/prefix. The UE-to-UE Relay includes UE1’s new IP address/prefix in the Relay Update Request message sent to UE2. Currently, the Relay Update Request message does not include UE2’s new IP address/prefix. Normally, when the UE changes its IP address/prefix, the peer UE would also change its IP address/prefix. Thus, in case UE1 changes its IP address/prefix, UE2 would initiate another link identifier update procedure with the UE-to-UE Relay after the link identifier update procedure initiated by UE1 is completed. In terms of efficiency, the Relay Update Request message can also include UE2’s new IP address/prefix if UE2’s IP address/prefix is allocated by the UE-to-UE Relay. Thus, this CR is to propose providing UE2’s new IP address/prefix in addition to UE1’s new IP address /prefix in the link identifier update procedure initiated by UE1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 6.7.1.2: In step 2 of Figure 6.7.1.2-1, the statement “If UE2's IP address/prefix is allocated by the UE-to-UE Relay, UE-to-UE Relay assigns a new IP address/prefix to UE2 and saves it locally” is added in bullet a, and “possibly UE2's new IP address/prefix” is added in bullet b. In step 3 and step 4 of Figure 6.7.1.2-1, the related statements of optionally including UE2’s IP address/prefix are added. In step 5 of Figure 6.7.1.2-1, the term of “its new IP address” is modified as “the new IP address(es)”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE2 would initiate additional link identifier update procedure for updating UE2’s IP address/prefix after the link identifier update procedure for updating UE1’s IP address/prefix initiated by UE1 is completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 6.7.1.2	link identifier update for PC5 communication via 5G ProSe Layer-3 UE-to-UE Relay

The existing Link Identifier Update procedure as defined in clause 6.4.3.2 is reused between an End UE and Layer -3 UE-to-UE Relay to perform a link identifier update and when the IP address/prefix needs to be changed, the procedure is updated as depicted in Figure 6.7.1.2-1.

Figure 6.7.1.2-1: Link Identifier Update procedure and IP address/prefix change with Layer-3 UE-to-UE Relay

0.	A PC5 link is established between UE1 and a UE-to-UE Relay. Another PC5 link is established between the UE-to-UE Relay and UE2. IP data may be exchanged between UE1 and UE2 via the UE-to-UE Relay over the PC5 links. During the link establishment, UE1 informs the UE-to-UE Relay that the link requires privacy.

1.	As stated in clause 6.4.3.2, according to the privacy requirement, UE1 may trigger link identifier update procedure. UE1 sends a Link Identifier Update Request message to the UE-to-UE Relay including the following parameters:

-	its new Layer-2 ID, new security information, new Application layer ID (if provided by the upper layer).

-	If UE1's IP address/prefix needs to be changed:

-	if UE 1's IP address/prefix is allocated by the UE-to-UE Relay, UE1 includes "new IP address needed" indication.

-	if UE1 self-assign its IP address/prefix, UE1 includes its new IP address/prefix.

-	its peer UEs information (e.g. UE2's IP address, UE2's Application layer ID), allowing the UE-to-UE Relay to inform UE1's peer UEs (e.g. UE2) about UE1's new allocated IP address/prefix and "peer update" indication are included.

2.	UE-to-UE Relay self-assigns a new L2 ID, new security information and possibly new IP address/prefix for PC5 link with UE1.

a.	If a "new IP address needed" indication is received, UE-to-UE Relay assigns a new IP address/prefix to UE1 and saves it locally. If UE2's IP address/prefix is allocated by the UE-to-UE Relay, UE-to-UE Relay assigns a new IP address/prefix to UE2 and saves it locally.

b.	If a "peer update" indication is received, UE-to-UE Relay fetches the peer UEs (e.g.UE2's) entry from its local table based on the received Application layer IDs or IP addresses/prefixes.

UE-to-UE Relay then sends a PC5 Relay Update Request message to each peer UE (e.g. UE2), including: UE1's old IP address/prefix, UE1's old and new Application layer ID, UE1's new IP address/prefix, possibly UE2's new IP address/prefix.

3.	UE2 receives the PC5 Relay Update Request message and saves UE1's new IP address/prefix and possibly UE2's new IP address/prefix. UE2 sends a PC5 Relay Update Response message to the UE-to-UE Relay including all parameters received on the PC5 Relay Update Request message.

a.	UE2 continues to receive IP data with UE1's old IP address (transit packets sent prior to UE1's receiving its new IP address) until an IP packet using UE1's new IP address is received. At this point, UE2 starts using UE1's new IP address and may forget UE1's old IP address.

4.	UE-to-UE Relay sends a Link Identifier Update Response message to UE1 including UE1's new IP address/prefix, possibly UE2's new IP address/prefix, UE-to-UE Relay's new Layer-2 ID, new security information and possibly new IP address/prefix and/or new Application layer ID.

5.	UE1 saves its new IP address/prefix and UE-to-UE Relay's new parameters and sends a Link Identifier Update ACK message to the UE-to-UE Relay, including the new IP address(es) received on the Link Identifier Update Response message. UE1 and UE-to-UE Relay start using the new Layer-2 IDs and new security information for PC5 communication. UE1 starts using the new IP address(es) for IP data exchange with UE2.

END OF CHANGES