3GPP TSG-WG SA2 Meeting #159	S2-2310475

# Xiamen, CN, October 9 – 13, 2023		(revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0445 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing information in the subscription data for exposure via CP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA3 has concluded the security mechanisms for Ranging and Sidelink positioning in TS 33.533. For SL Positioning Client UE using Ranging/SL Positioning service exposure through 5GC C-plane, the authorization reuses the authorization for MO-LR as specified in TS 23.273. The MO-LR authorization is performed by the AMF by checking the subscriber information detailed in clause 7.1. This information is missing and needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing information in the subscription information that the UE is allowed to receive Ranging/SL positioning result related to other UE subscribers by using the Ranging/SL Positioning exposure service over the control plane. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The authorization of a SL Positioning Client UE requesting to use the Ranging/SL Positioning exposure service over the control plane will fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 7.1	UDM

For each UE subscriber the UDM stores LCS related data as part of the Subscriber Data Management (SDM) service as defined in clause 5.2.3.3.1 of TS 23.502 [19].

The privacy profile data is defined in table 7.1-1 containing data for the privacy classes for which location of the target UE is permitted. For the meaning of each LCS privacy profile data type and included data, refer to clause 5.4.2.

Table 7.1-1: LCS privacy profile data stored in the UDM for a UE Subscriber

| Privacy Profile Data Type | Presence | UDM data |
| --- | --- | --- |
| Location Privacy Indication | M O | Indication of one of the following mutually exclusive global settings: - Location is disallowed - Location is allowed (default) Time period when the Location Privacy Indication is valid |
| Call/session Unrelated Class | M O O O O O O O O O O O O O | For any LCS client or AF not in the external LCS client list or otherwise identified for the Call/session Unrelated Class, the following data may be present: - One of the following mutually exclusive options: - Location not allowed (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC External LCS client list: a list of zero or more LCS clients, AFs and LCS Client groups with the following data for each entry: - One of the following mutually exclusive options: - Location allowed without notification (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed Service types list: a list of one or more service types for which the LCS client is allowed to locate the particular UE. The possible service types are defined in TS 22.071 [2]. The following data may be present for each service type in the list: - One of the following mutually exclusive options: - Location allowed without notification (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC |
| PLMN Operator Class | O | LCS client list: a list of one or more generic classes of LCS client that are allowed to locate the particular UE. The following classes are distinguished: - LCS client broadcasting location related information - O&M LCS client in the HPLMN - O&M LCS client in the VPLMN - LCS client recording anonymous location information - LCS Client supporting a bearer service, teleservice or supplementary service to the target UE - NWDAF in the HPLMN (when the UE is currently being served by the HPLMN) - NWDAF in the VPLMN |
| User Plane Connection between UE and LCS Client or AF | O | Indication of one of the following mutually exclusive global settings: - UE is allowed to report periodic or triggered location events via user plane to an LCS Client or AF - UE is not allowed to report periodic or triggered location events via user plane to an LCS Client or AF (default) |
| Event report expected area | O | Presents a geographical area generated by UE, which is used by GMLC to determine event report allowed area for the UE |
| Area usage indication | O | Indication of one of the following mutually exclusive global settings on using event report allowed area: - Inside reporting (default) - Outside reporting |
| GMLC address list | O | Addresses of GMLC located in local network(s) which are allowed to be used for the UE positioning |

The Mobile Originating data is defined in table 7.1-2 containing the LCS MO-LR services that a UE can receive.

Table 7.1-2: LCS Mobile Originated data for a UE Subscriber

| MO-LR Data | Presence | UDM data |
| --- | --- | --- |
| Mobile Originated data | M | List of MO-LR services allowed for a UE subscriber: - Basic Self Location (UE can receive its own location) - Autonomous Self Location (UE can receive location assistance data) - Transfer to Third Party - Ranging/SL Positioning exposure service (UE can receive Ranging/SL positioning result related to other UE subscribers) |
| List of Assistance Data Types for MO-LR | O | A list of one or more types of location assistance data that may be provided to the UE in the MO-LR procedure. |

The LCS broadcasting data is defined in table 7.1-3 containing a list of assistance data types for which ciphering keys should be provided to the UE if requested by the UE.

Table 7.1-3: LCS broadcasting data for a UE subscriber

| Broadcasting Data | Presence | Description |
| --- | --- | --- |
| List of Assistance Data Types | O | A list of one or more types of location assistance data for which ciphering keys should be provided to the UE if requested by the UE when the assistance data is broadcast using ciphering. |

Other UE LCS subscriber data is defined in table 7.1-4 containing UE's subscription data related with LCS.

Table 7.1-4: UE LCS subscriber data

| UE LCS subscriber data | Presence | Description |
| --- | --- | --- |
| PRU indication | O | When present, it is used to indicate that the UE is allowed to serve as a PRU. |
| LPHAP indication | O | When present, it indicates a UE requires low power and high accuracy positioning. |
| LMF ID | O | An LMF ID which presents an LMF deployed in local network to support location service for the UE. |
| Indication of user plane positioning between UE and LMF | O | When present, it indicates that the UE is allowed to use the user plane positioning between UE and LMF. |

# * * * * End of changes * * * *