3GPP TSG-SA WG2 Meeting #159	S2-2310505

# Xiamen, China, 9 - 13 Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1170 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on the Network Slice Replacement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is to It has been agreed that the slice replacement shall be transparent to the AF. To be able to handle the AF request for the old S-NSSAI correctly, the association of the old S-NSSAI of the PDU session needs to be kept for the PDU session (i.e., PDU session associated with both the S-NSSAI and the Alternative S-NSSAI) in the PCF for the PDU session (and in the BSF for PCF discovery). |  |  |  |  |  |  |  |  |
| Summary of change: |  | -The PCF for the PDU Session associates the SM policy association for the PDU session with both S-NSSAI and the Alternative S-NSSAI. -The PCF for the PDU Session associates the binding information for the PDU session with both S-NSSAI and the Alternative S-NSSAI in BSF. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | -The PCF for the PDU Session may reject the request including the old S-NSSAI for the PDU session -The PCF for the PDU Session whose S-NSSAI is replaced by the Alternative S-NSSAI may not be discovered using the old S-NSSAI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.29 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.1.3.29	Network Slice replacement for existing PDU Session

The Network Slice Replacement is specified in clause 5.15. 19 of TS 23.501 [2].

The PCF may set the Network Slice Replacement trigger event in the SMF as defined in clause 6.1.3.5.

When the existing PDU Session is transferred from S-NSSAI to Alternative S-NSSAI, if the SMF determines that the existing PDU Session and existing SM Policy Association can be retained (e.g. if the SMF determines that same PCF can be used for the Alternative S-NSSAI), then the SMF includes the Alternative S-NSSAI in SM Policy Association modification request to PCF to update the S-NSSAI of the PDU Session. The PCF associates the Alternative S-NSSAI and the S-NSSAI to the SM policy association. The PCF may request subscription information associated with the Alternative S-NSSAI from the UDR. Then the PCF makes policy decision based on the Alternative S-NSSAI and may provide updated PCC rules to SMF. The PCF may update the binding information in the BSF to be associated with the Alternative S-NSSAI and the S-NSSAI.

Otherwise, if the SMF determines to re-establish the PDU Session on the Alternative S-NSSAI, then the SMF for the new PDU Session includes the Alternative S-NSSAI and the S-NSSAI in SM Policy Association create request. The PCF associates the Alternative S-NSSAI and the S-NSSAI to the SM policy association. The PCF may create the binding information for the PDU session to be associated with both the Alternative S-NSSAI and the S-NSSAI in the BSF.

END of CHANGES