SA WG2 Meeting #SA2-159	S2-2310510

Oct 09 – 13 2023, Xiameng, China

Title:	[Draft] Reply LS on 5G ProSe UE-to-UE relay discovery

Response to:	LS to SA2 on 5G ProSe UE-to-UE relay discovery (S2-2310091/ C1-236448)

Release:	Rel-18

Work Item:	5G_ProSe_Ph2

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Wen Wang

E-mail Address:	wen.wang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	xxxx

- Overall description

SA2 thanks CT1 for their LS on 5G ProSe UE-to-UE relay discovery (S2-230aaaa/ C1-23xxxx).

SA2 has discussed questions from CT1 and provides the following responses.

Answer to Q1: SA2 has agreed to remove the relay_Indication related processes in 5G ProSe Direct Discovery with Model A as in the CR attached.

Question 2: How can the 5G ProSe UE-to-UE relay UE tell that the 5G ProSe UE-to-UE Relay Discovery Solicitation message is for the relay reselection so that it can skip finding the target end UE and respond to the source end UE directly?

Answer to Q2:  SA2 has decided to add an additional indication in Solicitation message to indicate that it is for the relay reselection, so that it can skip finding the target end UE and respond to the source end UE directly.

Question 3: How can the 5G ProSe UE-to-UE relay UE know which service the discoverer end UE wants to use before accepting the 5G ProSe UE-to-UE Relay Discovery Solicitation message?

Answer to Q3:  SA2 considers that the RSC is needed as the correction in the CR.

Question 4: Why are different approaches applied to the user info of the target 5G ProSe end UE in the above procedures?

Answer to Q4: SA2 decides to revise their specification that the user info of the target 5G ProSe end UE is also mandatory in the Direct Communication Request message for 5G ProSe UE-to-UE relay communication with integrated discovery.

2. Actions:

To CT1:

ACTION: 	SA2 kindly asks CT1 to take the above information into account.

3. Date of Next SA2 Meetings:

3GPP TSG-SA2 Meeting #160	13-17 November 2023	CHICAGO, USA