3GPP TSG-SA WG2 Meeting #159	S2-2310511

# Oct 9 – 13, 2023, Xiamen, China							 (revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0366 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Candidate U2U relay discovery updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As the issue pointed out by CT1 LS C1-236448: Question 2: How can the 5G ProSe UE-to-UE relay UE tell that the 5G ProSe UE-to-UE Relay Discovery Solicitation message is for the relay reselection so that it can skip finding the target end UE and respond to the source end UE directly? Question 3: How can the 5G ProSe UE-to-UE relay UE know which service the discoverer end UE wants to use before accepting the 5G ProSe UE-to-UE Relay Discovery Solicitation message? For the Q2: To tell that the 5G ProSe UE-to-UE Relay Discovery Solicitation message is for Model B discovery or candidate U2U relay discovery, it is proposed a candidate indication is included in the Solicitation message to represent the candidate U2U relay discovery. For the Q3, it is proposed the RSC is still kept in the Solicitation message |  |  |  |  |  |  |  |  |
| Summary of change: |  | A candidate indication is included in the Solicitation message to represent the candidate U2U relay discovery. RSC is still kept in the Solicitation message. Then the relay needs to determine the RSC is matched by itself when to determine the response. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Above issues still exist which may cause the Candidate U2U relay discovery to be failed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.4. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

##### 6.3.2.4.4	Candidate 5G ProSe UE-to-UE Relay Discovery

This procedure for candidate 5G ProSe UE-to-UE Relay Discovery to support the negotiated Relay reselection as described in clause 6.7.4 when the discoverer End UE discovers a candidate 5G ProSe UE-to-UE Relay.

The procedure for 5G ProSe UE-to-UE Relay Discovery with Model B (see clause 6.3.2.4.3) is used with the following differences:

-	Step 1: In the 5G ProSe UE-to-UE Relay Discovery Solicitation message the RSC is still included, the User Info ID of the discoveree 5G ProSe End UE is replaced with the User Info ID of a candidate 5G ProSe UE-to-UE Relay, a candidate relay discovery indication is additionally included to represent for candidate 5G ProSe UE-to-UE Relay Discovery. If the 5G ProSe End UE receives the Layer-2 ID of the candidate 5G ProSe UE-to-UE Relay in a Link Modification Request message, it may set the Layer-2 ID of the candidate 5G ProSe UE-to-UE Relay as the Destination Layer-2 ID.

-	Step 2 and step 3 are skipped if a candidate relay discovery indication is present  in the received 5G ProSe UE-to-UE Relay Discovery Solicitation message.

-	Step 4: If a 5G ProSe UE-to-UE Relay matches the User Info ID of a candidate 5G ProSe UE-to-UE Relay and the RSC received in the 5G ProSe UE-to-UE Relay Discovery Solicitation then it sends the 5G ProSe UE-to-UE Relay Discovery Response and does not include the User Info ID of the discoveree 5G ProSe End UE.

* * * * End of Change * * * *