3GPP TSG-WG SA2 Meeting #159	S2-2310547

Xiamen, China, Oct 09-14, 2023	(revision of S2-230xxxx)

Source:	Huawei, HiSilicon

Title:	Key Issue for WT#2.1

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: Propose Key Issue for WT#2.1.

1. Introduction/Discussion

In XR service, several media streams could be multiplexed on a single IP 5-tuple, and these streams may have different QoS requirements, for example,

For a single RTP session, to reduce the possibility of failure caused by NAT device, the video and audio streams are usually transported via the same IP 5-tuples, demultiplexed via field in RTP header [x], e.g. “PT” field; or

The streams of different layers of video may have different QoS requirements and transmitted via the same IP 5-tuple.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70.

# * * * * First change * * * *

5.x	Key Issue #x - Traffic detection and QoS flow mapping for multiplexed data flows

5.x.1	Description

In XR service, several media streams could be multiplexed on a single IP 5-tuple, and these streams may have different QoS requirements, for example,

For a single RTP session, video and audio streams could be transported via the same IP 5-tuples, demultiplexed via field in RTP header [x], e.g. “PT” field; or

The streams of different layers of video may have different QoS requirements and transmitted via the same IP 5-tuple.

This key issue proposes study traffic detection and QoS Flow mapping for different media types multiplexed data flows within a single end-to-end transport connection.

- How to identify streams with different QoS requirements within a single transport connection.

- How to map streams with different QoS requirements into different QoS flows.

- What information needs to be provided from AF for traffic detection.

# * * * *Second change * * * *

2	References

[x]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

# * * * * End of changes * * * *