3GPP TSG-WG SA2 Meeting #159	S2-2310548

Xiamen, China, Oct 09 – 13, 2023	(revision of S2-230xxxx)

# Title:	[DRAFT] LS reply on Data rate monitoring

# Response to:	LS (C4-233573 / S2-2310108) from CT4

# Release:	Release 18

# Work Item:	XRM

Source:	SA2

To:	CT4

Cc:	CT3

Contact Person:

#### Name:	Xinpeng Wei

#### Tel. Number:

#### E-mail Address:	weixinpeng@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	N/A

- Overall description

SA2 thanks CT4 for the liaison S2-2310108 (C4-233573) on data rate monitoring and would like to provide the following answers.

Question 1: the data rate information that the UPF shall expose.

For instance, if the (PFCP) Measurement Period IE (indicating the period to generate periodic measurement reports) is set to 10 seconds and the monitoring averaging window is 2 seconds, should the UPF measure the data rate during 2 seconds only once within the 10 seconds and report this measurement, or should the UPF measure the data rate up to 5 times within the 10 seconds and report the minimum, maximum and average data rate over the measurement period?

Reply#1:

In SA2’s understanding, UPF should measure the data rate up to 5 times within the 10 seconds and report the minimum and maximum data rate over the measurement period as defined in QoS monitoring mechanism.

Question 2: what is the standardized value of the monitoring averaging window for non-GBR QoS flows?

Reply#2:

AF can provide the monitoring averaging window for both GBR and non-GBR QoS flows.

- Actions

To CT4:

ACTION: 	SA2 asks CT4 to take the above information into account.

3	Dates of next TSG SA WG 2 meetings

TSG-SA2 Meeting #160		November 13 – 17, 2023			Chicago, USA