SA WG2 Meeting #S2-159	S2-2310614

09 - 13 October, 2023, Xiamen, P.R. China	(revision of S2-231xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0052 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification to the Registration description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During the registration proceudre, AMF may include Ranging/SL Positioning Support indicator in the Registration Accept message to indicate that the serving network supports Ranging/SL Positioning. In the clause 5.2.3 (SL Positioning Server UE Discovery & Selection), the registration description is not needed, a Note is enough. |  |  |  |  |  |  |  |  |
| Summary of change: |  | During registration, the AMF may include Ranging/SL Positioning Support indicator in the Registration Accept. Removing the registration description from clause 5.2.3 and adding a Note to clarify. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The registration description is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.2.3	SL Positioning Server UE Discovery & Selection

SL Positioning Server UE Discovery & Selection is performed by the Target UE, when it meets one or more of the following criteria:

-	The Target UE and the discovered SL Reference UEs are currently not served by a network supporting Ranging/SL Positioning (e.g. because they are out-of-coverage or the serving network does not support Ranging/SL Positioning). SL Reference UE and Target UE may indicate whether its serving network supports Ranging/SL Positioning or not to each other. The indication of serving network not supporting Ranging/SL Positioning can be reused to notify each other when UE has no NAS connection available.

NOTE:	If the serving network supports Ranging/SL Positioning, it may provide Ranging/SL Positioning Support indicator to UE as described in the clause 6.3.2.

Editor's note:	Need to determine if such indication is provided during discovery or capability exchange, and whether or not it requires RAN WG2 involvement.

-	The Target UE is not capable of performing SL Positioning Server UE functionalities.

-	The SL Reference UEs are not capable of performing SL Positioning Server UE functionalities.

The discovery of SL Positioning Server UE follows the same principles as specified in clause 5.2.1. The UE can indicate its role “SL Positioning Server UE” in its list of supported roles during discovery, if it is authorized to be a SL Positioning Server UE in a given PLMN as per the Authorization and Provisioning for Ranging/SL positioning service as specified in clause 5.1.

The Target UE shall discover and select a SL Positioning Server UEs that are in the same or different serving PLMN of the Target UE and the SL Reference UE(s).

Editor's note:	Other criteria for the selection of SL Positioning Server UE is FFS, and can be coordinated with RAN WGs.

Next CHANGE

### 6.3.2	Registration procedure

The Registration procedure for UE is performed as defined in TS 23.502 [3] clause 4.2.2.2 with the following additions:

-	The UE includes the Ranging/SL Positioning Capability as defined in clause 4.3.1 as part of the "5GMM capability" in the Registration Request message. The AMF stores the Ranging/SL Positioning Capability for Ranging/SL Positioning operation.

-	If the Ranging/SL Positioning Capability is included in the "5GMM capability", the 5G ProSe Capability as defined in TS 23.304 [7] or the V2X Capability as defined in TS 23.287 [6] shall also be included.

-	The AMF obtains the Ranging/SL positioning subscription data as part of the user subscription data from UDM during UE Registration procedure using Nudm_SDM service as defined in clause 4.2.2.2.2 of TS 23.502 [3].

-	The AMF determines whether the UE is authorised to use Ranging/SL positioning services based on UE's Ranging/SL Positioning Capability and the Ranging/SL positioning Service Authorisation included in the subscription data received from UDM. The AMF stores the authorized Ranging/SL Positioning Capability.

-	The AMF sends the authorized Ranging/SL Positioning Capability for Ranging/SL positioning operation to PCF. Based on the received Ranging/SL Positioning Capability from the AMF, the PCF provides the PC5 QoS parameters related to RSPP transport to AMF. The AMF stores such information as part of the UE context.

-	If the UE is authorised to use Ranging/SL positioning services, then the AMF shall include in a NGAP message sent to NG-RAN:

-	"Ranging/SL Positioning authorised" information, including one or more of the following:

-	whether the UE is authorized to use Ranging/SL Positioning over PC5;

-	whether the UE is authorized to act as a Located UE;

-	whether the UE is authorized to act as a SL Positioning Server UE.

-	the PC5 QoS parameters related to RSPP transport used by the NG-RAN for the resource management of UE's PC5 transmission for Ranging/SL Positioning services in network scheduled mode.

-	If the UE is authorised to use Ranging/SL positioning services, AMF may include Ranging/SL Positioning Support indicator in the Registration Accept message to indicate that the serving network supports Ranging/SL Positioning.

-	If the UE is authorised to use Ranging/SL positioning services, then the AMF should not initiate the release of the signalling connection after the completion of the Registration procedure. The release of the signalling connection relies on the decision of NG-RAN, as specified in TS 23.502 [3].

END OF CHANGES