3GPP TSG-SA WG2 Meeting #159	S2-2310707

October 09 – 13, 2023, Xiamen, China   	(Rev of)

# Title:	(DRAFT) Reply LS on Clarification related to Network Slicing enhancement

# Response to:	(S2-2310112/ C4-233776) Response to LS on Clarification related to Network Slicing enhancement

# Release:	Rel-18

# Work Item:	eNS_Ph3

Source:	SA2

To:	CT4

Cc:	CT1, CT3

Contact Person:

Name:	Peter Hedman

Tel. Number:

E-mail Address:	peter (dot) hedman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: N/A

1. Overall Description:

SA2 thanks CT4 for the LS on Clarification related to Network Slicing enhancement.

SA2 discussed the following CT4 questions and agreed on the following answers:

Clause 5.15.6 states:

If the serving AMF supports the Network Slice Replacement feature and is configured to use the NSSF for Network Slice Replacement triggering, the AMF subscribes with the NSSF of the VPLMN for notifications when an HPLMN S-NSSAI needs to be replaced with an Alternative S-NSSAI, in addition to notifications for the Serving PLMN S-NSSAIs. The NSSF of the VPLMN shall subscribe with the NSSF of the HPLMN for notifications when an HPLMN S-NSSAI needs to be replaced with an Alternative S-NSSAI.

Question 1: Does the above requirement apply only to Home Routed roaming use case? Or also to LBO roaming use case?

SA2 answer:
The functionality may be applied also to LBO PDU Sessions, but the referenced text is to be applied also before any PDU Session been established and the Network Slice Replacement feature is applied per UE for all PDU Sessions of the UE associated to the S-NSSAI to be replaced.

Clause 5.15.19 states:

, the AMF provides the Alternative S-NSSAI for this S-NSSAI in the Allowed NSSAI and in the Configured NSSAI, if not included yet, and the mapping between S-NSSAI(s) to Alternative S-NSSAI(s) to the UE in UE Configuration Update message as follows:

-    for non-roaming UEs, the AMF provides the mapping of the S-NSSAI to the Alternative S-NSSAI to the UE.

-    for roaming UEs when the VPLMN S-NSSAI has to be replaced by a VPLMN Alternative S-NSSAI, the AMF provides the mapping of the VPLMN S-NSSAI to the Alternative VPLMN S-NSSAI to the UE.

-    for roaming UEs when the HPLMN S-NSSAI has to be replaced by an Alternative HPLMN S-NSSAI, the AMF provides the mapping of the HPLMN S-NSSAI to the Alternative HPLMN S-NSSAI to the UE.

Question 2: In the above requirement, HPLMN S-NSSAI replaced with HPLMN Alternative S-NSSAI is applicable only in case of Home routed roaming use case? Or also to LBO roaming use case?

SA2 answer:
The functionality applies also to LBO PDU Sessions.

In the current TS 23.502,

Clause 4.3.2.2.1 states:

For roaming scenario in local breakout (LBO), the AMF also sends the corresponding S-NSSAI of the HPLMN from the Mapping Of Allowed NSSAI or Mapping Of Partially Allowed NSSAI to the SMF. If the the AMF determines to replace the HPLMN S-NSSAI received from the UE with the HPLMN Alternative S-NSSAI or the AMF receives the HPLMN Alternative S-NSSAI and the HPLMN S-NSSAI provided by the UE, the AMF sends both HPLMN S-NSSAI and HPLMN Alternative S-NSSAI to the SMF.

Question 3: In the above requirement, why would AMF replace HPLMN Alternative S-NSSAI in case of LBO roaming? (for example, why would congestion or unavailability of a slice in HPLMN impact an equivalent slice being used in VPLMN by an LBO roaming UE?)

SA2 answer:
Even if the main reasons were congestion and unavailability the possibility to do a network slice replacement was added generically and also an LBO PDU Session has an associated HPLMN S-NSSAI that was considered to be possible to replace, but SA2 does not expect any major logic to be applied by SMF for LBO PDU Session other than providing the HPLMN S-NSSAI e.g. to CHF.

2. Actions:

To CT4:

ACTION: 	SA2 kindly asks CT4 to take the above into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #160   November 13 – 17, 2023 	Chicago, US

TSG-SA2 Meeting #161   2024-02-26 -  2024-03-01	Athens, Greece