3GPP TSG SA WG2 Meeting #159	S2-2310867

# Xiamen, Oct 09 – 13, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5025 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of QoS flow repeporting for event triggered case 23.501 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are some ambiguous text present in the clause 5.8.2.18 of TS 23.501 as: - If the Reporting frequency indicates "event triggered", a Reporting threshold for each parameter in the QoS parameter(s) to be measured and a Minimum waiting time are provided as well. The UPF shall send a report when the measurement result matches or exceeds the indicated Reporting threshold. Subsequent reports shall not be sent by the UPF during the Minimum waiting time. If measurement results are received during the Minimum waiting time, the UPF shall report the minimum and the maximum measurement result when the Minimum waiting time is over. From the bold text it is unclear that UPF is getting multiple such measurement results during the minimum waiting time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified the QoS flow reporting for the event triggered case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Descriptions would remain ambiguous |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.8.2.18	QoS Flow related QoS monitoring and reporting

The SMF may configure the UPF to perform QoS monitoring for a QoS Flow and to report the monitoring results with the help of the following parameters provided in the Session Reporting Rule (SRR) described in clause 5.8.5.11:

-	QoS parameter(s) to be measured indicating the subject of the QoS monitoring as defined in clause 5.45;

-	Reporting period indicating the time interval in which a new measurement result and a potential report has to be available. If no measurement result is available to the UPF within the Reporting period, the UPF shall report a measurement failure;

-	Reporting frequency indicating the type of the reporting as "periodic" or "event triggered":

-	If the Reporting frequency indicates "periodic", the UPF shall send a report each time the reporting period is over.

-	If the Reporting frequency indicates "event triggered", a Reporting threshold for each parameter in the QoS parameter(s) to be measured and a Minimum waiting time are provided as well. The UPF shall send a report when the measurement result matches or exceeds the indicated Reporting threshold. Subsequent reports shall not be sent by the UPF during the Minimum waiting time. If multiple measurement results are received during the Minimum waiting time, the UPF shall report the minimum and the maximum measurement results when the Minimum waiting time is over.

-	(Optional) Target of the reporting and Indication of direct event notification indicating that the UPF shall send the reports to a different NF than the SMF (i.e. to the Local NEF or the AF). The NF is described by a Notification Target Address and a Notification Correlation ID. The SMF can also indicate that the UPF shall send the reports to both, the NF indicated by the Target of reporting and to the SMF. If so, the UPF shall send the reports to the SMF as well. If the Indication of direct event notification is not provided, the UPF shall send the reports to the SMF.

-	(Optional) Reporting suggestion information as defined in clause 5.8.2.17 applicable to Target of the reporting to reduce the UPF performance impacts.

The UPF sends reports to the SMF in the form of QoS Monitoring Report as described in clause 5.8.5.12 and reports to the Local NEF or the AF via the Nupf_EventExposure_Notify service operation described in clause 7.2.29.

END OF CHANGES