3GPP TSG-WG SA2 Meeting #159	S2-2310878

# Xiamen, China, October 9 – 13, 2023		(revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2118 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  | ME | X |  | Radio Access Network |  |  |  |  | Core Network | X |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: |  | URSP for 5G-RG and FN-RG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: |  | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  | SA2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  | 5WWC_Ph2 |  |  |  |  |  |  |  |  | Date: |  | 2023-10-09 |  |  |  |
| Category: |  | F |  |  |  |  |  |  |  |  | Release: |  | Rel-18 |  |  |  |
|  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |  |  |
| Reason for change: |  |  | The specification of URSP for 5G-RG and FN-RG is currently defined in TS 23.503 and in TS 23.316 which makes difficult to have a consistent reading of the specification, mainly from external SDOs which are reponsable for specification of RGs. It is proposed to move all normative text from TS 23.503 to TS 23.316 leaving in TS 23.503 the pointer to TS 23.316 The second issue is related to the definition of the URSP parameters and policies which are not applicable to the 5G-RG and FN-RG, therefore they should not be provided within the URSP, or if they are provided, the 5G-RG and W-AGF need to ignore them. The URSP parameters to be ignored are: - PIN ID since the PIN feature is not supported for FN-RG - The Connectivity Group ID since the W-AGF is not aware of any FN-RG local configuration - The ProSe Layer-3 UE-to-Network Relay Offload Policy, PDU Session Pair ID, RSN, ProSe Multi-path Preference of URSP policy since related to feature not supported by FN-RG. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  | The following modification are porposed: - Move text from 23.503 clause 6.2.1.1 referring to AUN3 device into 9.5.2.1 - Move the text from clause 9.6.4 to clause 9.5.2.2 - Added specification on URSP policy not supported by FN-RG - The text in 9.6.4 has been moved in 9.5.2.2 and new text reflecting the change is proposed in order to maintain correct reference from other specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  | Uncorrect UE behaviour and uncorrect support of features |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Clauses affected: | 9.5.2, 9.5.2.1, 9.5.2.2 and 9.6.4 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | N |  |  |
| Other specs |  | x | Other core specifications |  |
| affected: |  | x | Test specifications |  |
| (show related CRs) |  | x | O&M Specifications |  |
| Other comments: |  |  |  |  |
| This CR's revision history: |  |  |  |  |

FIRST CHANGE

### 9.5.2	UE access selection and PDU Session selection related policy information

#### 9.5.2.1	5G-RG

This clause specifies the delta related to UE policy distribution defined in TS 23.503 [4] clause 6.1.2.2 and related to URSP defined in TS 23.503 [4] clause 6.6. for 5G-RG.

If the PCF provides the URSP policy to the 5G-RG, the PCF should not include NSWO indication, PIN ID. or any ANDSP policies, ProSe Layer-3 UE-to-Network Relay Offload Policy, PDU Session Pair ID, RSN, ProSe Multi-path Preference of URSP policy. The 5G-RG shall ignore any of the listed parameters and policies, if received from the 5GC. The 5G-RG shall use the URSP policy as specified in TS 23.503 [4], for example for the association of application and PDU session, slices, etc.

The URSP indicates for the application of Auto-Configuration Server (ACS) which PDU session type, NSSAI and/or DNN is to be used. The 5G-RG establishes the connectivity to the management entity (e.g. ACS) via user plane connection on a PDU session according to the URSP.

UE Policy procedures defined in clause 6.1.2.2 of TS 23.503 [4] are applicable as follows:

-	Roaming is not applicable to W-5GAN access in this release of specification.

In case of Authenticable Non-3GPP (AUN3) device behind a 5G-RG each AUN3 device are managed in 5G-RG as separate devices each of them with its own NAS and states, therefore from point of view of 5GC, the AUN3 device is seen as a single device and the URSP received in the NAS dedicated to an AUN3 device shall apply to this AUN3 device and the 5G-RG shall apply URSP received within NAS for AUN3 device acting on behalf of AUN3 device. Therefore to support the case when AUN3 devices may be connected via 5G-RG, specific URSP rules may be configured by the PCF for the SUPI associated with the AUN3 device.

UE Route Selection Policy information targeting an AUN3 device (i.e. sent to a 5G-RG in the NAS connection corresponding to an AUN3 device) follows the structure defined in clause 6.2.2 of TS 23.503 [4] with following differences:

-	As an AUN3 can have only one PDU Session, its URSP shall contain a match all TD.

In order to support the case when NAUN3 devices may be connected via 5G-RG, specific URSP rules may be configured by the PCF on 5G-RG.

URSP rules for NAUN3 devices connected to 5G-RG follow the structure defined in clause 6.6.2 of TS 23.503 [4] and may contain any combination of the following traffic descriptors:

-	IP Descriptors: For IP traffic from NAUN3 devices connected to 5G-RG, IP descriptors are matched against header information contained in IP packets sent by NAUN3 devices; IP descriptors are only applicable for traffic from NAUN3 devices if network address translation (NAT) is performed for that traffic.

-	Non-IP descriptors: For Ethernet traffic from NAUN3 devices connected to 5G-RG, Non-IP descriptors are matched against header information contained in Ethernet frames sent by NAUN3 devices.

-	Connectivity Group ID: For traffic from a NAUN3 device connected to 5G-RG, Connectivity Group ID in the URSP rule is matched against the Connectivity Group ID that the NAUN3 device is associated with (see clause 4.10b).

#### 9.5.2.2	FN-RG

This clause specifies the delta related to UE policy distribution defined in TS 23.503 [4] clause 6.1.2.2 and related to URSP defined in TS 23.503 [4] clause 6.6 for 5G-RG.

If the PCF provides the URSP rules related to FN-RG to the W-AGF, the PCF should not include NSWO indication, PIN ID, Connectivity Group ID, Connection Capabilities and the Access Type preference. The PCF should not provide ANDSP policies, ProSe Layer-3 UE-to-Network Relay Offload Policy, PDU Session Pair ID, RSN, ProSe Multi-path Preference of URSP policy. The W-AGF shall ignore any of the listed indications and policies if received from the 5GC.

A W-AGF needs to be able to determine the (DNN, S-NSSAI) parameters of the PDU Sessions it requests on behalf of a FN RG. Therefore the URSP is used by a W-AGF on behalf of FN-RG to determine association of traffic from FN-RG.

The W-AGF shall use the URSP policy as specified in TS 23.503 [4] with the following modifications:

-	Traffic descriptor;

-	the Application Descriptor is not applicable;

-	the DNN is not applicable;

-	The Connection Capabilities Descriptor is not applicable.

NOTE 1:	The FN-RG initiates the W-5GAN session with the W-AGF, for example PPPoE, and consequently the W-AGF does not receive any indication of the application used for that session (e.g. whether it used for web browsing or for any specific application) and any DNN indication from the application, hence the policy including the Application Descriptors and/or DNN will never match the traffic.

If the PCF sends UE policy (e.g. URSP), the W-AGF shall store it for the duration that FN-RG is registered. When the FN-RG is deregistered, the UE policy can be removed. Whether it is done immediately, or after a certain period (e.g. for quick recovery from disconnection or fault), or stored permanently it is left to implementation and is out of the scope of this TS.

If the URSP for the FN-RG are present in W-AGF (e.g. pre-configured or received from PCF) the W-AGF shall use them as defined for a UE with URSP.

If the URSP for the FN-RG are not present in W-AGF, the W-AGF acts based on local configuration, as defined for a UE without URSP.

The W-AGF requests PDU Sessions upon data trigger (e.g. PPPoE , DHCP, etc.) received over a data path identified by a VLAN and a GLI; this is defined in BBF specifications (BBF TR-456 [9] and BBF TR-470 [38]).

Thus the W-AGF needs to be configured to request different PDU Sessions for different VLAN(s) terminated at different FN RG(s).

NOTE 1:	The VLAN configuration depends on the served FN RG as a W-AGF service area can serve different Wireline access networks with different VLAN configurations.

The corresponding W-AGF configuration about parameters of the PDU Sessions to request for a GLI corresponds to URSP that the W-AGF receives from the PCF for a SUPI corresponding to a GLI.

The URSP(s) may be used to map VLAN(s) at transport level (S-tags as defined in BBF TR-470 [38]) on the V interface of the W-AGF (identifying the target service of the corresponding data flows, e.g. internet / IMS Voice / IPTV) towards Route Selection components including PDU Session type, DNN, S-NSSAI, SSC mode, etc.

NOTE 2:	UDR policy data related with a FN-RG subscription (UE Policy Section, see clause 5.4.2.3 of TS 29.519 [39]) can be configured accordingly.

It is assumed that the FN-RG configuration (provided via BBF TR-069 [18]/BBF TR-369 [19]), the URSP rules and the local configuration in the W-AGF are consistent with each other. If the W-AGF detects conflicting requirements based on URSP, local configuration, or requests from the FN-RG, then the URSP rules takes precedence since they are considered the most updated and aligned to the current 5G system conditions.

UE Policy procedures defined in clause 6.1.2.2 of TS 23.503 [4] are applicable with the following modification:

-	Roaming is not applicable to W-5GAN access in this release of specification.

SECOND CHANGE

### 9.6.4	URSP for FN RG

The  URSP for FN-RG is defined in clause 9.5.2.2

END OF CHANGES