SA WG2 Meeting #159	S2-2310898

# Xiamen, PRC, October 9-13th, 2023        	  	  			       		          (revision of S2-231)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2120 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | control of agregated traffic (of 5G-RG and AUN3 devices) that uses a 5G-RG line |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | How to control aggregated traffic (of 5G-RG and AUN3 devices) that uses a 5G-RG line has been left open at SA2 #157 The solution using RG Level Wireline Access Characteristics has very limited impacts to 3GPP specifications (see this CR) and allows a dynamic control of the aggregated traffic done by the W-AGF: if multiple AUN3 devices are registered but only one of them is currently exchanging traffic the active AUN3 device can use up to its subscribed AMBR. With PCF based solutions, the PCF(s) would need to reduce the AMBR of all AUN3 devices in such a way that the sum of the AMBR(s) for the 5G-RG and of the AMBR of each AUN3 device would be below the maximum value allowed for the line. In that case (PCF control) if multiple AUN3 device are registered but only one of them is currently exchanging traffic this AUN3 device would not be able to use up to its subscribed AMBR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The RG Level Wireline Access Characteristics sent to the W-AGF for a 5G-RG may contain a maximum bit rate for the aggregated traffic of the 5G-RG and of the AUN3 devices served by this 5G-RG. The W-AGF uses this information to limit the maximum bit rate. of the aggregated user plane traffic of the 5G-RG and of the AUN3 devices served by this 5G-RG. This maximum bit rate in Level Wireline Access Characteristics is defined by BBF and CableLabs specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to control agregated traffic (of 5G-RG and AUN3 devices) that uses a 5G-RG line is left unspecified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10c; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 4.10c	Authenticable Non-3GPP devices behind 5G-RG

This clause defines the support of AUN3 devices, i.e. Authenticable Non-3GPP devices (AUN3) as defined in clause 3.1, behind a 5G-RG. This clause applies only to 5G-RG connected via wireline access.

Figure 4.10c-1 shows the architecture for support of AUN3 device.

Figure 4.10c-1: AUN3 device behind 5G-RG

Differentiated services for AUN3 devices behind 5G-RG are provided as specified below:

-	Each AUN3 device has its own UDM/UDR subscription data including its own SUPI and policy control subscription data.

-	The interface between 5G-RG and AUN3 devices is out of scope of 3GPP.

-	In order to serve the AUN3 device in 5GC, a 5G-RG issues a NAS register and handles RM and CM related signalling on behalf of an AUN3 device that it is requesting to be served and relays EAP signalling between the AUN3 device and the 5GC.

-	A 5G-RG serving an AUN3 device establishes a single PDU Session on behalf on this AUN3 device.

-	The AMF and the 5G-RG maintain a separate NAS connection per AUN3 device. This includes maintaining a GUTI and NAS (RM, CM, security, etc.) context per AUN3 device.

-	A 5G-RG shall be connected to the 5GC (be in RM-REGISTERED and CM-CONNECTED mode) over Wireline access to serve an AUN3 device: the 5G-RG shall not issue a NAS register or service request on behalf of an AUN3 device if it is itself not registered and connected to the 5GC.

-	The 5G-RG is configured with URSP for each AUN3 devices it serves. The UE PCF selected by the AMF at the registration of an AUN3 device sends this URSP to 5G-RG via the AMF and the NAS connection of the AUN3 device.

-	The AUN3 devices and the 5G-RG belong to the same PLMN.

-	A 5G-RG uses default values, which are the same for all AUN3 devices it serves, to populate the parameters in the Registration Request message built on behalf of an AUN3 device. For example, the 5G-RG issues the Registration Request with no S-NSSAI and the AMF selects the default S-NSSAI in the subscription of the AUN3 device.

-	There shall be a separate N2 connection per AUN3 device that is in state CM-CONNECTED.

-	The W-CP and W-UP protocols shall be able to manage multiple Registrations and PDU Sessions for different SUPIs between the same pair of 5G-RG and W-AGF. In particular, W-CP needs to be able to differentiate NAS messages related to a 5G-RG and to each different AUN3 device served by this 5G-RG and W-UP needs to distinguish between user plane packets for a 5G-RG and each different AUN3 device served by this 5G-RG.

-	When the registration of an AUN3 device has successfully completed, the 5G-RG establishes a PDU Session on behalf of the AUN3 device. This PDU Session is handled by 5GC as part of the AUN3 subscription and is associated with the SUPI of AUN3 device. An AUN3 device can at a given time only use a single PDU Session. The parameters to establish this PDU session are based on the URSP (if any) for the AUN3 device.

-	Different QoS parameters may apply to PDU sessions of different AUN3 devices.

-	Roaming is not applicable to subscriptions for AUN3 devices.

-	The RG Level Wireline Access Characteristics sent to the W-AGF for a 5G-RG may contain a maximum bit rate for the aggregated traffic of the 5G-RG and of the AUN3 devices served by this 5G-RG. The W-AGF uses this information to limit the maximum bit rate. of the aggregated user plane traffic of the 5G-RG and of the AUN3 devices served by this 5G-RG.

NOTE 1: The coding of the maximum bit rate in RG Level Wireline Access Characteristics is defined by BBF and CableLabs specifications.

NEXT CHANGE

END OF CHANGES