3GPP TSG SA WG2 Meeting #159	S2-2310919

# 09 - 13 October, 2023, Xiamen, P.R. China								(Revision of S2-231xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5029 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on access to SNPN services via SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - When access to localized services is specified in Rel-18, the UE may access home network services via hosting network. If the hosting network and home network are both SNPN, it introduces a new scenario that both underlay and overlay networks are SNPNs, which is not described in current specification. - There is no specific requirements to support this scenario but describing the same principle with other scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new clause to describe the principle of accessing SNPN services via SNPN; - add corresponding new figure in annex D.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.8a (new), Annex D.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 5.30.2.8a	Access to stand-alone non-public network services via stand-alone non-public network as hosting network

When access to localized service is supported, a UE that has successfully registered with a SNPN as the hosting network may perform another registration via the SNPN User Plane with an SNPN as the home network (using the credentials of that SNPN) following the same architectural principles as specified in clause 4.2.8 (including the optional support for PDU Session continuity between PLMN and SNPN using the Handover of a PDU Session procedures in clauses 4.9.2.1 and 4.9.2.2 of TS 23.502 [3]) and the SNPN taking the role of "Untrusted non-3GPP access" of the SNPN, i.e. using the procedures for Untrusted non-3GPP access in clause 4.12.2 of TS 23.502 [3]. Annex D, clause D.3 provides additional details.

NOTE:	QoS differentiation in the SNPN as the hosting network can be provided on per-IPsec Child Security Association basis by using the UE or network requested PDU Session Modification procedure described in clause 4.3.3.2 of TS 23.502 [3]. In the SNPN as the home network, N3IWF determines the IPsec child SAs as defined in clause 4.12 of TS 23.502 [3]. The N3IWF is preconfigured by SNPN as the home network to allocate different IPsec child SAs for QoS Flows with different QoS profiles.

To support QoS differentiation in the SNPN as the hosting network with network-initiated QoS, the mapping rules between the SNPN as the hosting network and the SNPN as the home network are assumed to be governed by an SLA including: 1) mapping between the DSCP markings for the IPsec child SAs on NWu and the corresponding QoS, which is the QoS requirement of the SNPN as home network and is expected to be provided by the SNPN as the hosting network, and 2) N3IWF IP address(es) in the SNPN as home network. The non-alteration of the DSCP field on NWu is also assumed to be governed by an SLA and by transport-level arrangements that are outside of 3GPP scope. The packet detection filters in the SNPN as the hosting network can be based on the N3IWF IP address and the DSCP markings on NWu.

To support QoS differentiation in the SNPN as the hosting network with UE-requested QoS, the UE can request for an IPsec SA the same 5QI from the SNPN as the hosting network as the 5QI provided by the SNPN as the home network. It is assumed that UE-requested QoS is used only when the 5QIs used by the SNPN as the home network are from the range of standardized 5QIs. The packet filters in the requested QoS rule can be based on the N3IWF IP address and the SPI associated with the IPsec SA.

Refer to clause D.7 for details on how to support QoS differentiation.

When the UE accesses the SNPN over NWu via a SNPN as hosting network, the AMF in the SNPN as home network shall send an indication toward the UE during the Registration procedure to indicate whether an IMS voice over PS session is supported or not.

**** Second Change ****

# D.3	Support for access to PLMN services via Stand-alone Non-Public Network, access to Stand-alone Non Public Network services via PLMN and access to Stand-alone Non Public Network services via Stand-alone Non Public Network

Figure D.3-1: Access to PLMN services via Stand-alone Non-Public Network

NOTE 1:	The reference architecture in Figure D.3-1 and Figure D.3-2 only shows the network functions directly connected to the UPF or N3IWF and other parts of the architecture are same as defined in clause 4.2.

In order to obtain access to PLMN services when the UE is camping in NG-RAN of Stand-alone Non-Public Network, the UE obtains IP connectivity, discovers and establishes connectivity to an N3IWF in the PLMN.

In the Figure D.3-1, the N1 (for NPN) represents the reference point between UE and the AMF in Stand-alone Non-Public Network. The NWu (for PLMN) represents the reference point between the UE and the N3IWF in the PLMN for establishing secure tunnel between UE and the N3IWF over the Stand-alone Non-Public Network. N1 (for PLMN) represents the reference point between UE and the AMF in PLMN.

Figure D.3-2: Access to Stand-alone Non-Public Network services via PLMN

In order to obtain access to Non-Public Network services when the UE is camping in NG-RAN of a PLMN, the UE obtains IP connectivity, discovers and establishes connectivity to an N3IWF in the Stand-alone Non-Public Network.

In Figure D.3-2, the N1 (for PLMN) represents the reference point between UE and the AMF in the PLMN. The NWu (for NPN) represents the reference point between the UE and the N3IWF in the stand-alone Non-Public Network for establishing a secure tunnel between UE and the N3IWF over the PLMN. The N1 (for NPN) represents the reference point between UE and the AMF in NPN.

Figure D.3-3: Access to Stand-alone Non-Public Network services via Stand-alone Non-Public Network

In order to obtain access to overlay Stand-alone Non-Public Network services when the UE is camping in NG-RAN of unerlay Stand-alone Non-Public Network, e.g. the hosting network providing localized services, the UE obtains IP connectivity, discovers and establishes connectivity to an N3IWF in the overlay Stand-alone Non-Public Network.

In the Figure D.3-3, the N1 (for underlay network) represents the reference point between UE and the AMF in underlay Stand-alone Non-Public Network. The NWu (for overlay network) represents the reference point between the UE and the N3IWF in the overlay Stand-alone Non-Public Network for establishing secure tunnel between UE and the N3IWF over the overlay Stand-alone Non-Public Network. N1 (for overlay network) represents the reference point between UE and the AMF in overlay Stand-alone Non-Public Network.

When using the mechanism described above to access overlay network via underlay network, the overlay network can act as authorized 3rd party with AF to interact with NEF in the underlay network, to use the existing network exposure capabilities provided by the underlay network defined in clause 4.15 of TS 23.502 [3]. This interaction is subject of agreements between the overlay and the underlay network.

**** End of Changes ****