3GPP TSG-WG SA2 Meeting #159	S2-2311108

# Xiamen, China, Oct 9, 2023 – Oct 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5055 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of the option to use TSCTSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. Clause 5.20c describes two options for AF request to provision traffic characteristics for a group; either by invoking TSCTSF, or invoking PCF via UDR. According to clause 6.1.3.28 in TS 23.503, the AF request that is targeted to a GPSI or External Group ID may contain traffic characteristics. If TSCTSF is not involved, the NEF stores the traffic characteristics (among other QoS information in the request) to the UDR, and UDR notifies the PCF for the content of the request. This means the PCF needs to compose a TSC Assistance Container and include it in PCC Rules; the TSCAC contains the given traffic characteristics, such as Periodicity, Burst Arrival Time, etc. This means that there is no need to involve the TSCTSF to compose the TSCAC, and the option to use TSCTSF can be removed from clause 5.20c. Other issues in the clause: - some steps in the description in clause 5.20c are not applicable for PCF. - PCF should process the QoS request on PDU Session basis. - PCF is not aware if the UE is added or removed from a group in middle of a PDU Session. - SM-PCF is not aware of users that are registered but do not have a PDU Session 2. Clause 5.20b.3 refers to 4.15.6.2 in TS 23.502 defining a delta on top if it. However, a specific clause 4.15.6.3e in TS 23.502 is described for this purpose, as it defines the delta on top of clause 4.15.6.2. Thus there is overlapping description in clause 5.20b in TS 23.501 and in clause 4.15.6.3e in TS 23.502. In addition, it is unclear how to select the "strictest" default QoS if multiple default QoS settings are applicable for a PDU Session. For example, 5QI consists of multiple attributes, and also e.g. Session AMBR would need to be considered in such evaluation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Remove the option to use TSCTSF in clause 5.20c. Apply the procedure in PCF on PDU Session basis. Remove the part of the description that is not applicable for PCF. 2. Change the reference to 4.15.6.3e in TS 23.502 and remove the overlapping parts from clause 5.20b.3. In case multiple default QoS settings are applicable (UE belongs to multiple groups), it is up to implementation which default QoS is applied. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TSCTSF is involved even though it does not provide any value to the flow. Incorrect, unimplementable procedure description for PCF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.20b.3, 5.20c |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR3441, TS 23.503 CR1187 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

### 5.20b.3	Support of default QoS for a group

The procedure as defined in clause 4.15.6.3e of TS 23.502 [3] can be used to provision DNN and S-NSSAI specific default QoS for a group of UEs.  The default QoS is stored in the UDR and used to set 5GS Subscribed QoS profile in Session Management Subscription data for each UE within the group, as described in clause 4.15.6.3e of TS 23.502 [3].

Mechanisms as defined in clause 5.7.2.7 are used to enforce the 5GS Subscribed QoS profile for each UE within a group for a given DNN and S-NSSAI thus to support enforcement of QoS for a group.

When a UE belongs to multiple groups, and two or more groups have different default QoS provisioned for the given DNN and S-NSSAI, the selection of the default QoS that is applied for the UE is up to implementation.

*************************** Next of Change ***************************

## 5.20c	Requesting QoS and provisioning of traffic characteristics and monitoring of performance characteristics for a group

NEF provisioning capability as defined in clause 5.20 allows an AF to request QoS and perform provisioning of traffic characteristics and monitoring of performance characteristics for a group of UEs as specified in clause 4.15.6.14 of TS 23.502 [3] and clause 6.1.3.28 of TS 23.503 [45].

If the request is targeted to an External Group ID, the NEF translates the External Group ID into Internal Group ID by invoking Nudm_SDM_Get service.

NOTE :	The AF may use application layer functionalities to handle requests for UE-to-UE traffic as defined by SA WG6.

When the PCF(s) receive the AF requested QoS information from UDR, the PCF (s) manage the AF requested QoS information for the PDU Session as follows:

-

-

-	If the DNN and S-NSSAI of the PDU Session, and the Internal Group ID of the user matches with the AF requested QoS information as received from the UDR, the PCF applies the AF requested QoS information. If the QoS resources are assigned to the PDU Session,the PCF sets the status to activated, otherwise the PCF sets the status to failed.

-

-

-	When the AF requested QoS information matches with the PDU Session and contains temporal invalidity condition:

-	Revoke AF requested QoS information for the PDU Session which is marked as active, so to e.g. to remove or modify PCC rules as defined in clause 4.16.5.2 of TS 23.502 [3] if the stop-time is reached.

-	Apply AF requested QoS information for the PDU Session, e.g. to add or modify PCC rules as defined in clause 4.16.5.2 of TS 23.502 [3] if the start-time is reached.

*************************** End of Changes ****************************