3GPP TSG-WG SA2 Meeting #159	S2-2311136

# Xiamen, China, Oct 9, 2023 – Oct 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1187 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of the option to use TSCTSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. Clause 6.1.3.28 describes two options for AF request to provision traffic characteristics for a group; either by invoking TSCTSF, or invoking PCF via UDR. According to clause 6.1.3.28 in TS 23.503, the AF request that is targeted to a GPSI or External Group ID may contain traffic characteristics. If TSCTSF is not involved, the NEF stores the traffic characteristics (among other QoS information in the request) to the UDR, and UDR notifies the PCF for the content of the request. This means the PCF needs to compose a TSC Assistance Container and include it in PCC Rules; the TSCAC contains the given traffic characteristics, such as Periodicity, Burst Arrival Time, etc. This means that there is no need to involve the TSCTSF to compose the TSCAC, and the option to use TSCTSF can be removed from clause 6.1.3.28. 2. Clause 6.1.3.28 re-defines the same parameters that are already defined in the referred clauses of 6.1.3.22, etc. This has already caused divergence between the clauses, as more parameters have been added to clause 6.1.3.22, and clause 6.1.3.28 has not been updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Remove the option to use TSCTSF in clause 6.1.3.28. 2. Remove the duplicative definition of the input parameters. Keep only the reference to clause 6.1.3.22, since all other referred clauses are already referred from 6.1.3.22. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary involvement of TSCTSF. Divergence between service parameters depending on whether the service parameters are stored to UDR or sent directly to PCF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.28 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.501 CR5055, TS 23.502 CR4541 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

#### 6.1.3.28	AF requested QoS for a UE or group of UEs not identified by a UE address

The AF may request that the data session(s) for a UE (identified by a GPSI) or a group of UEs (identified by an External Group ID) for a specific DNN and S-NSSAI are set up with a specific QoS (e.g. with a specific Requested 5GS delay for a low latency service or with monitoring and reporting the Packed Delay Variation) and priority handling.

The AF request may contain the parameters as described in clause 6.1.3.22.

The PCF receives the AF requested QoS information from the UDR as described in clause 4.15.6.14 of TS 23.502 [3]. If the AF requested QoS information contains temporal invalidity condition, the PCF activates, modifies, or removes PCC rules corresponding to the QoS information as needed based on the invalidity conditions.

*************************** End of Changes ****************************