3GPP TSG-WG SA2 Meeting #159	S2-2311145

# Xiamen, China, October 9 – 13, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0965 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MTLF-based ML Model Accuracy Monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_ph3 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - ML model accuracy information indicates the accuracy of the ML model, which includes the accuracy value or ML model metric as described in clause 6.2A.2. In the procedure for MTLF-based ML Model Accuracy Monitoring in clause 6.2E.2, it does not require AnLF to monitor accuracy, i.e. the accuracy value or metric is calculated by MTLF rather than AnLF. Thus, the ML model accuracy information cannot be provided by AnLF. Furthermore, ML model accuracy information is not used in the following procedures in clause 6.2E.2. - From the beginning, Nnwdaf_MLModelMonitor_Register service operation is introduced in AnLF-assisted MTLF ML model accuracy monitoring as described in clause 6.2E.3.2, in this case, AnLF is required to have accuracy monitoring capability. Aferwards, Nnwdaf_MLModelMonitor_Register service operation is reused in clause 6.2E.2, as a result that NWDAF containing AnLF registers the use of the ML model with the NWDAF containing MTLF and indicates its capability of sending Analytics feedback information at step 4.However, for MTLF-based ML Model Accuracy Mornitoring in 6.2E.2, only MTLF is required to have accuracy monitoring capability while AnLF is not necessarily required to have accuracy monitoring capability. Therefore, it is proposed that AnLF registers capability of accuracy monitoring and delivering analytics feedback information seperately by using different parameters in clause 7.9. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove ML model accuracy information provided by AnLF in clause 6.2E.2. - Separate monitoring capability and delivering Analytics feedback information capability in clause 7.9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous description on MTLF-based ML Model Accuracy Monitoring. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.2, 7.9.1, 7.9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.2E.2	Procedure for MTLF-based ML Model Accuracy Monitoring

Figure 6.2E.2-1 illustrates the procedure for monitoring the accuracy of the provisioned ML model using newly collected data. NWDAF containing AnLF may provide inference data to NWDAF containing MTLF for model accuracy monitoring and the NWDAF containing MTLF determines retraining or re-provisioning of the ML model.

Figure 6.2E.2-1: Procedure for MTLF-based ML Model Accuracy Monitoring

1.	An analytics consumer initiates a subscription for analytics exposure services towards an NWDAF containing AnLF.

2.	The NWDAF containing AnLF requests an ML model from the appropriate NWDAF containing MTLF, using the Nnwdaf_MLModelProvision_Subscribe service operation. The NWDAF containing AnLF may include an ML Model accuracy threshold which is used as an indicator to execute the accuracy monitoring operations as defined in clause 6.2A.2. NWDAF containing AnLF may include a DataSetTag (see clause 6.2B.1) and/or ADRF ID, which is used to store and fetch the inference data (including input data, prediction and the ground truth data at the time which the prediction refers to) from ADRF which are relevant for the accuracy monitoring and re-training/re-provisioning of ML model.

If the NWDAF containing AnLF receives ML model(s), the NWDAF containing AnLF sends set of tuples (unique ML model identifier and the DataSetTag and/or ADRF ID) to the NWDAF containing MTLF by invoking Nnwdaf_MLModelProvision_Subscribe service operation for subscription modification.

3.	The NWDAF containing MTLF provides trained ML model(s) to the NWDAF containing AnLF. The NWDAF containing MTLF may include an accuracy information which is used to indicate the accuracy of ML model during the training.

When the step 1 is for a subscription modification (i.e. including Subscription Correlation ID) and contains the set of tuples (unique ML model identifier and the DataSetTag and/or ADRF ID), the NWDAF containing MTLF determines the relationship between the ML model and the DataSetTag.

4.	The NWDAF containing AnLF registers the use of the ML model with the NWDAF containing MTLF to indicate its capability of sending Analytics feedback information from the analytics consumers for the ML model.

5.	Due to the registration in the previous step, the NWDAF containing MTLF may subscribe to the NWDAF containing AnLF to get Analytics feedback information from the analytics consumers for the provisioned ML model by invoking Nnwdaf_MLModelMonitor_Subscribe service operation, if the service operation is supported by the NWDAF containing AnLF.

6.	The Analytics consumer may send Analytics feedback information in an Nnwdaf_AnalyticsSubscription_Subscribe message as described in clause 6.1.1.

7.	The NWDAF containing AnLF may send the Analytics feedback information received from the analytics consumer for the provisioned ML model by invoking Nnwdaf_MLModelMonitor_Notify service operation as requested in step 5. When the NWDAF containing MTLF receives either Analytics feedback information, the NWDAF containing MTLF may trigger step from 8 to 13 to enhance the ML model accuracy.

8a-8f.	The NWDAF containing MTLF, based on the request(s) from one or more NWDAF containing AnLF or its local policy, determines whether to perform ML model accuracy monitoring and re-training/re-provisioning of ML model by collecting new data from various data sources:

-	The NWDAF containing MTLF may collect new data for ML Model Accuracy monitoring, re-training and re-provisioning from the data source NFs and DCCF by invoking Nnf_EventExposure_Subscribe and Ndccf_DataManagement_Susbscribe service operation, respectively.

-	When ADRF ID and/or DataSetTag is given by step 2, the NWDAF containing MTLF may retrieve historical data from the ADRF indicated by the NWDAF containing AnLF at step 2. by invoking Nadrf_DataManagementRetrievalRequest or Nadrf_DataManagementRetrieval_Subscribe service operation. Otherwise, the NWDAF containing MTLF may retrieves the historical data from the DCCF or the NWDAF containing AnLF by invoking Ndccf_DataManagement_Subscribe or Nnwdaf_DataManagement_Subscribe service operation, respectively.

-	If the NWDAF containing AnLF does not include a DataSetTag with ADRF ID at step 2, the NWDAF containing MLTF may request ADRF to subscribe for the collection of the analytics and data that correspond to the analytics generated by the ML model provisioned in step 3, using the procedures defined in clause 6.2B.3.

-	The NWDAF containing MTLF may subscribe to UDM to get notification on change in the subscription data for Target of ML Model Reporting by invoking Nudm_SDM_Subscribe service operation and the UDM subscribes to the UDR to get notifications of the modification on UE subscription data by invoking Nudr_DM_Subscribe service operation.

-	The NWDAF containing MTLF may consider the data quality into the accuracy monitoring by collecting fault prediction analytics data from MDAS to determine the status of Data Source NFs, using MDA Request.

If the NWDAF containing MTLF has already collected new test data and performed ML model accuracy monitoring and retraining which is triggered by other NWDAF containing AnLF(s) (for ML model accuracy monitoring and retraining), the NWDAF containing MTLF, based on its internal logic, determines whether to use the data for the subscription or not.

9a-9f.	The NWDAF containing MTLF receives the requested data from various sources as requested in steps 8a-8f.

10.	Based on the collected analytics and data from steps 9a-9f, the NWDAF containing MTLF computes the accuracy using the methods described in clause 5C.1. The NWDAF containing MTLF may discard data from data sources if it detects the data quality of that source is not good. The NWDAF containing MTLF may generate prediction with the collected input data to calculate the accuracy if only input data and ground truth data are available.

NOTE:	How the NWDAF containing MTLF determines whether the data from the data source is of good quality or needs to be discarded is up to the NWDAF implementation and configuration.

11.	An accuracy report is sent to the NWDAF containing AnLF, e.g. when the reporting threshold is met by invoking Nnwdaf_MLModelProvision_Notify service operation.

12.	Based on the computed accuracy, the NWDAF containing MTLF may decide to re-train/re-provision the ML model.

13.	When the newly generated or re-trained ML model is ready, the NWDAF containing MTLF sends new or re-trained ML model to the NWDAF containing AnLF by invoking Nnwdaf_MLModelProvision_Notify service operation. The NWDAF containing MTLF may send the accuracy report of the new or re-trained ML model to the NWDAF containing AnLF.

# * * * * Second change * * * *

7.9	Nnwdaf_MLModelMonitor Service

7.9.1	General

Service Description: This service enables the consumer to subscribe/unsubscribe for ML model accuracy information monitored by an NWDAF (i.e. NWDAF containing AnLF). The service also enables the NWDAF containing AnLF registers the use and monitoring capability and delivering Analytics feedback information capability for an ML model into the model provider NWDAF, i.e. NWDAF containing MTLF.

# * * * * Third change * * * *

### 7.9.5	Nnwdaf_MLModelMonitor_Register

Service operation name: Nnwdaf_MLModelMonitor_Register

Description: The consumer registers the use and monitoring capability and delivering Analytics feedback information capability for an ML model at an NWDAF containing MTLF.

Inputs, Required: Consumer NF ID, Unique ML Model identifier.

Inputs, Optional: Monitoring capability, Delivering Analytics feedback information capability, Endpoint address of the Nnwdaf_MLModelMonitor_Subscribe service operation.

Outputs, Required: ML model monitoring registration ID.

Outputs, Optional: None.

# * * * * End of changes * * * *