3GPP SA WG2 Meeting #159	S2-2311153

# 9-13 October 2023, Xiamen, China	(was S2-230xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5059 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of QoS flows when PDU Set QoS cannot be admitted |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN behavior is not clear for the case that a RAN node that supports PDU Set QoS receives PDU Set QoS parameters but cannot admit those PDU Set QoS parameters (e.g., due to missing RAN resources). To ensure that operators can take an informed policy decision, which QoS to apply in case PDU Set QoS cannot be admitted, it is essential that SMF and PCF get informed in case NG-RAN cannot admit PDU Set QoS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in clause 5.7.7.1 that - if NG-RAN supports PDU Set QoS and receives PDU Set QoS parameters, but RAN cannot admit the PDU Set QoS parameters then NG-RAN shall reject the related QoS flow establishment or modification; - if NG-RAN supports PDU Set QoS and has admitted PDU Set QoS parameters but needs to release the AN resources, then NG-RAN shall release the QoS flow and trigger the AN-initiated PDU Session Modification procedure as defined in TS 23.502 [2] clause 4.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent RAN behavior for handling of PDU Set QoS. Operators can't take an informed policy decision, which QoS to apply in case PDU Set QoS cannot be admitted by NG-RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 5.7.7.1	General

PDU Set QoS Parameters are used to support PDU Set based QoS handling in the NG-RAN. At least one PDU Set QoS Parameter shall be sent to the NG-RAN to enable PDU Set based QoS handling.

The following PDU Set QoS Parameters are specified:

1.	PDU Set Delay Budget (PSDB).

2.	PDU Set Error Rate (PSER).

3.	PDU Set Integrated Handling Information (PSIHI).

The QoS Profile may include the PDU Set QoS Parameters described in this clause (see clause 5.7.1.2). The PCF determines the PDU Set QoS Parameters based on information provided by AF and/or local configuration. The PDU Set QoS parameters are sent to the SMF as part of PCC rule. The SMF sends them to NG-RAN as part of the QoS Profile.

If the NG-RAN receives PDU Set QoS Parameters and supports them, it enables the PDU Set based QoS handling and applies PDU Set QoS Parameters as described in this clause. If NG-RAN supports PDU Set QoS and receives PDU Set QoS parameters, but RAN cannot admit the PDU Set QoS parameters then NG-RAN shall reject the related QoS flow establishment or modification. If NG-RAN supports PDU Set QoS and has admitted PDU Set QoS parameters but needs to release the AN resources, then NG-RAN shall release the QoS flow and trigger the AN-initiated PDU Session Modification procedure as defined in TS 23.502 [2] clause 4.3.3.

Editor's note:	[XRM] The applicability and details of supporting PDU Set handling in uplink direction may be updated based on RAN WG's progress.

# >>>>END OF CHANGES<<<<