SA WG2 Meeting #159	S2-2311161

# Xiamen, China, Oct 09 – 13, 2023                     	  	   	  (revision of S2-2308477)

Source:	China Mobile

Title:		Support of MT SDT for REDCAP UE in RRC Inactive with long Edrx

Document for:	Approval

Agenda Item:	9.38

Work Item:	REDCAP_SDT

Abstract:		Support of MT SDT for REDCAP UE in RRC Inactive with long eDRX.

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Support of MT SDT for REDCAP UE in RRC Inactive with long eDRX

Acronym: REDCAP_SDT

Unique identifier: to be allocated by MCC

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No | X | X |  |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

{Not applicable for Feature nor for a Study Item}

{For a Building Block: list here the parent Feature }

{For a Work Task: list here the parent Building Block }

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 941100 | Core part: Mobile Terminated-Small Data Transmission (MT-SDT) for NR | RAN aspects of the MT-SDT feature in Rel-18. |

3	Justification

Small Data Transmission (SDT) is a procedure allowing data transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state). SDT is enabled on a radio bearer basis and is initiated by the UE or the NG-RAN only if less than a configured amount of data awaits transmission across all radio bearers for which SDT is enabled.

In case of MT data, the NG-RAN determines whether the MT data should be delivered to UE via SDT DRB. When the MT SDT DRB should be used, the NG-RAN initiates the MT SDT paging with a MT-SDT indication inside. After receiving the paging message with the MT-SDT indication, the UE initiates the RRC Resume procedure to resume the SDT DRB/SRB configured by NG-RAN during the RRC release procedure. Based on the latest agreements in RAN, NG-RAN needs to take the DL data size for each QoS flow into account when determining whether to initiate the MT-SDT paging, as stated in R3-233347.

Currently, when UE in RRC Inactive is configured with long eDRX cycle (i.e. longer than 10.24s), the NG-RAN may indicate AMF and AMF may initiate the High Latency communication function to handle the MT data, i.e. to indicate SMF/UPF to buffer the DL data. When the UE is about to be reachable, the AMF sends N2 message to trigger the RAN paging and the NG-RAN has no idea regarding the DL data since they are handled by the Core Network. Hence, the support of MT-SDT for REDCAP UEs in RRC Inactive state with long eDRX is missing. Especially for the case of DL data arrival, the UPF notifies SMF and SMF invokes the Namf_MT_EnableUEReachability service to check the UE reachability with AMF. If the UE is not reachable due to the eDRX, the AMF rejects the SMF request with estimated maximum waiting time for subsequent DL data buffering. Unless there is subsequent DL data arrival for the QoS Flow with higher priority, the SMF is not allowed to re-notify the AMF.

Therefore, the AMF needs to trigger the SMF/UPF to report the DL data size for each QoS Flow before triggering the RAN paging in order to get the up-to-data DL data size for each QoS Flow.

4	Objective

The objective of this WID is to support of MT SDT for REDCAP UE in RRC Inactive with long eDRX in case of DL data arrival.

This work item will require 0.5 TU to discuss and agree the corresponding CR(s).

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2: The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Support of MT SDT for REDCAP UE in RRC Inactive with long eDRX | TSG#102 Dec. 2023 |  |
| 23.502 | Support of MT SDT for REDCAP UE in RRC Inactive with long eDRX | TSG#102 Dec. 2023 |  |

6	Work item Rapporteur(s)

Aihua Li, China Mobile, liaihua@chinamobile.com

7	Work item leadership

SA2

8	Aspects that involve other WGs

RAN aspects will be coordinated with relevant RAN WGs. .

9	Supporting Individual Members

Supporting IM name

China Mobile