SA WG2 Meeting #S2-159	S2-2311168

October 09 - 13, Xiamen, China	(revision of S2-231xxxx)

Source:	Intel

Title:	Architectural Assumptions for WT#1.2 and WT#2.2

Document for:	Approval

Agenda Item: 	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract of the contribution: This paper proposes architectural assumptions for WT#1.2 and WT#2.2.

# 1	Discussion

The present contribution focuses on the following sub-work tasks:

WT#1.2 Support QoS control and PDU Set identification for XR stream with e2e encryption (e.g. fully encrypted header, partially encrypted header). This is applicable for PDUs received at N6 for DL.

WT#2.2 Study whether and how to support dynamic change (via user plane) in traffic characteristics (e.g. burst related parameters), provided by the application in the DN.

These sub-work tasks have in common that they rely on user plane (metadata) information that is provided by the Application Server over the N6 reference point via some encapsulation protocol. The information in the encapsulation header that is used for PDU Set identification or for dynamic change in traffic characteristics must not be encrypted.

It is proposed to capture these assumptions in TR 23.700-70 clause 4.1.

# 2	Proposal

It is proposed to agree the text below for inclusion into TR 23.700-70.

* * * * First Change * * * *

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

The architecture in this study should be based on the following assumptions:

-	To support PDU Set identification for XR stream with e2e encryption it is assumed that the N6 encapsulation protocol (between UPF PSA and the Application Server) includes information for PDU Set identification that is not encrypted.

-	To support dynamic change (via user plane) in traffic characteristics it is assumed that the N6 encapsulation protocol (between UPF PSA and the Application Server) includes information for dynamic change in traffic characteristics that is not encrypted.

* * * * End of Changes * * * *