3GPP TSG-WG SA2 Meeting #159	S2-2311185

# Xiamen, China, Oct 9 – 13, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0460 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Feature Synchronization of User Plane |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In CT#101, Protocol selection for user plane positioning (5G_eLCS_Ph3 Rel-18 WI) A compromise was found at CT1#143 with the agreement of C1-235982, C1-236546 and C1-236547 (5G_eLCS_Ph3 Rel-18 Work Item). ‘SUPL as required in several markets, can be deployed as supported in 3GPP networks.’ ‘It is therefore proposed to introduce a clause in TS 24.572 to specify the co-existence of the 3GPP specified UP positioning solution, LCS-UPP, with another UP positioning solution (SUPL) deployed.’ ‘UEs supporting one or more user plane location solution(s) shall indicate its supported user plane location solution(s) to the network using the LCS-UPP bit, the SUPL bit or both’ The consensus of co-existence of LCS UPP and SUPL in CT#101 output should be synchronized in SA2 specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Synchronize the SA2 specification with the UP progress in CT1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The support of LCS User Plane is incomplete and inconsistent with CT1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 4.3.8, 4.3.7, 4.3.12, 5.1, 6.X, 8.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.071: "Technical Specification Group Systems Aspects; Location Services (LCS)".

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[5]	3GPP TS 43.059: "Functional Stage 2 description of Location Services in GERAN".

[6]	Void.

[7]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[8]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[9]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[10]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[11]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[12]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[13]	OMA MLP TS: "Mobile Location Protocol", [http://www.openmobilealliance.org].

[14]	Void.

[15]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[16]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[17]	3GPP TS 25.305: "Stage 2 functional specification of User Equipment (UE) positioning in UTRAN".

[18]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[19]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[20]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[21]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[22]	Void.

[23]	ITU-T Recommendation E.164: "The international public telecommunication numbering plan".

[24]	3GPP TS 23.222: "Common Application Programming Interface (API) framework for 3GPP northbound APIs".

[25]	RFC 2396: "Uniform Resource Identifiers".

[26]	RFC 3261: "SIP: Session Initiation Protocol".

[27]	3GPP TS 23.228: "IP multimedia subsystem (IMS)".

[28]	3GPP TS 23.003: "Numbering, addressing and identification".

[29]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[30]	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

[31]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[32]	Void.

[33]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[34]	Void.

[35]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[36]	3GPP TS 24.571: "5G System (5GS); Control plane Location Services (LCS) procedures; Stage 3".

[37]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[38]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[39]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[40]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

[41]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[42]	3GPP TS 23.632: "User data interworking, coexistence and migration; Stage 2".

[43]	3GPP TS 29.563: "Home Subscriber Server (HSS) services for interworking with Unified Data Management (UDM); Stage 3".

[44]	OMA-AD-SUPL-V2_0: "Secure User Plane Location Architecture Approved Version 2.0".

[45]	3GPP	TS 24.572: " User Plane Location Services (LCS) Protocols And Procedures; Stage 3"

[46]	OMA-TS-ULP-V2_0_6: "UserPlane Location Protocol Approved Version 2.0.6".

# * * * * Second change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

APN	Access Point Name

APN-NI	APN Network Identifier

EDT	Early Data Transmission

E-SMLC	Evolved Serving Mobile Location Centre

GMLC	Gateway Mobile Location Centre

HGMLC	Home GMLC

LCS	LoCation Services

LDR	Location Deferred Request

LIR	Location Immediate Request

LMF	Location Management Function

LOS	Line-Of-Sight

LPI	LCS Privacy Indicator

LPHAP	Low Power and High Accuracy Positioning

LRF	Location Retrieval Function

MBSR	Mobile Base Station Relay

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

NLOS	Non-Line-Of-Sight

NI-LR	Network Induced Location Request

PMD	Pseudonym mediation device functionality

POI	Privacy Override Indicator

PRU	Positioning Reference Unit

SDT	Small Data Transmission

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TWAP	Trusted WLAN Access Point

VGMLC	Visited GMLC

SL-MO-LR	Sidelink Mobile Originating Location Request

SL-MT-LR	Sidelink Mobile Terminating Location Request

SLP	SUPL Location Platform

SUPL	Secure User Plane Location

# * * * * Third change * * * *

### 4.3.8	Location Management Function, LMF

The LMF manages the overall co-ordination and scheduling of resources required for the location of a UE that is registered with or accessing 5GCN. It also calculates or verifies a final location and any velocity estimate and may estimate the achieved accuracy. The LMF receives location requests for a target UE from the serving AMF using the Nlmf interface. The LMF interacts with the UE in order to exchange location information applicable to UE assisted and UE based position methods and interacts with the NG-RAN, N3IWF or TNAN in order to obtain location information.

The LMF shall determine the result of the positioning in geographical co-ordinates as defined in TS 23.032 [8] and/or in local co-ordinates as defined in TS 23.032 [8]. If requested and if available, the positioning result may also include the velocity of the UE. The coordinate type(s) is determined by LMF when receiving a location request, based on LCS Client type and supported GAD shapes. If the location request indicates regulatory LCS Client type the LMF shall determine a geographical location and optionally a location in local coordinates. For location request indicates a value added LCS Client type, the LMF may determine the UE location in local coordinates or geographical co-ordinates or both. If the supported GAD shapes is not received or Local Co-ordinates is not included in the supported GAD shapes, the LMF shall determine a geographical location.

NOTE 1:	Some RAT independent position methods (e.g. GNSS based position methods) can only determine a UE location in geographical co-ordinates. In such a case, the LMF may translate a UE location in geographical co-ordinates into a location in local co-ordinates when an origin for the local co-ordinates has known global coordinates. When an origin for the local co-ordinates does not have known global coordinates, position methods that can only determine a UE location in geographical co-ordinates cannot be used to determine a UE location in local co-ordinates.

Additional functions which may be performed by an LMF to support location services include the following.

-	Support a request for a single location received from a serving AMF for a target UE.

-	Support a request for periodic or triggered location received from a serving AMF for a target UE.

-	Determine type and number of position methods and procedures based on UE and PLMN capabilities, QoS, UE connectivity state per access type, LCS Client type, co-ordinate type and optionally service type and indication of requiring reliable UE location information.

-	Report UE location estimates directly to a GMLC for periodic or triggered location of a target UE.

-	Support cancelation of periodic or triggered location for a target UE.

-	Support the provision of broadcast assistance data to UEs via NG-RAN in ciphered or unciphered form and forward any ciphering keys to subscribed UEs via the AMF.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	Support of receiving stored UE Positioning Capability from AMF and support of providing updated UE Positioning Capability to AMF.

-	Map the UE location to a geographical area where the PLMN is or is not allowed to operate based on the request from AMF.

-	Support determination of a UE location at a scheduled location time.

-	Support determination of indoor or outdoor for a location estimate.

-	Determine whether to use user plane (i.e., LCS-UPP[45], SUPL[44] or both) or control plane for positioning.

-	Support handling of 5GC-MT-LR, 5GC-NI-LR, 5GC-MO-LR and deferred 5GC-MT-LR for periodic or triggered location over a user plane connection between UE and LMF over TLS.

Editor's note:	LMF functionality of user plan connection handling will be updated according to the protocol stack decision made by CT WG1.

-	Support collection of GNSS assistance data from AFs.

-	Support service level PRU Association, PRU Association update or PRU Disassociation.

-	LMF supports verification of a PRU initiated Association or Disassociation by checking whether there is an PRU verified indication from AMF.

-	LMF stores the received PRU information contained in service level PRU Association message and removes the PRU information after PRU Disassociation.

-	LMF keeps PRU information for PRUs which are in OFF state.

-	LMF may indicate support of PRU function to NRF via NF profile and may further send the PRU indication via NF profile update if PRU is stationary PRU.

-	LMF may request a PRU to associate to a new LMF by returning a Routing ID of the new LMF.

PRU ON/OFF states indicate temporarily availability of the PRU functionality of a UE at the serving LMF (e.g. PRU OFF due to other high priority tasks/energy saving at the UE, or the UE temporarily loses network coverage).

-	Support selection of a PRU based on stored PRU information if the LMF needs to obtain the location measurements from the PRU to assist positioning of a target UE.

-	Support to obtain PRU location measurements as described in clause 5.4.5 of TS 38.305 [9] by triggering the procedure in clause 6.11.

-	Support to obtain PRU location measurements from other PRU serving LMF(s).

-	As a serving LMF of target UE(s), support discovery and selection of other PRU serving LMF(s) by querying the NRF and support to request PRU location measurements from the selected LMF(s).

-	As a serving LMF of PRU(s), support to provide PRU location measurements to other LMF(s) after receiving a request from other LMF(s).

-	Support to determine UE location by considering obtained PRU location measurements.

NOTE 2:	Country, area within a country, or an international area can be supported as different types of geographical area.

-	Support a request for user plane reporting from a UE to an LCS Client or AF for a periodic or triggered 5GC-MT-LR. Subsequently, support the transfer of cumulative event reports from the target UE via control plane back to the H-GMLC and LCS Client or AF. Also support any request for assistance data received in a cumulative event report.

-	Determine UE location for a UE connecting to a MBSR based on location and velocity of the MBSR and the timing of the location estimations for the target UE and MBSR.

-	For a regulatory location service, support reporting of multiple INTERMEDIATE location estimates to GMLC.

# * * * * Fourth change * * * *

### 4.3.7	Access and Mobility Management Function, AMF

The AMF contains functionality responsible for managing positioning for a target UE for all types of location request. The AMF is accessible to the GMLC and NEF via the Namf interface, to the RAN via the N2 reference point and to the UE via the N1 reference point.

Functions which may be performed by an AMF to support location services include the following.

-	Initiate an NI-LR location request for a UE with an IMS emergency call or to know a UE geographical area with NR satellite access for PLMN selection verification.

-	Receive and manage location requests from a GMLC for a 5GC-MT-LR and deferred 5GC-MT-LR for periodic, triggered and UE available location events.

-	Receive and manage location requests from a UE for a 5GC-MO-LR.

-	Receive and manage Event Exposure request for location information from an NEF.

-	Select an LMF.

-	Receive updated privacy requirements from a UE and transfer to a UDR via UDM.

-	Support cancelation of periodic or triggered location reporting for a target UE.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	When assistance data is broadcast by 5GS in ciphered form, the AMF receives ciphering keys from the LMF and forwards to suitably subscribed UEs using mobility management procedures.

-	Store UE Positioning Capability received from an LMF and send the UE Positioning Capability along with the received location request to an LMF.

-	Receive and store UE user plane positioning capability and UE support of LCS-UPP[45], SUPL[44] or both as part of the "5GMM capability" from UE.

-	Receive UL NAS Transport including a PRU Association, Association Update, or Disassociation Request (contained in an LCS supplementary service message) from a UE.

-	AMF may verify whether a UE can serve as a PRU based on UE subscription data after receiving the PRU Association Request, Association Update, or Disassociation. AMF may also use local policy to determine if UEs are allowed to serve as a PRU. AMF may verify based on subscription information or local policy if PRU can work as stationary PRU.

-	Sends the PRU Association Request or PRU Disassociation Request to LMF and may include a UE verification indication indicating whether this UE is authorized to serve as a PRU.

-	Support verifying whether UE is subscribed with user plane positioning between UE and LMF and triggering LMF to establish a User Plane Connection to UE if UE requested that.

-	Support subscribing from LMF status of LCS user plane connection between a UE and the LMF.

-	Store in UE context that UE has a maintained User Plane connection with certain LMFs.

NOTE:	Details of UE Positioning Capability is defined in TS 37.355 [20].

-	Support of local configuration of a mapping table of UE identifier ranges and LMF identifier(s) or querying the UDM the LMF identifier for a UE for a 5GC-MO-LR.

-	Support the 5G to EPS Handover by providing the target MME ID to GMLC as part of the LCS Service response.

-	Support interaction with NWDAF to obtain UE related analytics to assist with UE location verification for NR satellite access.

# * * * * Fifth change * * * *

### 4.3.12	Network Repository Function, NRF

In addition to the functions defined in TS 23.501 [18],	the NRF may perform the following functions:

-	Support to store or update PRU existence indication in TAI level in LMF profile based on request from PRU serving LMF.

-	Support LMF(s) with PRU function discovery by AMF.

-	LMF discovery by considering user plane Positioning Capability and UE support of LCS-UPP[45], SUPL[44] or both.

-	Support LMF(s) associated with PRUs discovery by other LMF(s), according to target area in the discovery request and the LMF profiles with PRU existence indication in TAI level.

# * * * * Sixth change * * * *

## 5.1	LMF Discovery and Selection

LMF selection functionality is supported by the AMF to determine an LMF for location estimation of the target UE or Ranging/Sidelink Positioning between Target UE and SL Reference UE. The LMF selection functionality is also supported by the LMF if it determines that it is unsuitable or unable to support location for the current UE access network or serving cell for the deferred 5GC-MT-LR procedure for periodic, triggered location events, or Modification of User Plane Connection. The LMF selection functionality may also be supported by the GMLC and the GMLC provides the selected LMF ID to AMF.

LMF reselection is a functionality supported by AMF when necessary, e.g. due to UE mobility.

The LMF selection/reselection may be performed at the AMF or LMF or GMLC based on the locally available information i.e. LMF profiles are configured locally at AMF or LMF or GMLC, or by querying NRF.

The following factors may be considered during the LMF selection:

-	LCS client type.

-	Requested Quality of Service information, e.g.:

-	LCS accuracy,

-	Response time (latency),

-	Access Type (3GPP /N3GPP).

NOTE 1:	Location methods may differ depending on the Access Type, e.g. in the case of WLAN Access Location determination may just correspond to retrieval of IP addressing information from the N3IWF/TNGF; As another example, for Wireline access, Location determination may just correspond to retrieval of geo coordinates corresponding to a GLI as defined in clause 4.7.8 of TS 23.316 [21] or a HFC Node ID.

-	RAT type (e.g. 5G NR, eLTE, or any of the RAT Types specified for NR satellite access) and/or the serving AN node (i.e. gNB or NG-eNB) of the target UE.

-	RAN configuration information.

-	LMF capabilities, including the support of Uu based positioning as defined in clause 4.3.8 and/or Ranging/Sidelink positioning as defined in clause 4.3.8 of TS 23.586 [40].

-	LMF load.

-	LMF location.

-	Indication of either a single event report or multiple event reports.

-	Duration of event reporting.

-	Network slicing information, e.g. S-NSSAI and/or NSI ID.

-	LMF Service Area consisting of one or more TA(s).

-	Supported GAD shapes.

-	Support LCS when MBSR is involved.

-	Requested UE has maintained user plane connection over LCS-UPP[45], SUPL[44] or both with certain LMFs/SLPs.

When receiving a NAS message from UE, including an LMF ID together with a LPP message (refer to step 25 in clause 6.3.1 for event reporting for a deferred 5GC-MT-LR), AMF sends the LPP message to the LMF, as indicated by the LMF ID.

NOTE 2:	Description on how UE encapsulates the LMF ID in the NAS message is documented in TS 24.571 [36].

UDM may store the LMF ID in UE subscription data. During the location procedure, GMLC received the LMF ID from the UDM and provides it to AMF.

GMLC may be configured with the following parameters:

-	LMF ID and/or

-	per group ID and its correlating LMF ID.

The AMF may use locally provisioned configuration to determine LMF based on UE identify or its group information.

NOTE 3:	It is AMF implementation specific for the priority of different selection criteria from GMLC, AMF and LMF.

When the GMLC receives a MT location request from LCS client/AF, GMLC determines the LMF ID based on the configured parameters for an LCS Client/AF. In case a group ID is provided or derived from the location request, GMLC determines the correlating LMF ID based on the provisioned group ID.

GMLC may have configuration with one or several dedicated LMF ID(s), irrelevant to any LCS client/AF. When the GMLC receives a MT location request from LCS client/AF, GMLC only determines the LMF ID within the configuration for all LCS client/AF.

NOTE 4:	When AMF can't access the LMF instance of the LMF ID, by default, AMF replies to corresponding error to GMLC. GMLC could retry or fail the request accordingly; with explicit configuration to serve as backup selection in this, AMF can also select different configured LMF instance.

# * * * * Seventh change [all new text] * * * *

## 6.X	Co-Exist of SUPL Support

SUPL is supported based on clause 5.1, clause 6.2.1, clause 7.1.1, clause A.3, clause A.4  of TS 38.305[7]. SUPL procedures may be handled by SLP based on UE capabilities independently, or in parallel with LPP[20] and/or NRPPa[15] procedures handled by LMF.

Details of the signalling interaction between an LMF and SLP are outside the scope of this specification.

SLP may perform SUPL Initiation Function by following means:

- SLP may invoke the SMSF to trigger MT SMS push [46].

- SLP may send SIP push[46] directly or invoke IMS to support the SIP push[46]

- If LMF/SLP knows the UE IP address information, SLP can also perform SUPL Initiation Function over UDP/IP[46].

# * * * * Eighth change * * * *

#### 8.3.2.2	Nlmf_Location_DetermineLocation service operation

Service operation name: Nlmf_Location_DetermineLocation

Description: Provides UE location information to the consumer NF.

NOTE:	For deferred location request, this service operation is used to implicitly subscribe to the notification of the UE location information.

Input, Required: Client Type, LCS Correlation Identifier.

Input, Optional: Serving cell identifierof the Primary Cell in the Master RAN node and the Primary Cell in the Secondary RAN node when available based on Dual Connectivity scenarios if the UE is using 3GPP access, required Location QoS instance(s), Supported GAD shapes, service type, indication of requiring reliable UE location information, AMF identity if a UE associated Namf_Communication service is to be invoked by LMF, Type of request for a 5GC-MO-LR, Embedded LPP message(s) for a 5GC-MO-LR (1-3 messages), subscribed assistance data for a 5GC-MO-LR, Deferred location type, Deferred location parameters, indication if UE supports LPP or not, UE connectivity state per access type, Notification Target Address, Notification Correlation ID, indication of UE geographical area determination for PLMN selection verification, UE Positioning Capability, UE User Plane Positioning Capability, UE support of LCS-UPP[45], SUPL[44] or both, TNAPId (see TS 29.571 [33]), TWAPId (see TS 29.571 [33]), Scheduled Location Time, request for user plane reporting to an LCS Client or AF, user plane address of an LCS Client or AF, user plane security information for an LCS Client or AF, cumulative event report timer, maximum number of user plane event reports to an LCS Client or AF, event report allowed area, reporting indication, UE unaware indication, LPHAP indication, the serving cell identity belongs to a MBSR indication, IAB-UE ID of the MBSR.

Output, Required: Success/Failure indication

Output, Optional: Geodetic Location, Local Location including Coordinate ID, Civic Location, Indoor/Outdoor indication, LOS/NLOS measurement indication, Position Methods Used (in the case of success indication provided), Serving LMF identification, Failure Cause (in the case of failure indication provided), achieved Location QoS Accuracy, UE geographical area where a PLMN is or is not allowed to operate, UE Positioning Capability, the timestamp of the Location, indication that the location determination will be sent directly to GMLC.

See clause 6.1, clause 6.2. clause 6.3.1 and clause 6.9.1 for examples of usage of this service operation.

# * * * * End of changes * * * *