3GPP TSG-WG SA2 Meeting #159	S2-2311186

# Xiamen, China, Oct 9 – 13, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.032 | CR | 0026 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on relative velocity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-09-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The relative velocity, which is now expressed in a radial component plus a transverse component, it needs further clarification. Velocity is a vector, velocity of anchor UE plus relative velocity should be equal to velocity of target UE: The relative velocity direction could be decomposed into radial and transverse directions, so the relationship should be : Velocity of anchor UE + relative velocity over radial direction + relative velocity over transverse direction = velocity of target UE. Relative Velocity could be known without the knowledge of absolute velocity of anchor or target UE. This means that no matter how pairs of anchor UE and/or target UE moves with different and/or same speeds and directions, there could always be possible that these pairs of UEs generate same and/or different speeds and directions. For relative velocity, its direction and speed should be based on the anchor UE, the current orientation based on north in clause 8.4a of TS 23.032 should be removed since relative velocity still exists in north pole. For the uncertainty of relative velocity, it should be independent to the uncertainty of uncertainties of anchor and target UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the relationship of relative velocity and related pair of UEs’ velocity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The support of relative velocity is wrong and incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 23.586 CRxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 8.4a	Relative Velocity with Uncertainty

The relative velocity with uncertainty of a device B relative to a device A is characterised by a radial velocity component and a perpendicular transverse velocity component. The sum of velocity of device A and the 2 velocity component vectors (i.e., the radial velocity component and the perpendicular transverse velocity component) is equal to the velocity of device B in both speed and direction. The radial velocity component is characterized by a rate of change of a range between the device A and device B with direction from device B to device A. The transverse velocity component is characterized by a rate of change of a direction to the device B from the device A, where the direction is perpendicular to radial velocity counter clockwise from device A to device B. The rates of change of the range and the angles can be each independently included or excluded in the relative velocity and each has an uncertainty and a confidence. The uncertainties and confidence of relative velocity can be independent with uncertainty and confidence of the velocities of device A and device B.

Figure 12a: Description of a Relative Velocity with Uncertainty

# * * * * End of changes * * * *