SA WG2 Meeting #159	S2-2311593

October 9-13, 2023, Xiamen, P.R. China,

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5002 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UPF Data Collection with QoS Monitoring event |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2023-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UPF may need to indicate to the receiver of the direct event notification of QoS Monitoring that the QoS measurements have been taken on the QoS flow for the default QoS rule. UPF needs to receive this information from the SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A companion CR to TS 23.502 clause 4.15.4.5 introduces a correction so that when at subscription to QoS monitoring event with application identifier, SMF tries to identify the active PCC Rule that includes that Application identifier, SMF does not reject the subscription request when the identified PCC Rule has no QoS Monitoring policy or when no PCC rule is identified. Instead, SMF accepts the subscription. SMF will try to identify a PCC Rule with QoS Monitoring policy enabled when the PDU Session PCC rules change. The consumer can indicate in the request that it can receive measurements for the QoS Flow for the default QoS Rule as fallback. In this CR, SMF configuration information to UPF for QoS Monitoring is updated to include, when applicable, indication that montoring is on the QoS flow for the default QoS rule. This way, UPF can add this indication in the notification report. The QoS Monitoring of the QoS flow for the default QoS rule is used as part of UPF Data Collection (TS 23.502 clause 4.15.4.5), and this indication is only needed in UPF direct event notifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Consumers may fail to interpret the UPF collected data. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 4505 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.8.2.18	QoS Flow related QoS monitoring and reporting

The SMF may configure the UPF to perform QoS monitoring for a QoS Flow and to report the monitoring results with the help of the following parameters provided in the Session Reporting Rule (SRR) described in clause 5.8.5.11:

-	QoS parameter(s) to be measured indicating the subject of the QoS monitoring as defined in clause 5.45;

-	Reporting period indicating the time interval in which a new measurement result and a potential report has to be available. If no measurement result is available to the UPF within the Reporting period, the UPF shall report a measurement failure;

-	Reporting frequency indicating the type of the reporting as "periodic" or "event triggered":

-	If the Reporting frequency indicates "periodic", the UPF shall send a report each time the reporting period is over.

-	If the Reporting frequency indicates "event triggered", a Reporting threshold for each parameter in the QoS parameter(s) to be measured and a Minimum waiting time are provided as well. The UPF shall send a report when the measurement result matches or exceeds the indicated Reporting threshold. Subsequent reports shall not be sent by the UPF during the Minimum waiting time. If measurement results are received during the Minimum waiting time, the UPF shall report the minimum and the maximum measurement result when the Minimum waiting time is over.

-	(Optional) Target of the reporting and Indication of direct event notification indicating that the UPF shall send the reports to a different NF than the SMF (i.e. to the Local NEF or the AF). The NF is described by a Notification Target Address and a Notification Correlation ID. The SMF can also indicate that the UPF shall send the reports to both, the NF indicated by the Target of reporting and to the SMF. If so, the UPF shall send the reports to the SMF as well. If the Indication of direct event notification is not provided, the UPF shall send the reports to the SMF.

-	(Optional) Reporting suggestion information as defined in clause 5.8.2.17 applicable to Target of the reporting to reduce the UPF performance impacts.

-	(Optional) Indication of QoS Flow associated with the default QoS Rule indicating that the QoS monitoring report is for the QoS Flow associated with the default QoS Rule (see clause 4.15.4.5.1 of TS 23.502 [3] for details about the usage of this indication by SMF). The UPF shall forward this indication in the Nupf_EventExposure_Notify service operation when sending reports.

The UPF sends reports to the SMF in the form of QoS Monitoring Report as described in clause 5.8.5.12 and reports to the Local NEF or the AF via the Nupf_EventExposure_Notify service operation described in clause 7.2.29.

# * * * * End of changes * * * *