3GPP TSG-WG SA2 Meeting #159	S2-2311604

Xiamen, October 9th – 13th, 2023	(revision of S2-2310941, 11383, 15554)

Source:	Huawei, HiSilicon, CATT, Deutsche Telecom, Vodafone, Novamint, Sateliot

Title:	FS_5GSAT_ARCH_Ph3 Architecture Assumptions

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract: Architecture assumptions for FS_5GSAT_ARCH_Ph3.

# 1. Introduction/Discussion

In Rel-19 SAT_Ph3 SID (SP-231199), new architecture objective for satellite access has been proposed which is different from transparent payload discussed in Rel-17 and Rel-18. In SID, it is proposed that eNB/gNB will be embedded on board, which should be considered as an architecture assumption.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-29.

# * * * * First change (all new) * * * *

# 4.	Architectural Assumptions and Principles

The following architecture assumptions are applied to the study:

-	The 5GC architecture for satellite access for NR as defined in TS 23.501 [2] is used as a baseline.

-	The EPC architecture for satellite access for IoT as defined in TS 23.401 [5] and the architecture enhancements to facilitate communications with packet data networks and applications as defined in TS 23.682 [x] are used as a baseline.

-	At least eNB/gNB is assumed to be on board the satellite.

-	Impacts to UE, network functions and entities are minimised. To the extent possible, existing procedures and functionality is reused.

-	Inter-Satellite Links (ISL) and Feeder link are assumed to act only as transport layer links and are not specified in 3GPP.

-	Store and Forward Satellite Operation assumes that UE-satellite-ground network connectivity can be intermittent as defined in clause 3.1.

-	Store and Forward Satellite Operation shall work without ISL.

-	NR UE-Satellite-UE communication assumes communication between two or more UEs under the coverage of the same satellite or different satellites.

# * * * * End of changes * * * *