SA WG2 Meeting #S2-159	S2-2311719

09 - 13 October, 2023, Xiamen, P.R. China

Title:	Response to LS on NSACF Role and NSAC Service Area

Response to:	S2-23100078/ C4-232359

Release:	Rel-18

Work Item:	eNS_Ph3

Source:	3GPP SA2

To:	3GPP CT4

Cc:

Contact person:	Jinguo Zhu

zhu dot jinguo at zte dot com dot cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

SA2 thanks CT4 for LS on NSACF Role and NSAC Service Area. SA2 would like to answer the questions from CT4 as follows:

Q1. For hierarchical NSACF architecture, an NSACF may function as a primary NSACF or a subordinate NSACF. However, it is not clear to CT4 that whether the configuration of NSACF role (i.e. primary NSACF or subordinate NSACF) is per S-NSSAI basis, or for the whole PLMN?

SA2 answer: The configuration of primary NSACF or subordinate NSACF is per S-NSSAI basis.

Q2. If the network is configured with multiple NSAC service areas, is it possible or valid deployment that, for a given S-NSSAI, an NSACF instance can act as the primary NSACF and at the same time for certain NSAC service area(s) also act as a distributed NSACF (with other NASC service areas served by other distributed NSACFs).

SA2 answer: Yes.

Q3. It is not clear to CT4, whether a NSAC service area may be identified by TAI(s) or should be instead identified by a new NSAC service area identifier that may take the form of an operator defined string?

SA2 answer: SA2 plans to further discuss the NSAC service area topic at next SA2 meeting and will update CT4 later.

2	Actions

To CT4

ACTION: 	SA2 kindly asks CT4 to take the information above into account.

3	Dates of next TSG SA WG 2 meetings

SA2#160	13th–17th November 2023		Chicago, US

SA2-Ad Hoc	22nd –29th January, 2024		Emeeting