3GPP TSG-WG2 Meeting #160AH	S2-2401102

Emeeting, 22- 26 Jan, 2024

Title:	[Draft] Reply LS on packet delay measurement

Response to:	LS on packet delay measurement (C4-234616)

Release:	Rel-18

Work Item:	XRM

Source:	ZTE

To:	CT4

Cc:	CT3

Contact person:	Jinguo Zhu

zhu.jinguo@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.501 CR5289 and 23.503 CR1261

- Overall description

SA2 thanks CT4 for its LS on Issues on packet delay measurement. SA2 has discussed CT4 questions and provides answers below as follows:

1.	Whether the UPF is required (from Rel-18 onwards) to report minimal and maximum measurement result for packet delay for event-based reporting when the minimal waiting timer is over?

SA2 Reply: Not, it is not. When the minimal waiting time is over the UPF sends minimal measurement result only when further measurement result is received that matches or exceed the reporting threshold.

2. 	If so, should the UPF send a measurement report containing minimal delay and maximal delay if the latest packet delay measured (just before the expiry of the minimal waiting timer) is smaller than the threshold?

SA2 Reply: see reply to question 1.

3.	Whether the above requirement for the UPF is applicable also for periodic reporting? If not, what are reasons?

SA2 Reply:  No. The Reporting threshold and Minimum waiting time  are not defined for periodic reporting.

SA2 has agreed the attached CRs.

- Actions:

To CT4:

ACTION:

SA2 asks CT4 to take the information above into consideration.

- Dates of next TSG SA WG 2 meetings

SA2#161                 2024/02/26 – 2024/03/01	Athens

SA2#162                 2024/04/15 – 2024/04/19	China