SA WG2 Meeting #160AH	S2-2400745

# E-meeting, January 22 – 29, 2024        	  	  			       		          (revision of S2-240)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2125 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AUN3 device de-registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Approved C1-239242 reads. If the AMF receives an indication from the W-AGF that there is no 5G-RG connected to the same wireline for an AUN3 device as specified in 3GPP TS 23.316 [6D], the AMF shall locally de-register the AUN3 device. […] Upon receiving the DEREGISTRATION REQUEST message, the 5G-RG shall delete the contexts of the AUN3 devices behind the 5G-RG, if any. SA2 spec should align |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that: For each AUN3 device, upon receiving the AN Release request with the cause specified in step 5, the AMF shall initiate the deregistration of the AUN3 device as specified in clause 4.2.2.3.3 of TS 23.502 [3]. The cause in the deregistration message should indicate the disconnection of 5G-RG. This deregistration involves only the 5GC and the release of the signaling connection with the W-AGF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | incoherence between CT1 and SA2 specs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 7.2.8.3	5G-RG Deregistration via W-5GAN when it is also serving AUN3 devices

Figure 7.2.8.3-1: 5G-RG Deregistration procedure via W-5GAN when serving AUN3 devices

1-3.	The same as steps 1-3 for Figure 7.2.1-1 with the following modification:

In the case of 5G-RG initiated deregistration, 5G-RG shall first deregister each of the registered AUN3 devices connected to it (if any) before initiating the deregistration of itself.

4.	W-AGF controls if there exist any AUN3 devices connected to the 5G-RG that are registered to the 5GC through the W-AGF. If there are no AUN3 devices, then steps 5 and 6 shall be ignored.

NOTE 1:	How the W-AGF temporarily maintains the mapping of AUN3 devices and 5G-RG to perform steps 5 and 6 when the 5G-RG's context is released is based on implementation.

5.	[conditional] For each AUN3 device identified in step 4, the W-AGF shall send AN Release request as specified in clause 4.2.6 of TS 23.502 [3]. Here, the cause should indicate the disconnection of 5G-RG.

6.	[conditional] For each AUN3 device, upon receiving the AN Release request with the cause specified in step 5, the AMF shall initiate the deregistration of the AUN3 device as specified in clause 4.2.2.3.3 of TS 23.502 [3]. The cause in the deregistration message should indicate the disconnection of 5G-RG. This deregistration involves only the 5GC and the release of the signaling connection with the W-AGF.

7.	The W-AGF may initiate the release of the signalling connection between 5G-RG and W-AGF.

NOTE 2:	Whether this step is needed, and if so, the details of this step are defined by BBF and/or CableLabs.

8.	The 5G-RG disconnects the AUN3 device. How this is done is outside of 3GPP scope.

NEXT CHANGE (2)

NEXT CHANGE (3)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES