SA WG2 Meeting #S2-160AHE	S2-2400840

22 - 29 January, 2024, Electronic	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0351 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the mobility procedures for UEs in RRC_INACTIVE state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the existing spec text, the SMF may provide the MBS Assistance Information to target RAN node in Xn HO or Connection Resume from an MBS supporting NG-RAN towards an MBS supporting target NG-RAN, depending on configuration. It is unclear how SMF provides this information. In fact, SMF will provide this information in the same way as Xn-based HO or Connection Resume from a non-MBS supporting source RAN node towards an MBS supporting target RAN node. This is because if the source RAN supports MBS but does not support RRC inactive reception, it does not provide MBS assistance information. So according to configuration, if not all MBS-capable RAN nodes support the RRC Inactive reception, the SMF needs to provide this information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying the SMF provides the MBS Assistance info in the N2 SM information as specified in the existing procedures for Xn HO and Resume from support RAN node to Support RAN node case. Clarifying the scenario in the Note. Editorial change on how source RAN provides this info to target RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how SMF provides this information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 7.2.3.8	Mobility procedures to enable delivery of multicast MBS session data to UEs in RRC_INACTIVE state

##### 7.2.3.8.1	General

The procedures in clauses 7.2.3.2, 7.2.3.3, 7.2.3.4, 7.2.3.6 and 7.2.3.7 apply with the following additions:

-	If the MBS assistance information is available at the SMF for an MBS session that the UE joined,

-	If there is signalling of MBS session related information from SMF to target RAN node, the SMF includes MBS Assistance Information in the N2 SM information.

-	For N2-based handover, when the SMF provides the MBS session related information as part of the associated PDU Session context data as specified in the existing procedures, the SMF also includes the MBS Assistance Information in the N2 SM information.

-	For Xn-based handover or Connection Resume from a non-MBS supporting source RAN node towards an MBS supporting target RAN node, the SMF provides MBS session related information as part of the associated PDU Session context data as specified in the existing procedures and the SMF also includes the MBS Assistance Information in the N2 SM information.

-	For Xn-based handover or Connection Resume from an MBS supporting source RAN node towards an MBS supporting target RAN node, depending on configuration, the SMF may provide the MBS Assistance Information in the N2 SM information as specified in the existing procedures.

-	If the MBS assistance information is available at the source RAN node for an MBS session that the UE joined, for Xn-based handover or Connection Resume, the MBS session information including MBS assistance information is transferred from source NG-RAN towards target NG-RAN as specified in TS 38.300 [9].

NOTE 1:	In deployments where not all the MBS supporting NG RAN nodes support delivery of multicast MBS session data to UEs in RRC_INACTIVE state, a source NG-RAN node not supporting delivery of multicast MBS session data to UEs in RRC_INACTIVE state will not provide MBS assistance information for the MBS session to the target NG-RAN node. In this case, the SMF needs to provide the MBS Assistance Information to target MBS-capable RAN node in Xn-based Handover or Connection Resume according to operator’s policy.

NOTE 2:	It is assumed that a RAN node not supporting delivery of multicast MBS session data to UEs in RRC_INACTIVE state ignores the MBS assistance information for the MBS session.

For a UE that is receiving multicast MBS data in CM-CONNECTED with RRC_INACTIVE state, the scenarios for UE mobility are as follows:

-	UE moves to new cell within the RAN Notification Area (RNA).

-	UE moves outside the current RAN Notification Area but within the current Registration Area (RA).

-	UE moves out of the current Registration Area.

END OF CHANGES