SA WG2 Meeting #S2-160 AH-e	S2-2400294r1

22 - 29 January, 2024, Electronic meeting

- Title:	KI #2, New Sol：PCF based credit limit control for subscribers

- Source:	vivo

- Document for:	Approval

Agenda Item:	19.4

Abstract:	Propose to a new solution to KI#2.

# 1 Discussion

This contribution proposed a solution to achieve credit limit control.

# Proposal

It is proposed to add the following solution to solve KI#2.

START OF CHANGES

## 6.x	Solution #X: Credit limit control according to subscription

### 6.x.1	Key Issue mapping

Table 6.X.1-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | X |
| Solution x: PCF based credit limit control for subscribers |  | x |  |  |
| … |  |  |  |  |

### 6.x.2	Functional Description

In this solution, CHF controls the available energy credit limit of subscribers, which is enhanced by CHF's Nchf_SpendingLimitControl service. CHF is aware of how much credit is used by one single subscriber and whether a credit limit is exceeded. When the credit limit is exceeded, CHF is responsible to notify to the PCF to trigger policy update for the subscribers.

For the updated policy, if one subscriber’s energy usage is exceeded to the subscribed credit limit, the network may refuse or modify UE’s access to application services provided by specific network slices, the PCF will generate new URSP and provided to UE. When the new policy is to change the accessed network slice, e.g., choose a new network slice which has low energy consumption or degraded service experience, the policy update will also cause network slice reselection or AMF re-allocation. Another situation is not to change the accessed network slice, but to change the network performation of the whole application service, this may be achieved by modify the SM policy to update the QoS parameter related to the application service.

### 6.x.3	Procedures

- According to the subscription of UE, PCF knows policy of the subscriber needs to be decided according to the energy efficiency for example energy usage of the UE, then PCF triggers the reporting/notification of the spending limit of CHF.

There may be one or more policy counters for the UE's energy consumption credit limit:

- Daily/monthly EE consumption reaches XX J or XX Wh

- Daily/monthly EE expenses/phone bills reached XX dollars

- The current energy efficiency EE is larger than or less than a threshold

- PCF requests to subscribe CHF’s SpendingLimitControl service

- Response from CHF to PCF

- When CHF detects that the EE policy counter state has changed, or CHF may detect that the EE policy counter state will change at some point in the future, which means the credit limit exceeds for the subscriber, CHF notify to the PCF

- Response from PCF to CHF

- PCF decides to update the related policy. Here, the policy maybe UE policy, AM policy and SM policy.

### 6.x.4	Impacts on existing services, entities and interfaces

The following impacts have been identified:

CHF:

- During spending limit control, when the total energy usage exceeds or approaches the policy counter, CHF report or notify to the PCF

- Support policy counter related to energy efficiency

PCF

- Policy decision based on the spending limit control related to energy efficiency

END OF CHANGES