SA WG2 Meeting #S2-160AHE	S2-2400389r01

22 - 29 January, 2024, Electronic	 (Revision of S2-24xxxx)

Source:	Samsung

Title:	KI#3, New Sol: Support for NWDAF-Based Energy Analytics

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys/Rel-19

Abstract of the contribution: This contribution proposes new solution for KI#3.

# Discussion

As it has been documented in the KI description of KI#3 in clause 5.3:

- Whether and how to enhance network analytics for network energy saving and network energy efficiency.

In order to support the network energy saving and network energy efficiency, enhancements to NWDAF-based analytics to provide the energy related analytics outputs are needed in Rel-19.

# Proposal

It is proposed to adopt the following changes into TR 23.700-66.

*** Start of the change ***

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TR 22.882: "Study on Energy Efficiency as service criteria".

[6]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[7]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[x]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[y]	3GPP TS 29.510: "Network Function Repository Services".

[z]	3GPP TS 28.552: "5G performance measurements".

[m]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[n]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

*** Next change ***

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | … | 3 |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| … |  |  |  |  |
| x |  |  |  | x |

*** Next change (all new text) ***

## 6.x	Solution #X: Support for NWDAF-Based Energy Analytics

### 6.x.1	Key Issue mapping

This solution maps to KI#3.

### 6.x.2	Functional Description

#### This solution aims to address the KI#3 to extend the NWDAF-based analytics with energy related aspects to support network energy saving and enhance network energy efficiency.

Currently, NWDAF by collecting and analysing data from multiple sources including 5GC NFs, OAM can provides various analytics including UE behaviour, network performance, service experience defined in TS 23.288 [x]. The existing analytics do not provide information about the energy-related aspects of the network which is quite important for the energy-aware operation of network, e.g., network function selection or exposing the analytics for the authorized third parties. Hence, this solution proposes providing new analytics by NWDAF for this purpose.

In order to assist with network energy saving and efficiency decision making, determining and enforcing the energy related strategies and policy control, optimising NF planning on its energy saving behaviour (e.g. energy saving states planning, reducing capabilities), multiple energy related objectives need to be considered by the 5GC and the AF, i.e. the historical, current and future energy consumption and energy efficiency at different granularities (e.g. at RAN level, Core Network level, network slice level, NF level, UE level, PDU session level, and/or QoS flow level). The 5GC and AF may interact and negotiate to determine or update energy saving and energy efficiency decision and energy related strategies and policy control before the service starts and during the service operation. For example, the 5GC and 3rd party may negotiate the maximum energy consumption allowance for providing a service to one or more subscribers by considering the prediction of the feasible energy consumptions and the associated service operation strategy provided by the NWDAF. PCF can change the PCC rule to decrease the QoS level if a specific QoS flow cost too much energy. AF can change the service level QoS requirement when the received energy saving and energy efficiency analytics indicate a specific service will cost too much energy.

The NWDAF may also expose the analytics of other service quality information associated to the derived energy related information (e.g. the traffic volume that can be transmitted, the delay and data rate that can be achieved by 5GS using the derived EC or EE) to assist the consumers to make energy saving and efficiency decision making. As a result, the 5CG and AF are able to reach agreements on the implementable energy related policy control that satisfies the requirements of both the AF and 5GC.

In order to assist the network energy saving and enhance network energy efficiency, in different use cases, the NWDAF might be required to provide the energy related output analytics at different granularities by the consumer. For example, to make energy saving decision by considering NF (re-)selection or to change the energy states of NFs as required by SA1, the consumer may require the NWDAF to provide the energy related analytics outputs at NF level. To assist with the session management or to determine the implementable energy consumption and efficiency of a service, the consumer may require the NWDAF to provide the energy related analytics outputs at UE level, PDU session level or QoS flow level. Therefore, according to the consumer request, different inputs might be collected by the NWDAF from different data sources, e.g. performance related data collected from OAM, energy monitoring and management related data collected from energy management network function (EMF) if any, QoS monitoring data collected from SMF or UPF, NF load and status data collected from NRF etc. The NWDAF may also collect MDAS-based analytics as the inputs to derive the NWDAF-based analytics. In short, in order to provide the statistics and/or prediction on energy consumption and/or energy efficiency towards the respective consumer for the desired granularity, the NWDAF would then need to collect input data to gain knowledge related to:

- the involved UE(s) and their location including potential mobility

- UE communication patters or the data volume UL/DL and bit rate per desired granularity

- 5G core and RAN nodes involved and their utilization for enabling the desired granularity

- energy consumption and energy efficient information related to the involved 5G core and RAN nodes

In order to compute the prediction and statistics of the energy consumption and energy efficiency at finer granularities (e.g. at UE level, PDU session level, and/or QoS flow level), the NWDAF can consider the percentage of the total resources of the 5GC, NF, slice or RAN node used by the UE, PDU session or the QoS flow. For example,

- The energy consumption of a PDU session can be determined by summing up the percentage of the EC consumed by all network functions that serve this PDU session.

- The energy consumption of a QoS flow can be calculated by estimating the percentage of the PDU session or slice resource utilisation by the QoS flow, i.e. the percentage can be estimated by considering the data volume of the specific QoS flow and the overall data volume of the slice the QoS flow belong to.

- The energy consumption of a UE can be calculated by summing up the energy consumption of all the PDU sessions or QoS flows that belong to the UE.

Based on the energy efficiency (EE) defined in clause 3.1, Energy Efficiency (EE): The relation between a useful output and energy consumption and the definition in clause 6.7 of TS 28.554: the EE of the mobile network could be defined as Data Volume (DV) divided by Energy Consumption (EC) of the considered network elements. The data volume transmitted could be considered as a useful output. Therefore, one way for the NWDAF to determine the EE at different granularity (i.e. slice, UE, PDU session, QoS level) is to use the data volume at the required granularity divided by the Energy Consumption (EC) at the same granularity.

In some scenario, the energy consumption at NWDAF/ NF service level might be also considered from two more perspectives can be considered: the energy consumption for generating the analytics, the energy consumption for data collection. In detail,

- The energy consumption of the generated analytics can be based on the actual energy consumption during analytics generation or estimated ML model inference energy consumption received in the ML model information, and can be used by the consumer NF to determine whether to continue subscribing the analytics.

- Adaptive notification service operation defines time-varying reporting periodicity supported by the producer NF for energy saving.

#### 6.x.2.1 General Description

This clause describes how NWDAF can provide energy analytics, in the form of statistics or predictions or both, to another NF.

The service consumer may be an AF, 5GC NF (e.g. NEF, PCF), or the OAM.

The consumer of these analytics shall indicate in the request:

-	Analytics ID = "Energy information";

-	Target of Analytics Reporting: Core Network level, network slice level, NF Instance/NF set level, PDU session level, QoS flow level, and/or UE level. E.g. a single UE (SUPI/GPSI) or a group of UEs or any UE, a QoS flow or a set of QoS flows or a PDU session(s).

-	Analytics Filter Information:

-	DNN;

-	Application ID;

-	QoS requirements (e.g., 5QI, QoS Characteristics);

-	S-NSSAI;

-	NF Instance ID;

-	NF services ID(s);

-	Area of Interest (AoI);

- 	SUPI;

-	an optional list of analytics subsets that are requested;

-	Optional preferred level of accuracy of the analytics;

-	Optional preferred level of accuracy per analytics subset;

-	Optional Reporting Threshold;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Indication whether to return the estimated energy cost required to provide these analytics.

NOTE:	This indication can instruct the NWDAF to a) only return the predicted energy cost estimated for this analytics request/subscription (without actually executing the analytics), b) return in the analytics subscription response an estimate of the predicted energy cost for this analytics subscription, c) return an estimate of the consumed energy cost for the analytics generation also in the analytics response/notify, or a combination of b) and c).

Editor’s Note: Other input parameters of the request are FFS.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.5.3 of TS 23.288 [x].

#### 6.x.2.2 Input Data of Energy Related Analytics

The NWDAF may collect the service data as listed in Table 6.x.2.2-1 from OAM and/or the 5GC network function in charge of energy monitoring and management of the energy consumption and energy efficiency (i.e. EMF, if any).

Table 6.x.2.2-1: Data collected by NWDAF for energy related analytics from OAM and EMF

| Information | Source | Description |
| --- | --- | --- |
| Time window | EMF, OAM | The time window or the time duration for the collected parameters. The window is between the window start time and stop time. |
| Energy Consumption at 5GC, NF, Network Slice, gNB or NG-RAN level. | EMF, OAM | Energy consumption (EC) at various granularities, according to output analytics to be derived based on consumer request. The EC can be collected from OAM or EMF (if available). For the Energy Consumption parameters collected from OAM: - 5GC Energy Consumption is obtained by summing up the Energy Consumption of all the Network Functions (ECNF) that compose the 5G core network as defined in clause 6.7.3.2 of TS 28.554. - NF energy EC is the sum of the energy consumption of PNF(s) and/or VNF(s) which compose the NF, as defined in clause 6.7.3.1 of TS 28.554. - Network Slice Energy Consumption is obtained by summing up the Energy Consumption of all the Network Functions (ECNF) that compose the network slice as defined in clause 6.7.3.3 of TS 28.554. - The Energy Consumption (EC) of the gNB is obtained by summing up the Energy Consumption of all the Network Functions (NF) that constitute the gNB, as defined in clause 6.7.3.4.2 of TS 23.554. - Energy Consumption of the NG-RAN is obtained by summing up the Energy Consumption of all the gNB that constitute the NG-RAN, as defined in clause 6.7.3.4 of TS 28.554. |
| Identifiers (i.e. PLMN ID, NF ID/ NF Set ID, S-NSSAI, gNB ID, etc.) | EMF, OAM | Identifiers of 5GC/NF instance or NF set/ slice/RAN node correspond to the collected EC or EE. |
| Energy efficiency of NG-RAN or network slice | EMF, OAM | The energy efficiency of NR-RAN or network slice as defined in clause 6.7.1 and 6.7.2 of TS 28.554. |

The NWDAF may also collect data from MDAF. The Type of Energy Efficiency Problem provided by MDAF can help NWDAF with identifying and verifying the derived prediction or statistics of the energy efficiency, i.e. at NF level.

Table 6.x.2.2-2: Input data collected from OAM (MDAF)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | MDAF | A time stamp associated with the collected information. |
| EnergyEfficiencyProblemType | MDAF | Indication of type of the energy efficiency issues. The allowed value is one of the enumerated values: HighEnergyConsumption, LowEenergyEfficiency, Other, Unknown, as defined in clause 8.4.4 of TS 28.104 [n]. |
| EnergyEfficiencyProblematicObject | MDAF | Indication of NR cells or NFs where the energy efficiency issues occurred or potentially occur, as defined in clause 8.4.4 of TS 28.104 [n]. |

In order to determine the EC and EE, in particular the EC and EE at relatively finer granularities (i.e. UE level, PDU session, and QoS flow level), the NWDAF also needs to collect data from other NFs. For example, based on the principle of computing the EC of UE, PDU session or QoS level, the NWDAF needs to determine or estimate the percentage of resources used by the UE, PDU session or QoS level among the overall usage. For example, by acknowledging the number of UEs for a SMF or a slice and the EC of the SMF or slice, the NWDAF is able to determine the average UE level EC. By acknowledging the traffic volume of the UE and that of the associated SMF, and the EC of the SMF, the NWDAF is able to determine the EC of this specific UE. Using similar methods, the NWDAF is able to derive the prediction and statics of the EC and EE at finer granularity by collecting the informative data.

Table 6.x.2.1-3: Data collected by NWDAF for energy related analytics from NFs

| Information | Source | Description |
| --- | --- | --- |
| Time window | UPF, SMF, or AF | The time window or the time duration for the collected parameters. The window is between the window start time and stop time. |
| UL/DL/overall data volume | UPF, SMF, or AF | UL/DL/overall (UL and DL) data volume of a slice, a UE, a PDU session, Qos flow, UPF, RAN node, 5GC, according to the consumer request. Could be Volume Measurement and/or Throughput Measurement. |
| latency | OAM | latency in NG-RAN per Cell Id for the traffic type of interest and in the Area of Interest, which are collected from the OAM. |
| UserDataUsageTrends | UPF | Throughput Statistic Measurement (average and/or peak throughput) over the measurement period for the PDU Session or per application. |
| NF Load information | NRF | Load per NF |
| NF status | NRF | The status of a specific NF instance(s) (registered, suspended, undiscoverable) as defined per TS 29.510 [y]. |
| NF resource usage | OAM | The usage of assigned virtual resources currently in use for specific NF instance(s) (mean usage of virtual CPU, memory, disk) as defined in clause 5.7 of TS 28.552 [z]. |
| NF resource configuration | OAM | The life cycle changes of specific NF resources (e.g. NF operational or interrupted during virtual/physical resources reconfiguration) |
| NF energy state | NRF or NF | Current NF energy state or NF energy state schedule/planning. |
| number of Qos flows | OAM, SMF | The Max/mean number of Qos flows. The measurement can be split into per S-NSSAI at the SMF as defined in clause 5.3.2.1 of TS 28.552. Or the number of the QoS flow per PDU session at the SMF collected from SMF. |
| number of PDU sessions | OAM, SMF | The Max/mean number of PDU sessions established by SMF. The measurement can be split into per S-NSSAI as defined in clause 5.3.1 of TS 28.552. Or the number (active) PDU sessions collected from SMF. |
| Number of UEs | OAM, AMF | The Max/mean number of UEs register to AMF collected from AMF or the number of subscribers per slice as defined in clause 5.2.1 of TS 28.552 collected from OAM. |

In order to derive the output analytics of the renewable energy related information, the NWDAF needs collected the ratio of renew energy in the total energy supply and the Coefficient of carbon from the 3rd party or EMF if any.

Table 6.x.2.1-4: Input service data related to renewable energy

| Information | Source | Description |
| --- | --- | --- |
| ratio of renew energy in the total energy supply | AF, EMF, OAM | The ratio of renew energy in the total energy supply |
| Coefficient of carbon or carbon emission | AF, EMF, OAM | Coefficient of carbon is Used to derive the carbon emission, i.e. the statistically observed grams per mill watthour for any use of energy in the country or region where the service is provided. |

Editor's note:	Coordination with SA WG5 is needed regarding network energy related data collection from OAM.

#### 6.x.2.3 Energy Related Output Analytics

According to the consumer request, the NWDAF is able to determine the prediction and statistics of energy consumption and energy efficiency along with other corresponding information at different granularities using the input data

Table 6.x.2.3-1: statistics of the output analytics

| Information | Description |
| --- | --- |
| List of energy-related information(1…max) (NOTE 1) | Observed statistics during the Analytics target period in the consumer request. The energy-related information includes statistics of maximum, minimum, variance or average value of energy consumption and energy efficiency at the consumer required granularity over the time slot(s). The energy consumption includes the EC of renewable energy. |
| > Time slot entry (1..max) | List of time slots within the Analytics target period. The time slots of within which the statistics and/or predictions energy-related information is provided. |
| >>Time slot start | Time slot start within the Analytics target period. |
| >>Duration | Duration of the time slot. |
| >>Application ID | Identifiers of the one or more applications in use during the time slot within which the EE and EC is derived. |
| >> Identifiers (i.e. S-NSSAI/ NSI ID, NF ID/ NF Set ID, gNB ID, PLMN ID, PDU session ID, QFI, UE ID(s) etc.) | Identifiers of slice or slice instance/NF instance or NF set/ RAN node/5GC/PDU session.-/QoS flow/ UE that correspond to the EE and EC derived by the NWDAF, based on consumer request. |
| >> NF type | Type of the NF instance. |
| >> energy consumption at required granularities | The maximum/ minimum / average / variance value of EC at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, or of an Area subset (e.g. AOI). |
| >> energy consumption of renewable energy at required granularities | The maximum/ minimum / average / variance value of renewable energy consumption at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, or of an Area subset (e.g. AOI). |
| >> Carbon emission at required granularities | The maximum/ minimum / average / variance of Carbon emission at the consumer required granularity over a period of time, of an Area subset (e.g. AOI), or of a datacentre hosting the NF. |
| >> energy efficiency at required granularities | The maximum/ minimum / average/ variance value of energy efficiency at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, of an Area subset (e.g. AOI), or of a datacentre hosting the NF. |
| >> maximum energy credit (limit) | The upper bound of energy used by the 5G system to provide services provided to one or more subscribers, i.e. the EC of per service per UE level. |
| >> Energy saving for the QoS flow | Increasing or decreasing the QoS requirement level to save or increase energy consumption |
| >> NF energy state planning | Indicates the statistics of energy state of relevant NF(s). The information might also include information on the time/schedule when the NF instance was observed to be in a specific energy state. |
| >> traffic volume | The traffic volume used to derive the predictions or statistics of the energy related information at the required granularity, i.e. the energy consumption or energy efficiency. |
| >> traffic/packet rate | The traffic/packet rate associated predictions or statistics of the energy related information at the required granularity or the traffic/packet rate used to derive the EE and EC. |
| >> traffic latency/ packet delay | The traffic latency/ packet delay associated predictions or statistics of the energy related information at the required granularity or the traffic/packet rate used to derive the EE and EC. |
| >> Analytics energy consumption | Indicates the estimated energy costs derived for this analytics request/subscription. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". |  |

Table 6.x.2.3-1: predictions of the output analytics

| Information | Description |
| --- | --- |
| List of energy-related information(1…max) (NOTE 1) | Predictions during the Analytics target period in the consumer request. The energy-related information includes predictions of maximum, minimum, variance or average value of energy consumption and energy efficiency at the consumer required granularity over the time slot(s). The energy consumption includes the EC of renewable energy. |
| > Time slot entry (1..max) | List of time slots within the Analytics target period. The time slots of within which the statistics and/or predictions energy-related information is provided. |
| >>Time slot start | Time slot start within the Analytics target period. |
| >>Duration | Duration of the time slot. |
| >>Application ID | Identifiers of the one or more applications in use during the time slot within which the EE and EC is derived. |
| >> Identifiers (i.e. S-NSSAI/ NSI ID, NF ID/ NF Set ID, gNB ID, PLMN ID, PDU session ID, QFI, UE ID(s) etc.) | Identifiers of slice or slice instance/NF instance or NF set/ RAN node/5GC/PDU session.-/QoS flow/ UE that correspond to the EE and EC derived by the NWDAF, based on consumer request. |
| >> NF type | Type of the NF instance. |
| >> energy consumption at required granularities | The maximum/ minimum / average / variance value of EC at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, or of an Area subset (e.g. AOI). |
| >> energy consumption of renewable energy at required granularities | The maximum/ minimum / average / variance value of renewable energy consumption at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, or of an Area subset (e.g. AOI). |
| >> Carbon emission at required granularities | The maximum/ minimum / average / variance of Carbon emission at the consumer required granularity over a period of time, of an Area subset (e.g. AOI), or of a datacentre hosting the NF. |
| >> energy efficiency at required granularities | The maximum/ minimum / average/ variance value of energy efficiency at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, of an Area subset (e.g. AOI), or of a datacentre hosting the NF. |
| >> maximum energy credit (limit) | The upper bound of energy used by the 5G system to provide services provided to one or more subscribers, i.e. the EC of per service per UE level. |
| >> Energy saving for the QoS flow | Increasing or decreasing the QoS requirement level to save or increase energy consumption |
| >> NF energy state planning | Indicates the statistics of energy state of relevant NF(s). The information might also include information on the time/schedule when the NF instance was observed to be in a specific energy state. |
| >> traffic volume | The traffic volume used to derive the predictions or statistics of the energy related information at the required granularity, i.e. the energy consumption or energy efficiency. |
| >> traffic/packet rate | The traffic/packet rate associated predictions or statistics of the energy related information at the required granularity or the traffic/packet rate used to derive the EE and EC. |
| >> traffic latency/ packet delay | The traffic latency/ packet delay associated predictions or statistics of the energy related information at the required granularity or the traffic/packet rate used to derive the EE and EC. |
| >> Analytics energy consumption | Indicates the estimated energy costs derived for this analytics request/subscription. |
| Confidence | Confidence of this prediction. |

Based on energy consumption information,

- If the consumer is AF, the AF may decide whether to enter the energy saving mode or not.

- If the consumer is PCF, the PCF may make policy adjustments.

- If the consumer is OAM, the OAM may configure the NF to enter an energy-saving mode.

- If the consumer is AMF, the AMF may performs cell-based paging and provides a list of recommended cells.

- If the consumer is SMF, the SMF may select a suitable UPF for the UE based on the energy consumption information.

- If the consumer is CHF, the CHF may record the energy consumption information for charging.

### 6.x.3	Procedures

Figure 6.x.3-1 Procedures for energy related analytics

1.	The Consumer NF, e.g. AF, PCF or NEF, requests or subscribes to analytics for energy related analytics from NWDAF (possibly via NEF in case the consumer NF is an untrusted AF) and provides the input information as specified in clause 6.x.2.2 by invoking either Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request.

2a-b.	The NWDAF may subscribe to service data from EMF (5GC network function in charge of energy monitoring and management) in Table 6.x.2.2-1, if this EMF is available, by invoking Nemf_EventExposure_Subscribe service.

2c.		NWDAF subscribes to service data from SMF in Table 6.x.2.1-3 by invoking Nsmf_EventExposure_Subscribe (Event ID, SUPI(s) or Application ID).

In order to provide the requested analytics, the NWDAF may subscribe to N4 Session related input data from SMFs as defined in Table 6.x.2.1-3

2d.		The N4 session event is triggered.

2e-2f.	N4 related input data is provided by UPF to NWDAF via SMF.

2f1.	Instead of step 2e-2f, the UPF directly provides the requested N4 related input data to NWDAF.

2g-h.	The NWDAF may subscribe to input data in Table 6.x.2.2-1 and Table 6.x.2.2-2 from the OAM according to the data collection principles from the OAM, including data collection from MDAS.

2i-j.	The NWDAF may subscribe the service data from AF in the Table 6.x.2.1-3 and Table 6.x.2.1-4 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe (Event ID = energy related information, Application ID, Event Filter information, Target of Event Reporting) service as defined in TS 23.502 [3].

2k-l.	The NWDAF may subscribe the service data from UPF in the Table 6.x.2.1-3 by invoking Nupf_EventExposure_Subscribe (Event ID = energy related information, Application ID, Event Filter information, Target of Event Reporting) service as defined in TS 23.502 [3].

2m-n.	The NWDAF subscribes the service data from AMF in Table 6.x.2.1-3 using Namf_EventExposure_Subscribe service.

2o-p.	The NWDAF subscribes to the load of NF instances Table 6.x.2.1-3 by using Nnrf_NFManagement_NFStatusSubscribe.

3.	The NWDAF derives requested analytics, in the form of energy consumption and energy efficiency related statistics or predictions or both.

4.	The NWDAF provides the requested energy related analytics to the consumer, using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5-7.	If the consumer NF subscribes to energy related analytics at step 1, when the NWDAF generates new analytics, it notifies the newly generated analytics to the consumer NF.

### 6.x.4	Impacts on existing services, entities and interfaces

NWDAF:

- Collect new energy related inputs from 5GC NFs (i.e. SMF, EMF), AF and OAM.

- Generate new energy related output analytics including predictions and statistics.

- Expose energy related analytics outputs to consumers.

AF:

- Expose energy related information to NWDAF.

5GC NFs (e.g. NRF, SMF, UPF):

- Expose required information for energy related analytics to NWDAF.

*** End of the change ***