3GPP TSG-SA WG2 Meeting #161	S2-2402643

26 February - 01 March, 2024, Athens, GR	(revision of S2-2400244)

Source:	CATT

Title:	Update Architectural Assumptions

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract: Proposes to update the Architectural Assumptions for the FS_XRM_Ph2 based on the Architectural Assumptions for the FS_XRM.

1. Introduction

Release-19 will include the non-3GPP access enhancement for XRM services, and the PDU Set QoS parameters and PDU Set information for 3GPP access are used for non-3GPP access, no additional PDU Set QoS parameters and PDU Set information is defined for non-3GPP access. For the non-3GPP access, only one NF (i.e. PSA UPF or non-3GPP access NF) performs the ECN marking for the L4S.

An end-to-end transport connection may include different media types as described in RFC 8860 [x] ;

An end-to-end transport connection may include a media stream and its associated signalling as described in RFC 5761[y]. The associated signalling can be treated as a special type of media data.

In order to support DSCP, the UPF and the IP router(s) in the N3/N9 interface support the different IP packet forward handling based on the DSCP of the outer IP header of the GTP-U packets.

The tethered devices of the UE are assumed to support IP (IPv4 and/or IPv6) traffic.

2. Text Proposal

It is proposed to capture the following changes to the TR 23.700-70 for the FS_XRM phase2.

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2".

[5]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[6]	IETF RFC 6904: "Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)".

[7]	IETF RFC 9335: "Completely Encrypting RTP Header Extensions and Contributing Sources".

[8]	IETF draft-ietf-avtcore-rtp-over-quic: "RTP over QUIC (RoQ)".

[9]	IETF draft-ietf-moq-transport: "Media over QUIC Transport".

[10]	IETF experimental draft-ietf-avtext-framemarking: "Frame Marking RTP Header Extension".

[11]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[12]	TR 26.926 v18.1.0: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[13]	TR 26.925 v18.1.0: "Typical traffic characteristics of media services on 3GPP networks".

[14]	IETF RFC 9330: "Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture.

[15]	IETF RFC 9331: "The Explicit Congestion Notification (ECN) Protocol for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[16]	IETF draft-ietf-tsvwg-ecn-encap-guidelines-22: "Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP".

[17]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[18]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[19]	IETF RFC 9332: " Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[20]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[21]	IETF draft-ietf-tsvwg-udp-options: "Transport options for UDP".

[22]	IETF RFC 6363: "Forward Error Correction (FEC) Framework".

[23]	IETF RFC 6364: "Session Description Protocol Elements for the Forward Error Correction (FEC) Framework".

[24]	IETF RFC 6681: "Raptor Forward Error Correction (FEC) Schemes for FECFRAME".

[25]	IETF RFC 6682: "RTP Payload Format for Raptor Forward Error Correction (FEC) ".

[26]	IETF RFC 6695: "Methods to Convey Forward Error Correction (FEC) Framework Configuration Information".

[27]	IETF RFC 6816: "Simple Low-Density Parity Check (LDPC) Staircase Forward Error Correction (FEC) Scheme for FECFRAME".

[28]	IETF RFC 6865: "Simple Reed-Solomon Forward Error Correction (FEC) Scheme for FECFRAME".

[29]	IETF RFC 8680: "Forward Error Correction (FEC) Framework Extension to Sliding Window Codes".

[30]	IETF RFC 8681: "Sliding Window Random Linear Code (RLC) Forward Erasure Correction (FEC) Schemes for FECFRAME".

[31]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[32]	IETF RFC 8627: "RTP Payload Format for Flexible Forward Error Correction (FEC)".

[x]	IETF RFC 8860: "Sending Multiple Types of Media in a Single RTP Session".

[y]	IETF RFC 5761: "Multiplexing RTP Data and Control Packets on a Single Port".

Editor’s note:	References [8], [9] and [10] cannot be formally referenced until published as RFC.

# * * * * Second change * * * *

4.1	Architectural Assumptions

-	The architecture, framework and the QoS model as specified in TS 23.501 [2], TS 23.502 [3], and TS 23.503 [4] are regarded as the baseline for this study for both 3GPP access and non-3GPP access. The procedures for XRM in 3GPP Rel-18 are described in TS 23.501 [2], TS 23.502 [3] and TS 23.503 [4].

-	The functional split in 5GS between UE/5G-RG, AN and CN remains unchanged.

-	End-to-end media flows may be fully or partially encrypted.

-	The interface between 3GPP UE and tethered devices behind the UE is outside of scope.

-	An end-to-end transport connection may include different media types as described in RFC 8860 [x].

-	An end-to-end transport connection may include a media stream and its associated signalling as described in RFC 5761 [y].

-	The UPF and the IP router(s) in the N3/N9 interface support the different IP packet forward handling based on the DSCP of the outer IP header of the GTP-U packets.

-	The tethered devices of the UE are assumed to support IP (IPv4 and/or IPv6) traffic.

-	Only the PDU Set QoS parameters and PDU Set information defined for 3GPP access can be used for non-3GPP access, no additional PDU Set QoS parameters and PDU Set information is defined for non-3GPP access.

-	Only one NF (i.e. PSA UPF or non-3GPP access NF) performs the ECN marking for the L4S to the non-3GPP access UE traffic at any time.

# * * * * End of Change * * * *