3GPP TSG-WG SA2 Meeting #161	S2-2402720

Athens, Greece, 26 February - 1 March, 2024	(revision of S2-240xxxx)

Source:	Lenovo

Title:	KI#3, AIoT communication for connectivity topology 2

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: This paper proposes a new solution to KI#1 and KI#3 describing the indirect AIoT communication via UE as an intermediate node and a new AIoT NF managing the AIoT communication.

# 1. Introduction/Discussion

The KI#1 includes the among others the following aspect to be studied:

-	System architecture identified along with the solutions for KI#2 and KI#3.

-	Validation of the Ambient IoT Device identifier;

-	Whether and how to secure device operations and services for an Ambient IoT Device or a group of Ambient IoT Devices;

Further, the Key Issue #3 includes the following aspect to be studied:

-	Study how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices.

NOTE:	Including whether there is a need to support session based transfer between Ambient IoT Device and the network considering the device types and capabilities.

-	Study which of the enabled Ambient IoT services are exposed to AF and how, e.g. for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.

There are 2 main communication topologies assumed in the Ambient AIoT study, namely:

-	Topology 1: BS <--> Ambient IoT Device;

-	Topology 2: BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control.

This solution focuses on the Topology 2. It is proposed that a UE, which is capable of AIoT transmissions towards AIoT devices, indicates its capability during the registration with the network.

This solution proposes a solution for the AIoT information transfer between the AIoT application server and the AIoT devices via the control plane of the 5GS. A new NF called Ambient AIoT gateway (AIoT-GW) is introduced.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-13.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 | Key Issue #3 |
| #1 |  |  |  |
| #2 |  |  |  |
| #X | x |  | x |

# * * * * Second change * * * *

## 6.x	Solution #X: AIoT communication for connectivity topology 2

### 6.x.1	Key Issue mapping

This solution addresses the following requirements:

-	from Key Issue #1 the requirement:

-	System architecture identified along with the solutions for KI#2 and KI#3.

-	from Key Issue #3 the requirements:

-	Study how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices.

NOTE:	Including whether there is a need to support session based transfer between Ambient IoT Device and the network considering the device types and capabilities.

-	Study which of the enabled Ambient IoT services are exposed to AF and how, e.g. for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.

### 6.x.2	Functional Description

This solution focuses on the Topology 2. It is proposed that a UE, which is capable of AIoT transmissions towards AIoT devices, indicates its capability during the registration with the network.

This solution proposes that the AIoT information transfer between the AIoT application server (AIoT AS) and the AIoT devices is via the control plane of the 5GS. A new NF called Ambient AIoT gateway (AIoT-GW) is introduced. The assumed architecture is shown in Figure 6.X.2-1.

Figure 6.X.2-1:  Assumed architecture for AIoT services using topology 2

The AIoT-GW implements the following functionalities:

-	Establishing an association with the AMF per AIoT capable UE.

-	Receiving and transmitting AIoT data/signalling from/to the AIoT application server.

-	Selecting appropriate AIoT capable UE for the transmission of DL AIoT data/signalling to the target transmission area. Receiving and transmitting AIoT data/signalling from/to the AIoT capable UE via AMF.

-	Perform store-and-forward functionality for the AIoT data/signalling.

-	Creating charging data for the AIoT data/signalling and transmit the data to the CHF.

### 6.x.3	Procedures

Two procedures are shown in Figure 6.X.3-1 to explain the concept of this solution:

-	the procedure for AIoT capable UE registration with the AMF and AIoT session establishment with the AIoT-GW.

-	the procedure for and AIoT data/signalling transmission via the control plane.

Figure 6.X.3-1:  Signalling flow of the AIoT session establishment for the AIoT capable UE between the AIoT-GW and the AIoT AF/AS

The detailed description of the steps is provided as follows:

0a.	The UE implements capability to act as an intermediate node AIoT data/signalling transmission to AIoT devices. The UE may implement a client functionality which is responsible for: 1) reception and transmission of AIoT data/signalling from/to the network (e.g. from AIoT-GW), 2) processing the data and 3) transmission and reception of AIoT data/signalling to/from the AIoT devices.

0b.	The UDM/UDR stores AIoT device subscription data and (AIoT) application subscription data. The application subscription data contains the configuration information for the AIoT application including credentials. The AIoT device subscription data can include individual device ID and corresponding credentials.

0c.	The AIoT-GW is configured to serve one or more AIoT services as described in the clause 6.x.2.

1.	The UE sends a Registration Request message which includes 1) indication that the UE is capable of AIoT communication serving as intermediate node, 2) whether the UE can receive and transmit AIoT data from/to the network via CP or UP communication path, 3) AIoT capabilities (e.g. supported frequency bands, range of communication or AIoT radio coverage area, one or more power modes.

2.	The AMF can retrieve the UE’s subscription data from the UDM/UDR. The UDM/UDR may indicate whether the UE is enabled/disabled to serve as AIoT intermediate node, a list of AIoT service(s) for which is allowed to transmit AIoT data/signalling to the AIoT devices, AIoT-GW selection information, etc.

3a.	The AMF determines to register the UE (which is authorized to act as AIoT intermediate node) with a AIoT-GW for AIoT service(s).

3b.	The AMF sends a request message to the AIoT-GW to register or establish an association for the UE.

4a.	The AIoT-GW creates context for the UE acting as an AIoT intermediate node. The AIoT-GW may determine to update the UE configuration for the AioT data/signalling transmission operation.

4b.	The AMF sends a Registration Accept message (or UE Configuration Update Command, UCU) which includes an indication whether the operation as AIoT intermediate node is allowed (or disallowed) and which AIoT services are allowed to be served. If the AIoT-GW has included N1 AIoT container in step 4a, the AMF also may include the N1 AIoT container in the message.

5.	The AIoT AF/AS may send to the AIoT-GW AIoT data/signalling to be transmitted to the AIoT devices. The message may include AIoT service ID, type of AIoT service, target transmission area, AIoT data/signalling, list of AIoT device IDs or group ID.

6.	The AIoT-GW may send a request to the AMF for the current UE status, i.e. whether the UE is reachable and UE location.

7.	The AIoT-GW performs a selection of one or more UEs acting as AIoT intermediate node or RAN nodes for the transmission of the AIoT data/signalling.

The steps 8a, 8b, 8c, and 8d build a procedure for AIoT service or session configuration between the UE and the AioT-GW. The session can be used for AIoT data exchanges in the DL or in the UL. During this step the UE and the AIoT-GW may exchange a session identifier (e.g. AIoT Service ID, or application ID, or UL port ID and DL port ID, etc.) which is used for the correct routing of the AIoT data/signalling in the UE and the AIoT-GW.

8a.	The AIoT-GW creates an AIoT service configuration information for the selected UE. This information is included in a N1 AIoT container.

8b.	The AMF uses NAS DL Transport message to carry the AIoT service ID, N1 AIoT container (including AIoT configuration data) to the UE.

8c.	The UE replies to the AIoT-GW by sending NAS UL Transport message to the AMF which includes the AIoT service ID, AIoT container (including AIoT configuration data), etc.

8d.	The AMF forwards the received N1 AIoT container to the target AIoT-GW. The AMF determines the target AIoT-GW based on the AIoT service ID.

The steps 9a, 9b, 9c, and 9d build a procedure for AIoT data/signalling transmission between the AIoT-GW and the UE.

9a.	The AIoT-GW includes the AIoT data/signalling and associate parameters in an N1 AIoT container to the AMF.

9b.	The AMF sends to the UE a NAS DL Transport message which includes the N1 AIoT container and information for the UE to route the container to the correct AIoT application client in the UE or AIoT devices.

9c.	The UE acquires information from the AIoT devices. The UE (or application client in the UE) prepares an uplink AIoT data/signalling to be transmitted to the AIoT-GW.

9d.	The AMF forwards the received N1 AIoT container to the target AIoT-GW.

10.	The AIoT-GW extracts the AIoT data/signalling from the N1 AIoT container. If needed, the AIoT-GW verify the security protection of the AIoT device or AIoT data integrity.

The AIoT-GW creates and transmits a notification message (which can be also a bulk message) to the AIoT AF/AS including the AIoT service/application ID, AIoT signalling result like status of the device (e.g. type of good to which the device is attached, ID verification failed/succeeded), AIoT device location, etc.

11.	The AIoT-GW may create charging record for the transmitted AIoT data/signalling in both UL and DL.

### 6.x.4	Impacts on existing services, entities and interfaces

The following NFs and interfaces are impacted:

-	AIoT-GW: a new NF with the functionality described in clause 6.x.2.

-	AIoT capable UE: 1) supporting AIoT capabilities to communication with the AIoT devices and 2) capabilities to select an AIoT-GW and to establish N2 association with the AIoT-GW.

-	NEF: expose a new API for AIoT session creation.

-	UDM/UDR: store AIoT device subscription data and AIoT application subscription data.

# * * * * End of changes * * * *