3GPP TSG-SA2 Meeting #S2-160-Ad Hoc-e 	S2-2402759

# E-meeting, January 22 – 29, 2024                                                      (revision of S2-2400891)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common EAS discovery and common DNAI determination for roaming UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_CEaDRoam |  |  |  |  |  | Date: |  |  | 2024-01-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In EDGE_phase2, the common EAS discovery and common DNAI determination for a set of UEs is investigated. However, it only supports the non-roaming scenario, which precludes the case that a set of UEs include roaming UE served by same operator or different operators. It is very normal the roaming UEs are using the same application and need to access particular common EAS. Currently however, it doesn't support a common EAS discovery or common DNAI determination for roaming UE. The AF request on PDU Sessions supporting HR-SBO can’t be applied to current common EAS discovery and common DNAI determination due to PCF is not involved in the procedure. See more discussion in DP S2-2312881. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add descriptions for AF request on PDU Sessions supporting HR-SBO for common EAS discovery and common DNAI determination. - Add “NEF information” as optional input in Nnef_TrafficInfluenceData_Notify service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In roaming scenarios, the import feature common EAS discovery and common DNAI determination can’t be supported, which causes many essential use cases for edge computing can’t be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.5, 5.2.6.35.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

4.3.6.5	Processing AF requests to influence traffic routing for HR-SBO session

Processing an AF requests to influence traffic routing for HR-SBO session are based on procedure described in clause 4.3.6.2 and clause 4.3.6.4 with the following differences:

In step 0 of figure 4.3.6.2-1, V-SMF supporting HR-SBO subscribes to notification of AF request by invoking Nnef_TrafficInfluenceData_Subscribe service from V-NEF (Data Set = Application Data; Data Subset = AF traffic influence request information; Data Key = S-NSSAI and/or DNN and UE IP address) and (Data Set = Application Data; Data Subset = AF traffic influence request information; Data Key = S-NSSAI and/or DNN and Any UE) and (Data Set = Application Data; Data Subset = AF traffic influence request information; Data Key = S-NSSAI and/or DNN and SUPI).

-	For any UE, the V-SMF supporting HR-SBO performs the above subscription using S-NSSAI and/or DNN as Data Key.

-	For individual UE or group of UE(s), when a PDU Session is authorized for HR-SBO as described in clause 6.7 of TS 23.548 [74], the V-SMF serving the PDU Session performs the above subscription using S-NSSAI and/or DNN and/or Internal Group Identifier(s) or SUPI as Data Key. The V-NEF subscribes to notification of AF request from V-UDR.

Steps 4 and 5 of figure 4.3.6.2-1 are replaced with the following steps:

-	Step 4: The V-UDR notifies the subscribed V-NEF of the AF traffic influence request information via invoking Nudr_DM_Notify service operation.

-	Step 5: The V-NEF notifies the subscribed V-SMF of the AF traffic influence request information via invoking Nnef_TrafficInfluence_Create service operation.

If the AF request includes an EAS Correlation indication or indication of traffic correlation, the V-NEF notifies an EAS Correlation indication or indication of traffic correlation and a Traffic Correlation ID corresponding to a set of UEs that AF request aims at, also if AF request includes a common EAS IP address or common DNAI for a set of UEs, the V-NEF notifies the common EAS IP address or common DNAI.

>>>> Next Change <<<<

5.2.6.35.4	Nnef_TrafficInfluenceData_Notify service operation

Service operation name: Nnef_TrafficInfluenceData_Notify

Description: NEF notifies NF consumer(s) about AF traffic influence request information, when receives notification of AF traffic influence request information from UDR.

Inputs, Required: Notification Correlation ID, Data Set Identifier as defined in clause 5.2.12.2.1, Target of Event Reporting as defined in clause 5.2.12.2, AF traffic influence request information, NEF information.

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

>>>> End of Changes <<<<