3GPP TSG-WG SA2 Meeting #161	S2-2403712

26 February – 1 March, 2024, Athens

Title:	Reply LS on RAN3 agreements on mobile IAB

Response to:	S2-2400065 / LS on RAN3 agreements on mobile IAB

Release:	Rel-18

Work Item:	NR_mobile_IAB-Core

Source:	SA2

To:	RAN3

Cc:

Contact person:

Name:	Meng Li

Tel. Number:

E-mail Address:	raymond.limeng@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: TS 23.501 CR 5250, CR 5303, CR 5220; TS 23.502 CR 4698, CR 4742

1. Overall Description:

SA2 would like to thank RAN3 for their LS on RAN3 agreements on mobile IAB.

Specific to the RAN3 consensus in R3-238043, SA2 would like to provide the following feedback:

- TAC/RANAC: The mobile IAB-DU’s TAC/RANAC is only configured by the OAM.

SA2 feedback:

SA2 has agreed TS 23.501 CR 5250 to align with RAN3 agreements.

- N2 based handover of mobile IAB with no PDU session activated: in some cases, a mobile IAB may have no PDU sessions activated. In these cases, RAN3 agreed that, during the N2 handover preparation procedure, the AMF shall ignore the PDU Session Resource List IE, as captured in the TP to 38.413 BL CR R3-237430.

SA2 feedback:

SA2 discussed the issue and agreed CR5220 and CR 4742 for TS 23.501 and TS 23.502, including also Xn handover.

- Indication to AMF: After the IAB-donor has completed the operation (e.g. handover the UEs, F1 removal, etc) for a non-authorized mobile IAB, the IAB-donor serving the mobile IAB-MT sends an NGAP indication to AMF in the NGAP UE CONTEXT RELEASE REQUEST message including a new cause value, so the AMF may take further action, e.g. deregister the mobile IAB-MT. The agreed RAN3 TP (R3-238021) is attached.

SA2 feedback:

Considering the SA2 has already agreed the CRs reflecting the agreement in RAN3.

SA2 has agreed the TS 23.501 CR 5303, CR 5220, TS 23.502 CR4698 to capture such changes.

2. Actions:

To RAN3 group.

ACTION: 	SA2 kindly asks RAN3 to take the above into account.

3. Date of Next SA2 Meetings:

TSG-SA2 Meeting #162		 15th – 19th April, 2024		Changsha, China

TSG-SA2 Meeting #163	       27th – 31th May, 2024		Jeju, Korea