SA WG2 Meeting #161	S2-2401986

Athens, GREECE, February 26 – March 01, 2024

Source: 	vivo

Title: 	KI#2: New Solution for NF Registration and Discovery Enhancement for supporting VFL

Document for: 	Approval

Agenda Item: 	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This contribution proposes to add a solution for KI#2 to support NF Registration and Discovery for VFL

1	Discussion

As indicated in KI#2: 5GC Support for Vertical Federated Learning, the following contents will be study:

-	Whether and how the existing NF discovery and selection needs to be enhanced.

It is proposed to enhance NF Registration and Discovery Enhancement for supporting VFL.

2. Proposal

It is proposed to add the following contents to TR 23.700-84.

Start of Change

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues and Use Cases

|  | Key Issues |  | Use cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <use case #x> | <use case #y> |
| #1 |  |  |  |  |
| #2 |  |  |  |  |
| X |  | X |  |  |

Next Change (all new changes)

6.X	Solution #X: NF Registration and Discovery Enhancement for supporting VFL

6.X.1	Description

This is a solution for Key Issue#2: 5GC Support for Vertical Federated Learning.

The solution is based on the existing NWDAF discovery and selection as defined in TS 23.288 [5].

To discover an NWDAF and/or AF who supports VFL:

-	An NWDAF and/or trusted AF include its VFL capability information registered in its NF Profile registered in the NRF. An untrusted AF includes its VFL capability information registered in the NRF via NEF. The VFL capability information may include:

- VFL capability type (e.g. VFL coordinator, VFL active participant, and/or VFL passive participant),

- Time interval supporting VFL indicates when the NWDAF/AF could coordinate/participant in a VFL procedure (e.g. VFL training),

- Supported VFL type indicates which VFL type (e.g. with coordinator or without coordinator) is used to train the ML Model.

-	During the discovery of NWDAF and/or AF supporting VFL, a consumer may include in the discovery request to NRF with required VFL capability information. The NRF returns one or more candidate for instances of NWDAF and/or AF to the consumer.

6.X.2	Registration and discovery of NWDAF and/or AF supporting VFL

Figure 6.x.2-1: Registration and discovery for NWDAF/AF supporting VFL

1-3. The NWDAF/AF registers its NF profile as defined in clause 5.2.7.2.2 of TS 23.502 [3], with the difference it includes the Supported VFL capability information (VFL capability type (e.g. VFL coordinator, VFL active participants, and/or VFL passive participants), Time interval supporting VFL, Supported VFL type) into NRF via Nnrf_NFManagement_NFRegister service. The NRF stores the NF profile of NWDAF/AF and acknowledge NF Registration is accepted via Nnrf_NFManagement_NFRegister response. The Time interval supporting VFL indicates when the NWDAF/AF could coordinate/participant in a VFL procedure (e.g. VFL training). The Supported VFL type indicates which VFL type (e.g. with coordinator or without coordinator) is used to train the ML Model.

NOTE: the untrusted AF could register indirectly via NEF.

4-6. The consumer (e.g. another AF/NWDAF) is to discover the NWDAF/AF supporting VFL via the NRF by invoking the Nnrf_NFDiscovery_Request (including required VFL capability information) service operation. The NRF notifies the consumer with one or more NWDAF/AF instances. The required VFL capability information may include VFL capability type, Time period of Interest and VFL type.

The consumer (e.g. another AF/NWDAF) selects the NWDAF/AF, and triggers the VFL training procedure.

6.X.3	Impacts on services, entities and interfaces

NWDAF:

-	Register "Supported VFL capability information (VFL capability type, Time interval supporting VFL, Supported VFL type)" in NF profile to NRF.

AF:

-	Register "Supported VFL capability information (VFL capability type, Time interval supporting VFL, Supported VFL type)" in NF profile to NRF.

NRF:

-	Support NWDAF/AF supporting VFL registration and discovery procedure.

End of Changes