3GPP TSG-WG SA2 Meeting #161	S2-240xxxx

# February 26 – March 1, 2024, Athens, GR  			       		      	 (revision of S2-2400765r01)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | DRAFT | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction to 5GS support of Roaming value Added Services (RVAS) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_RVAS |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The RVAS WID requires SA2 to define 5GS support of Roaming value Added Services (RVAS) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provides a high level introduction to 5GS support of Roaming value Added Services (RVAS) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RVAS feature is not supported in 5GS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5G DDNMF	5G Direct Discovery Name Management Function

5G LAN	5G Local Area Network

5GS	5G System

5G-AN	5G Access Network

5G-AN PDB	5G Access Network Packet Delay Budget

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G GM	5G Grand Master

5G NSWO	5G Non-Seamless WLAN offload

5G-RG	5G Residential Gateway

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5G VN	5G Virtual Network

5QI	5G QoS Identifier

ADRF	Analytics Data Repository Function

AF	Application Function

AI/ML	Artificial Intelligence/Machine Learning

AKMA	Authentication and Key Management for Applications

AnLF	Analytics Logical Function

AMF	Access and Mobility Management Function

AoI	Area of Interest

AS	Access Stratum

ATSSS	Access Traffic Steering, Switching, Splitting

ATSSS-LL	ATSSS Low-Layer

AUSF	Authentication Server Function

BMCA	Best Master Clock Algorithm

BSF	Binding Support Function

CAG	Closed Access Group

CAPIF	Common API Framework for 3GPP northbound APIs

CH	Credentials Holder

CHF	Charging Function

CN PDB	Core Network Packet Delay Budget

CP	Control Plane

CQRCI	Clock Quality Reporting Control Information

DAPS	Dual Active Protocol Stacks

DCCF	Data Collection Coordination Function

DCS	Default Credentials Server

DetNet	Deterministic Networking

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

DS-TT	Device-side TSN translator

EAC	Early Admission Control

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

EUI	Extended Unique Identifier

FAR	Forwarding Action Rule

FL	Federated Learning

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FN-RG	Fixed Network RG

FQDN	Fully Qualified Domain Name

GBA	Generic Bootstrapping Architecture

GEO	Geostationary Orbit

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network Selection

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HMTC	High-Performance Machine-Type Communications

HR	Home Routed (roaming)

IAB	Integrated access and backhaul

IMEI/TAC	IMEI Type Allocation Code

IPUPS	Inter PLMN UP Security

I-SMF	Intermediate SMF

I-UPF	Intermediate UPF

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LEO	Low Earth Orbit

LMF	Location Management Function

LoA	Level of Automation

LPP	LTE Positioning Protocol

LRF	Location Retrieval Function

L4S	Low Latency, Low Loss and Scalable Throughput

MBS	Multicast/Broadcast Service

MBSF	Multicast/Broadcast Service Function

MBSR	Mobile Base Station Relay

MBSTF	Multicast/Broadcast Service Transport Function

MB-SMF	Multicast/Broadcast Session Management Function

MB-UPF	Multicast/Broadcast User Plane Function

MEO	Medium Earth Orbit

MFAF	Messaging Framework Adaptor Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MINT	Minimization of Service Interruption

ML	Machine Learning

MPQUIC	Multi-Path QUIC

MPS	Multimedia Priority Service

MPTCP	Multi-Path TCP Protocol

MTLF	Model Training Logical Function

N3IWF	Non-3GPP InterWorking Function

N3QAI		Non-3GPP QoS Assistance Information

N5CW	Non-5G-Capable over WLAN

NAI	Network Access Identifier

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NID	Network identifier

NPN	Non-Public Network

NR	New Radio

NRF	Network Repository Function

NS-AoS	Network Slice Area of Service

NSAC	Network Slice Admission Control

NSACF	Network Slice Admission Control Function

NSAG	Network Slice AS Group

NSI ID	Network Slice Instance Identifier

NSSAA	Network Slice-Specific Authentication and Authorization

NSSAAF	Network Slice-specific and SNPN Authentication and Authorization Function

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NSSRG	Network Slice Simultaneous Registration Group

NSWO	Non-Seamless WLAN offload

NSWOF	Non-Seamless WLAN offload Function

NW-TT	Network-side TSN translator

NWDAF	Network Data Analytics Function

ONN	Onboarding Network

ON-SNPN	Onboarding Standalone Non-Public Network

PCF	Policy Control Function

PDB	Packet Delay Budget

PDR	Packet Detection Rule

PDU	Protocol Data Unit

PDV	Packet Delay Variation

PEGC	PIN Element with Gateway Capability

PEI	Permanent Equipment Identifier

PEMC	PIN Element with Management Capability

PER	Packet Error Rate

PFD	Packet Flow Description

PIN	Personal IoT Network

PINE	PIN Element

PLR	Packet Loss Rate

PNI-NPN	Public Network Integrated Non-Public Network

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

PSDB	PDU Set Delay Budget

PSER	PDU Set Error Rate

PSIHI	PDU Set Integrated Handling Information

PTP	Precision Time Protocol

PVS	Provisioning Server

QFI	QoS Flow Identifier

QoE	Quality of Experience

RACS	Radio Capabilities Signalling optimisation

(R)AN	(Radio) Access Network

RG	Residential Gateway

RIM	Remote Interference Management

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSN	Redundancy Sequence Number

RTT	Round Trip Time

RVAS	Roaming Value Added Services

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SCP	Service Communication Proxy

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SF	Service Function

SFC	Service Function Chain

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SO-SNPN	Subscription Owner Standalone Non-Public Network

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

SV	Software Version

TA	Tracking Area

TAI	Tracking Area Identity

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSC	Time Sensitive Communication

TSCAI	TSC Assistance Information

TSCTSF	Time Sensitive Communication and Time Synchronization Function

TSN	Time Sensitive Networking

TSN GM	TSN Grand Master

TSP	Traffic Steering Policy

TSS	Timing Synchronization Status

TT	TSN Translator

TWIF	Trusted WLAN Interworking Function

UAS NF	Uncrewed Aerial System Network Function

UCMF	UE radio Capability Management Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

W-5GAN	Wireline 5G Access Network

W-5GBAN	Wireline BBF Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

NEXT CHANGE (2) all text is new

### 5.X	5GS support of Roaming Value Added Services (RVAS)

Editor's note:	It is FFS if the desdscription in this sub-clause is needed.

#### 5.X.1	Overview

Roaming Value Added Services (RVAS) form part of the roaming ecosystem and are defined in TS 22.261 [2]. The RVAS provider acting on behalf of the HPLMN may be a Roaming Intermediary trusted by the HPLMN or any target PLMN trusted by the HPLMN.

Following features are used for 5GS support of Roaming Value Added Services (RVAS):

- The "Welcome SMS" feature supports using an AF to send a dedicated SMS to a roaming subscriber’s UE when this UE is registered in a new roamed-to network for the first time. The 5GS support for this feature is defined in clause 4.15.3.1 of TS 23.502 [3].

- The "Steering of Roaming (SoR) during the registration procedure" feature makes the home operator able to steer a user to a certain network during the registration procedure when the user tries to register to a new (non-preferred) VPLMN. The 5GS support for this feature is defined in TS 23.122 [17].

- Subscription-based routing to a particular core network (e.g. in a different country). The 5GS support for this feature is defined in clause 5.X.2.

#### 5.X.2	Subscription-based routing to a particular core network

The "Subscription-based routing to a particular core network" feature forwards traffic related with a UE to a target (partner) PLMN that may be neither the serving PLMN of the UE nor the HPLMN of the UE. This feature may also apply when the UE is served by its HPLMN.

This feature of "Subscription-based routing to a particular core network" may apply to all signaling and User Plane traffic related with a UE or only to the SM signaling and User Plane traffic related with PDU sessions established by the UE to some DNN(s) and/or S-NSSAI(s) and/or related to a SUPI range and/or subscription. Which Subscription-based routing to a particular core network applies to which UE and using which target network is determined by the HPLMN of the UE based on operator policies.

NOTE:	This feature assumes some SLA between the HPLMN of the UE and the target PLMN.

When "Subscription-based routing to a particular core network" feature is based on all signaling and User Plane traffic related to a UE to be served by a target PLMN, the HPLMN, or the provider acting on HPLMN behalf, will make use of the corresponding Routing Indicator and/or SUPI range and/or UDM Group ID  to route the matching traffic to the  target network.

When "Subscription-based routing to a particular core network" feature is based on SM signaling and User Plane traffic related to a UE to be served by a target PLMN, the HPLMN, or the provider acting of behalf of the HPLMN, will make use of the corresponding DNN(s) and/or S-NSSAI(s) and/or SUPI range and/or subscription to route the matching traffic to the  target network.

END OF CHANGES