SA WG2 Meeting #161	S2-2402868

February 26 – March 1, 2024, Athens, Greece

Source:	InterDigital Inc.

Title:	KI#3, New Solution, NTZ enforcement in 5GC

Document for:	Approval

Agenda Item: 	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a solution on NTZ enforcement in 5GC.

# 1	Discussion

This contribution proposes a solution for NTZ enforcement in 5GC addressing key issue #3.

# 2	Proposal

It is proposed to adopt the following new solution in TR 23.700-59.

* * * * Start of Changes (All Next Text) * * * *

## 6.X	Solution #X: NTZ Enforcement in 5GC

### 6.X.1	Key Issue mapping

This solution addresses KI#3, “Support of No Transmit Zones”.

### 6.X.2	Description

In this solution, the NTZ enforcement request is initiated from the USS/UTM and sent to the UAS-NF in 5GC. The request includes the designated NTZ information (areas, spectrum, etc.) and target UAV identifiers (or all UAVs). The UAS NF maps the NTZ areas to network areas and locates the serving AMFs of the affected UAVs, and forwards the NTZ enforcement request to the serving AMFs.

The serving AMF may perform the following actions:

- Use the location service to determine whether the UAV is in or out of the NTZs.

- For those in the NTZs and in Connected mode, inform the SMF to deactivate the PDU sessions of the UAV (C2 communication may be exempted), and inform the RANs to not schedule UL transmissionsc for those UAVs.

- For those in the NTZs and in Idle mode, or not in the NTZs, inform the UAVs of the NTZ restrictions via NAS signalling.

### 6.X.3	Procedures

Figure 6.X.3-1: NTZ enforcement in 5GC

- The UAS NF receives the NTZ enforcement request from the USS/UTM. The request include the NTZ information and affected UAV Identifiers (or all UAVs). The NTZ information includes one or multiple NTZ areas, frequency bands that UAVs are not allowed to transmit, period of time that NTZs are valid, etc.

- The UAS NF uses the UAV tracking mechanism to determine the location of the affected UAVs, whether they are in NTZs, and locate their serving AMFs.

- The UAS NF forwards the NTZ enforcement request to the serving AMFs.

- The AMF may further use location service to determine the location of the affected UAVs.

If the affected UAV is in Connected mode,

5a. The AMF requests the SMF to deactivate the PDU Sessions of the UAV.

6a. The AMF also notifies the UAV of the NTZ enforcement.

7a. The AMF may also inform the serving RAN to enforce the NTZ for the UAV, e.g. not scheduling UL transmission for the UAV.

8a. The RAN may release the RRC connection of the UAV.

If the affected UAV is in Idle mode,

5b. The AMF notifies the UAV of the NTZ restriction. The UAVs may proactively comply with the NTZ restriction.

6b/7b. If non-conformant UAVs initiate Service Request, it may be rejected with the reason of NTZ control.

### 6.X.4	Impacts on services, entities and interfaces

UAS NF:

- Supports handling of NTZ enforcement request, including forwarding the request to other NFs (e.g. AMF).

AMF

- Supports notification of NTZ enforcement/restriction to the affected UAVs using NAS signalling.

- Supports requesting the SMF to enforce the NTZ (e.g., deactivate PDU Sessions)

- Supports requesting the RAN to enforce the NTZ (e.g., not scheduling UL transmission)

- Reject service request for the reason of NTZ control.

RAN

- Supports NTZ enforcement (e.g., not scheduling UL transmission, release RRC)

* * * * End of Changes * * * *