SA WG2 Meeting S2#161	S2-2403010

Athens, Greece, 26 Feburary - 01 March, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#3: New solution on traffic routing between local and central part of DN

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract of the contribution: This contribution proposes a new solution to KI#3.

# Introduction

This contribution proposes a solution to KI#3 on TR 23.700-49 including architectural aspects.

# 2	Proposal

It is proposed to add following text to TR 23.700-49.

# * * * * First change* * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  | 3 |  |  |
| 6.X: solution on traffic routing between local and central part of DN |  | X |  |  |

# * * * * Next change* * * *

## 6.X	Solution #: Solution to traffic routing between local and central part of DN

### 6.X.1	Description

#### 6.X.1.1	Overall architecture of the solution

The architecture provides connectivity between the local and central parts of the DN (“L-DN” and “C-DN” as depicted in Figure 6.X.1-1), so that traffic can be exchanged between different parts of the DN e.g. between a L-DN part and the C-DN part. This allows EASs in different parts of a DN to communicate with each other.

Figure 6.X.1-1: Connectivity between the local and central parts of the DN.

NOTE: 	In the above figure, even though a single L-PSA UPF interfacing a DN part has been represented, the L-PSA terminating the PDU Session may be different from the L-PSA allowing to interface the L-DN part with other parts of the DN.

This solution proposes to establish below two data paths:

1	a data path per PDU session between the UE and a L-PSA UPF interfacing the L-DN part.

2.	a tunnel between the two parts of the DN (i.e., a L-DN part and a C-DN part in Figure 6.X.1-1).

The tunnel between L-PSA(s) interfacing the two parts of the DN is independent from the data path established per PDU session between the UE and the L-PSA UPF. The data path per PDU session between the UE and the L-PSA UPF interfacing the L-DN part is established as part of the PDU session establishment or update procedures as defined in TS 23.502 [3]. The tunnel between the L-PSA UPF(s) interfacing two parts of the DN can be shared by all PDU sessions whose traffic requires joint handling by the L-DN and the C-DN part. The solution focuses on the establishment and usage of the tunnels between the L-PSA(s) interfacing two parts of the DN.

The Application Function (AF) may request 5GC to establish the tunnel between the two parts of the DN based on the extended Data Network Access Identify (DNAI) related information in EAS Deployment Information (EDI).

#### 6.X.1.2	AF control of the connectivity between different DN parts

The solution updates EDI in TS 23.548 [5] Table 6.2.3.4-1 with following information:

- an indicator whether the DNAI is to be considered as local or central;

- an indicator whether connectivity to central DNAI(s) is needed;

For the AF, to create, update, or delete the information described above, the existing procedure defined in TS 23.548 [5] Clause 6.2.3.4.2 “EAS Deployment Information Management in the AF Procedure” apply. SMF may receive the information as part of the EDI from NEF via the existing procedure defined in TS 23.548 [5] Clause 6.2.3.4.2 “EAS Deployment Information Management in the SMF”.

### 6.X.2	Procedures

Based on the AF control of the connectivity between different DN parts as defined in clause 6.X.1.2, the 5GS establishes the tunnel between the different parts of the DN as described in this clause.

When SMF receives DNAI information requiring setting up connectivity between different DN parts, the SMF:

1.	selects a L-PSA UPF based on the received DNAI of the L-DN and a Central PSA UPF (C-PSA UPF) based on the received DNAI of the C-DN.

2.	via existing N4 session signalling (providing PDRs and FARs) to the PSA UPF(s) to generate an Up Link Terminal Endpoint Identify (UL TEID) in C-PSA UPF and a Down Link Terminal Endpoint Identify (DL TEID) in L-PSA UPF, so that a tunnel can be established between L-DN and C-DN.

3.	Configures the L-PSA UPF with PDR(s) and FAR(s) as defined below.

Figure 6.X.2-1 depicts the call flow of the establishing a tunnel to provide connectivity between the local and central parts of DN.

Figure 6.X.2-1: Procedure of establishing a tunnel between local and central parts of DN

The steps in the call flow are as follows:

1.	AF provides 5GC with information to provide connectivity between the local and central part of DN, for example, the AF invokes the Nnef_EASDeployment_Create/Update/Delete service operation as the step 1 defined in clause 6.2.3.4.2 of TS 23.548 [5].

2.	SMF is triggered to retrieve AF provided information. The trigger can be a NEF notification (related with EDI), or a 5GC event (e.g., PDU session establishment/modification, EAS (re-)discovery). SMF retrieves EDI, which includes information as defined in the above Clause 6.X.1.2. This information is conveyed from NEF to SMF via EDI based on EAS Deployment Information Management in the SMF defined in clause 6.2.3.4.3 of TS 23.548 [5]. The SMF determines to establish the tunnel based on this information received from the AF.

Steps 3 to 11 may take place for each pair of DNAIs (local/central) where the SMF is to establish a tunnel.

3.	The SMF selects two UPFs as the L-PSA UPF and the C-PSA UPF, based on the two retrieved DNAIs.

4.	The SMF initiates a N4 Session Establishment procedure with the selected C-PSA UPF. The N4 message contains PDR-1 and FAR-1 defined in the below Table 6.X.2-1 for UL traffic. C-PSA UPF performs the session configuration and UL TEID selection. During the session configuration, UL TEID (i.e., UL TEID of the tunnel) is used for L-PSA UPF to forward L-DN’s UL traffic to C-DN via C-PSA UPF.

5.	The C-PSA UPF responses SMF by sending an N4 Session Establishment Response, including the UL TEID of the C-DN tunnel which is a part of CN tunnel information.

6.	SMF initiates a N4 Session establishment procedure with the selected L-PSA UPF, to provide UL TEID of the tunnel to L-PSA UPF, as well as PDR-2 and FAR-2 defined in Table 6.X.2-1 for UL traffic. L-PSA UPF performs the tunnel configuration and DL TEID of C-DN tunnel/LL-DN tunnel selection. During the tunnel configuration, DL TEID of the tunnel is selected for L-PSA UPF to forward UL traffic to C-DN via C-PSA UPF.

7.	L-PSA UPF provides SMF with the DL TEID of the C-DN tunnel which is a part of CN tunnel information.

8.	SMF initiates a N4 Session Update procedure with the selected L-PSA UPF and provide PDR-3 and FAR-3 defined in Table 6.X.2-1 for DL traffic.

9.	L-PSA UPF responses SMF by sending an N4 Session Update Response.

10.	SMF initiates a N4 Session Update procedure with the selected C-PSA UPF and provides DL TEID of C-DN tunnel to C-PSA UPF, as well as PDR-4 and FAR-4 defined in Table 6.X.2-1 for DL traffic.

11.	C-PSA UPF responses SMF by sending an N4 Session Update Response.

Table 6.X.2-1 PDRs and FARs for L-PSA UPF and C-PSA UPF configuration.

| Rule | Description |
| --- | --- |
| PDR-1 | Source Interface = access (N9) Packet Filter Set (optional): IP SA = L-EAS IP (range), IP DA = C-EAS IP (range) GTP-U outer header removal FAR ID = FAR-1’s ID |
| FAR-1 | Rule ID Action = forward Destination interface = N6 |
| PDR-2 | Source Interface = N6 (optional) Packet Filter Set: IP SA = L-EAS IP (range), IP DA = C-EAS IP (range) FAR ID = FAR-2’s ID |
| FAR-2 | Rule ID Action = forward Destination interface = N9 GTP-U outer header adding: GTP-u header that the SMF has received from the C-PSA |
| PDR-3 | Source Interface = access N9 (optional) Packet Filter incoming GTP-U tunnel GTP-U outer header removal FAR ID = FAR-3’s ID |
| FAR-3 | Rule ID Action = forward Destination interface = N6 |
| PDR-4 | Source Interface = N6 (optional) Packet Filter Set: IP SA = C-EAS IP (range), IP DA = L-EAS IP (range) FAR ID = FAR-4’s ID |
| FAR-4 | Rule ID 004 Action = forward Destination interface = access (N9) GTP-U outer header adding: GTP-U header that the SMF has received from the L-PSA |

Table 6.X.2-1: PDRs and FARs for L-PSA UPF and C-PSA UPF

### 6.X.3	Impacts on existing nodes and functionality

Impacted nodes and functionality are listed as follows:

-	AF:

- 	provides information to 5GC requesting application data forwarding between two parts of the DN (L-DN and a C-DN) via information described in clause 6.X.1.2.

-	SMF:

-	select PSA UPFs (L-PSA UPF and C-PSA UPF), so that two parts of the DN can be anchored and a tunnel can be established in-between.

-	configure two PSA UPFs (L-PSA UPF and C-PSA UPF) to send/receive traffic at L-DN or C-DN.

# * * * * End of changes * * * *