SA WG2 Meeting #161	S2-2403021

February 26 – March 1, 2024, Athens, Greece

Source:	Xiaomi

Title:	update solution#17 for downloading the Avatar id and Avatar metadata

Document for:	Approval

Agenda Item: 	19.2

Work Item / Release:	FS_NG_RTC_Ph2/ Rel-19

Abstract of the contribution: This paper proposes to update solution#17 for downloading the Avatar id and Avatar metadata to resolve the ENs. .

1	Discussion

This solution#17 has following Editor’s note:

Editor's note: How the UE-A is configured or is aware of the Avatar-id is for FFS.

In this paper, it proposes a new term Avatar Object which is an Avatar representation for user that including Avatar id and the corresponding Avatar metadata.

In order to resolve the EN, it proposes to download all the Avatar Objects [Avatar-ids and the corresponding Avatar metadatas] from XR Application Server to UE during XR media rendering negotiation with the XR Application Server. During this negotiation, user selects one preferred Avatar Object, at this moment, the Avatar id is determined.

.

2	Proposal

It is proposed to include the following changes in TR 23.700-77 V0.3.0.

* * * * Start of Changes * * * *

. 6.17	Solution #17: Solution for support of IMS avatar communication using data channel

6.17.1	Description

Figure 6.17.1-1: Architecture to support IMS avatar communication

To support of IMS avatar communication, the architecture is enhanced as follows:

Digital Asset Container (DAC):

-	The Digital Asset Container is used for storing and retrieving the avatar model from 5GC. DAC can be internal to the PLMN, e.g. a new network function, a part of network function (HSS, UDM, IMS AS), a XR application server or a web server within the IMS and/or 5GC; or DAC can be external to the PLMN, e.g. a webserver of a cloud provider,

XR Application Server (XR AS):

-	The XR Application Server is responsible for XR service control related to XR communication, including XR session media control, XR media capability negotiation with the UE.

Editor's note:	The definition of the reference point and the interaction between DAC and XR Application Server is for FFS.

Three new information elements are defined as follows:

Avatar metadata:

-	Avatar metadata is the 2D/3D representational content/model of an avatar stored in a file, which can be accessed via a URL. Avatar metadata are stored in the DAC. The avatar metadata, or a reference of the avatar metadata (e.g. identifier), is associated per subscriber and/or per subscriber's user identifier. Avatar metadata needs to correspond to face, full body, or a portion of the body. Additional metadata can be associated with the avatar metadata, e.g. format, resolution, access/usage rights & time, date of creation, licensing, etc.

Avatar-id:

-	Avatar metadata are accessible via an Avatar-id and can be fetched from the DAC using the Avatar-id. It can be an integer, string, UUID, or a URI (URN or URL).

Avatar media:

-	Avatar media is a media stream (e.g. video) that mixes gesture/movement using avatar metadata. It can be a combination of (real-time) face/gesture movement and the avatar metadata. Avatar media shall be encrypted using the standard secured RTP or data channel mechanism using DTLS.

Avatar Object:

- Avatar objects is an Avatar representation for user that including Avatar id and the corresponding Avatar metadata.

Editor's note:	The avatar-related terminology will be updated once agreed upon.

Editor's note:	Further details regarding the three information elements above are FFS.

6.17.2	Procedures

Supporting IMS avatar communication including providing avatar or a reference to one or several avatars in the IMS signalling, downloading and negotiating the avatar with B-party. In the proposed solutions of this document, avatar can be transferred via data channel, and rendering of avatar media can be performed by network, UE-A, or UE-B, which are described in clauses 6.17.2.1, 6.17.2.2, and 6.17.2.3, respectively.

6.17.2.1	Network centric IMS avatar call flow using data channel

Based on Figure AC.9.3.2-1 in clause AC.9.3.2 of TS 23.228 [5], Figure 6.17.2.1-1 depicts a call flow to establish a network centric IMS avatar call with rendering in the network. UE-A avatar media are transmitted to UE-B via MF/MRF.

Figure 6.17.2.1-1: Establishment of network centric IMS avatar call using data channel

The main steps in the call flow are as follows:

1.	The UE-A initiates an IMS session and establishes audio and video session connections with the UE-B. The bootstrap data channel(s) are established at the same time for both the UE-A and UE-B.

2.	The UE-A decides to request network media rendering of an avatar intended to use for the call based on its status such as power, signal, computing power, internal storage, etc. The UE-A selects the Avatar-id of the avatar.

3.	The UE-A performs the XR media rendering negotiation with the XR Application Server. The negotiation includes usage of the Avatar-id.

If the Avatar id and Avatar metadata is not stored in UE, during the negotiation, all the Avatar Objects are downloaded to UE, and the avatar id is determined when user selects the preferred Avatar Object.

4.	If the negotiation result is successful in step 3, the UE-A initiates new P2A application data channels, which are used for XR data transmission between the UE-A and the network. During the P2A application data channel establishment procedure, the DCSF will instruct the MF/MRF via IMS AS how to establish the data channel and corresponding media processing specification.

5(Optional). IMS AS initiates a media re-negotiation request with UE-A by exchanging the Avatar-id via the application DC, to connect UE-A's audio/video media stream to MF/MRF. UE-A provides to the XR Application Server via the application DC the Avatar-id of the avatar intended to use for the call.

6.	IMS AS initiates a media re-negotiation request with UE-B, to connect UE-B's audio/video media stream to MF/MRF. The Avatar-id is exchanged with UE-B to indicate about the avatar session during the signalling. UE-B has the option to reject the avatar alone or terminate the session based on Avatar-id.

Editor's note:	The SDP content is to be decided by SA4.

7.	The XR Application Server retrieves the avatar metadata using the Avatar-id from DAC.

Editor's note:	How the XR Application Server is triggered to download the Avatar metadata is FFS.

8.	The DAC responds to the XR Application Server with the avatar metadata.

9.	The XR Application Server starts controlling the XR media rendering.

10.	The XR Application Server sends the avatar metadata to MF/MRF and requests rendering of the avatar by MF/MRF.

11.	The UE-A sends information about UE-A to MF/MRF.

Editor's note:	What information about UE-A to be sent is FFS.

12.	The MF/MRF receives the information of UE-A from the UE-A and replaces the face/body with the selected avatar metadata, e.g. via face detection and/or recognition mechanisms.

13.	The rendered avatar media is sent as regular video media to UE-B.

Editor's note:	If the audio should be part of the avatar media is FFS.

14.	The rendered avatar media is sent back to the UE-A as feedback (same content as the one sent to the UE-B in step 13), e.g. to display a thumbnail view of the avatar to the UE-A in the IMS session.

6.17.2.2	UE-A centric IMS avatar call flow using data channel

Based on Figure AC.9.3.1-1 in clause AC.9.3.1 of TS 23.228 [5], Figure 6.17.2.2-1 depicts a call flow procedure to establish a UE centric IMS avatar call with rendering in the UE-A.

Figure 6.17.2.2-1: Establishment of UE-A centric IMS avatar call using data channel

The main steps in the call flow are as follows:

1.	The UE-A initiates an IMS session and establishes audio and video session connections with the UE-B. The bootstrap data channel(s) are established at the same time for both the UE-A and UE-B.

2.	The UE-A decides to request UE-A media rendering of an avatar intended to use for the call based on its status such as power, signal, computing power, internal storage, etc. The UE-A selects the Avatar-id of the avatar.

3.	The UE-A performs the XR media rendering negotiation with the XR Application Server. The negotiation includes usage of the Avatar-id.

If the Avatar id and Avatar metadata is not stored in UE, during the negotiation, all the Avatar Objects are downloaded to UE, and the avatar id is determined when user selects the preferred Avatar Object.

4.	If the negotiation result is successful in step 3, the UE-A initiates new P2A application data channel, which are used for XR data transmission between the UE-A and the network. During the P2A application data channel establishment procedure, the DCSF will instruct the MF/MRF via IMS AS how to establish the data channel and corresponding media processing specification.

5.	IMS AS initiates a media re-negotiation request with UE-B, to connect UE-B's audio/video media stream to MF/MRF. The Avatar-id is exchanged with UE-B to indicate about the avatar session during the signalling. UE-B has the option to reject the avatar alone or terminate the session based on Avatar-id.

Editor's note:	The SDP content is to be decided by SA4.

6-9.	The XR Application Server requests DAC to retrieve avatar metadata by providing UE-identity and Avatar-id via DC, and send the avatar metadata to the UE-A. If the UE-A has the association avatar metadata available in cache, step 6-9 shall be skipped.

Editor's note:	How the XR Application Server is triggered to download the Avatar metadata is FFS.

10.	The UE-A locally mixes the avatar metadata together with the audio/video media received from local sensors (e.g. camera) to animate the avatar (the rendered avatar audio/video media).

11.	The UE-A sends the rendered avatar audio/video media as regular video media to the UE-B.

6.17.2.3	UE-B centric IMS avatar call flow using data channel

Based on Figure AC.9.3.1-1 in clause AC.9.3.1 of TS 23.228 [5], Figure 6.17.2.3-1 depicts a call flow procedure to establish a UE centric IMS avatar call with rendering in the UE-B.

Figure 6.17.2.3-1: Establishment of UE-B centric IMS avatar call using data channel

The main steps in the call flow are as follows:

1.	The UE-A initiates an IMS session and establishes audio and video session connections with the UE-B. The bootstrap data channel(s) are established at the same time for both the UE-A and UE-B.

2.	The UE-A decides to request UE-B media rendering of an avatar intended to use for the call based on its status such as power, signal, computing power, internal storage, etc. The UE-A selects the Avatar-id of the avatar.

3.	The UE-A performs the XR media rendering negotiation with the XR Application Server and the UE-B. The Avatar-id is exchanged with UE-B to indicate about the avatar session during the signalling.

If the Avatar id and Avatar metadata is not stored in UE, during the negotiation, all the Avatar Objects are downloaded to UE, and the avatar id is determined when user selects the preferred Avatar Object.

4.	If the negotiation result is successful in step 3, the UE-B initiates new P2A application data channels, which are used for XR data transmission between the UE-A and UE-B. During the P2A application data channel establishment procedure, the DCSF will instruct the MF/MRF via IMS AS how to establish the data channel and corresponding media processing specification.

5.	IMS AS initiates a media re-negotiation request with UE-B, to connect UE-B's audio/video media stream to MF/MRF. UE-B has the option to reject the avatar alone or terminate the session based on Avatar-id.

NOTE:	The SDP content is to be decided by SA4.

6-9.	The XR Application Server requests DAC to retrieve avatar metadata by providing UE-identity and Avatar-id via DC, and send the avatar metadata to the UE-B. If the UE-B has the association avatar metadata available in cache, step 6-9 shall be skipped.

Editor's note:	How the XR Application Server is triggered to download the Avatar metadata is FFS.

10.	The UE-A sends information about UE-A to UE-B.

Editor's note:	What information about UE-A to be sent is FFS.

11.	The UE-B locally mixes the avatar metadata together with the received information of UE-A to animate the avatar (the rendered avatar audio/video media).

6.17.3	Impacts on existing nodes and functionality

Impacted nodes and functionality are listed as follows:

-	DAC: stores avatar model, names, and further data, based on avatar ID.

-	XR Application Server: retrieves avatar model from DAC based on avatar ID and user identity, triggers MF/MRF rendering/transcoding.

-	IMS AS: supports avatar call, provide avatar ID to XR Application Server.

-	MF/MRF: enables avatar rendering/transcoding.

-	UE (UE-A centric or UE-B centric): selects and sends avatar ID, downloads avatar model via IMS data channel, negotiates avatar codec (optional) with other parties and networks via SIP/SDP, provides rendering capability (optional).

* * * * End of Changes * * * *