3GPP TSG-WG SA2 Meeting #161	S2-2403098

# Athens, Greece, February 26 - March 1, 2024

Source: 	China Mobile

Title: 	New Solution: Edge computing handling by local SMF

Document for: 	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_EDGE_Ph3 / Rel-19

Abstract of the contribution: Propose a solution on KI#1: Enhancements for EAS (re)discovery and UPF (re)selection with reducing impact on central 5GC NFs.

1	Discussion

This paper is to propose a new solution on KI#1.

In this solution, UL CL/BP controlled by local SMF (L-SMF) is used to offload edge computing related traffic to local DN. The central SMF is not involved with EDI management, DNS message handling rules configuration or update, and UL CL/BP or L-PSA insertion, and all the above is handled by L-SMF.

2	Proposal

It is proposed to include the below changes into TR 23.700-49 v0.1.0.

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | ... |
| Solution x: Edge computing handling by local SMF | X |  |  |  |
| … |  |  |  |  |

NEXT CHANGE (all the text is new)

6.x	Solution #X: Edge computing handling by local SMF

6.x.1	Key Issue mapping

This solution is related to the KI#1.

6.x.2	Functional Description

The architecture of this solution is described as Figure 6.x.2-1.

AN

Neasdf

Nsmf

Nsmf

Nnef

Nudm

Npcf

Namf

UDM

EASDF

DN

L-DN

UPF (L-PSA)

PSA UPF

(C-PSA)

NEF

UPF (ULCL/BP)

PCF

L-SMF

SMF

AMF

UE

Local Network

Figure 6.x.2-1: Network Architecture for the L-SMF

In this solution, UL CL/BP controlled by local SMF (L-SMF) can offload edge computing related traffic to local DN. The L-SMF is responsible for EDI management, DNS message handling rules configuration or update, and UL CL/BP or L-PSA insertion, and central SMF is not involved for the above information.

During UE Registration procedure and at establishment of the AM Policy Association, AMF request the the conditions to check whether to contact the PCF at PDU Session establishment for DNN for L-SMF selection. And if the PCF supports the DNN for L-SMF selection, the PCF provides the AMF with triggers for DNN for L-SMF selection.

During PDU Session Establishment procedure and as a result of the DNN of L-SMF selection, AMF interact with the PCF and PCF provides the selected DNN that is applicable for the L-SMF selection at the PDU Session Establishment.

Therefore, Firstly, the AMF performs PDU session establishment with the central SMF, and complete the step 3-11 of clause 4.3.2.2.1 in TS 23.502. In Step 11, the central SMF response with selected PCF ID to AMF.

Then the AMF interacts with the PCF to get the DNN of L-SMF selection and performs PDU session establishment with the L-SMF, and select the same PCF to retrieve the SM policy.

The tunnels between AN, ULCL/BP UPF, central PSA UPF are setup by the following: UL N3 tunnel is setup by AMF provides UL CL/BP UPF CN Tunnel Info received from L-SMF to (R)AN, DL N3 tunnel is set up by AMF provides AN Tunnel Info received from (R)AN to UL CL/BP, UL N9 tunnel between UL CL/BP UPF and central PSA UPF is setup by AMF provides PSA CN Tunnel Info received from SMF to L-SMF, DL N9 tunnel between UL CL/BP UPF and central PSA UPF is setup by AMF provides UL CL/BP UPF CN Tunnel Info received from L-SMF to SMF.

6.x.3	Procedures

ULCL/BP

UE

(R)AN

AMF

SMF

UPF

L-SMF

PCF

UDM

2. Steps 1-11 of PDU Session Establishment procedure, clause 4.3.2.2.1 of TS 23.502

4. L-SMF selection

5. Nsmf_PDUSession_CreateSMContext Request

6. SM Policy Association Establishment

7. UPF selection

1. Registration procedure, clause 4.2.2.2.2 of TS 23.502

8. Steps 9-14 of PDU Session Establishment procedure, clause 4.3.2.2.1 of TS 23.502

9. Steps 15-17 of PDU Session Establishment procedure, clause 4.3.2.2.1 of TS 23.502

10. Steps 15-17 of PDU Session Establishment procedure, clause 4.3.2.2.1 of TS 23.502

3. Npcf_SMPolicyControl

Figure 6.x.3-1: Procedure of edge computing handling by local SMF

1.	The registration procedure is performed as described in clause 4.2.2.2.2 of TS 23.502 [3], with the following change:

- In step 16, the PCF provides the AMF with triggers for DNN for L-SMF selection.

2.	Steps 1-11 of the UE-requested PDU Session Establishment procedure is performed as described in clause 4.3.2.2.1 of TS 23.502 [3], with the following change:

-In Step 11, the central SMF response with selected PCF ID or PCF address to AMF.

3.	If AMF determines the DNN of the PDU Session Establishment Request is indicated for L-SMF selection, the AMF obtains the DNN of L-SMF selectionfrom PCF.

4.	AMF performs L-SMF selection based on the DNN obtained from PCF.

5.	AMF sends the PCF address or PCF ID to the L-SMF by invoking Nsmf_PDUSession_CreateSMContext Request service.

6.	L-SMF obtains offloading policy from PCF via SM Policy Association Establish procedure as described in clause 4.16.4 of TS 23.502 [3].

7.	L-SMF selects UL CL/BP and optionally L-PSA as described in clause 6.3.3 of TS 23.501 [2].

8.	Steps 9-14 of the UE-requested PDU Session Establishment procedure is performed as described in clause 4.3.2.2.1 of TS 23.502 [3], with the following changes:

- SMF is replaced by L-SMF.

- CN Tunnel Info is UL CL/BP CN Tunnel Info.

In this step, the (R)AN obtains the CN Tunnel Info of the UL CL/BP UPF.

9.	Steps 15-17 of the UE-requested PDU Session Establishment procedure is performed as described in clause 4.3.2.2.1 of TS 23.502 [3], with the following change:

- AN Tunnel Info is replaced by UL CL/BP CN Tunnel Info.

In this step, the central PSA UPF obtains the UL CL/BP CN Tunnel Info.

10. Steps 15-17 of the UE-requested PDU Session Establishment procedure is performed as described in clause 4.3.2.2.1 of TS 23.502 [3], with the following changes:

- SMF is replaced by L-SMF.

- In step 15, AMF also sends PSA CN Tunnel Info to the L-SMF.

- In step 16a, L-SMF also provides PSA CN Tunnel Info to the UL CL/BP.

In this step, the UL CL/BP obtains the AN Tunnel Info and PSA CN Tunnel Info.

Editor’s NOTE: how to support the local DNS handling in this solution is FFS.

6.x.4	Impacts on existing services, entities and interfaces

AMF:

-	L-SMF selection indicated by DNN of the PDU Session Establishment Request.

-	Providing the PCF address from Central SMF to L-SMF

-	Providing the PSA CN Tunnel Info to L-SMF.

-	Providing the UL CL/BP CN Tunnel Info to SMF.

SMF: Providing PCF address to AMF.

PCF: Providing Offloading policy to L-SMF.

End of CHANGE