SA WG2 Meeting #162  	  S2-2403991

15 April-19 April 2024, Changsha,China

Source:	ZTE

Title:	TR 23.700-84: EN removal related to the VFL definition

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

# 1. Discussion

This pCR propose to remove the EN about feature space of VFL client. Most companies agree that the VFL clients shall have different feature space. Besides, completely different feature space of VFL clients could maximize the generalization performance of the local model.

# 2. Proposal

---------- Start of change ----------

3	Definitions of terms and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

-	Horizontal Federated Learning (HFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different FL clients for local model training have the same feature space for different samples (e.g. UE IDs).

-  Vertical Federated Learning (VFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs).

-  Label: A label is the property of interest that is to be learned in supervised machine learning.

Editor's note: the label definition is to be reviewed and extended if needed.

-  VFL Active Participant: An NF with labels for a VFL training task that may have related inputs data.

Editor's note: whether and how VFL server is different from VFL Active Participant is FFS. If they are the same, one term needs to be selected.

-	VFL Passive Participant: An NF with the required inputs data without the required labels for a VFL training task. There can be multiple passive participants in VFL.

Editor's note: It is FFS whether and how VFL client is different from VFL passive Participant. If they are the same, one term needs to be selected.

Editor's note: whether there is a VFL coordinator is FFS.

Editor's note: the terminology definition will be updated.

---------- End of change ----------