SA WG2 Meeting #162	S2-2404235

# 15 – 19 April 2024, Changsha, China	                (revision of S2-2402534)

Source: 	NTT DOCOMO

Title: 	KI#3, Sol#12: Update for clarification of inputs and outputs

Document for: 	Approval

Agenda Item: 	19.4 (FS_EnergySys)

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This paper provides clarification and corrections on the inputs and output of NWDAF energy analytics in Sol#12.

# 1	Discussion

The following inconsistencies and overlaps in the input information and output analytics proposed by Solution #12 are resolved:

- “Time Window” in input data is replaced by “Time Stamp” similar to other analytics where a time stamp is associated to collected input data.

- Input “identifier” is removed, because it is not an input data to be collected from OAM, instead, it is a input sent to OAM in NWDAF requests to collect data.

- Energy Efficiency at 5GC level is added.

- “NF energy usage per slice” is removed because it is already addressed in second row (clause 6.7.3.3)

- Output “Application ID” is removed as there is no corresponding input the purpose of this output is not clear specially in the granularity levels such as NF.

- Output “NF Type” is removed as the consumer either knows or can discover using the ID.

- Output “maximum energy credit” is removed because according to the given description it is equal to the “energy consumption at required granularities” in the PDU session/QoS flow/… granularity level.

# 2 Proposal

It is proposed to update TR 23.700-66 according to the following text.

********** First Change **********

#### ss6.12.2.2	Input Data of Energy Related Analytics

The NWDAF may collect the service data from OAM, AF and/or the 5GC network function in charge of energy monitoring and management of the energy consumption and energy efficiency (i.e. EMF, if any) as listed in Table 6.12.2.2-1 to Table 6.12.2.2-4.

Editor's note:	Whether there is a need for a new 5GC NF such as EMF or whether proposed functionality can be provided by existing NFs (e.g. NWDAF) will be evaluated during evaluation and conclusion phase, and need to consider the progress of KI#1.

Editor's note:	How EMF collects data from other entities is FFS.

Table 6.12.2.2-1: Data collected by NWDAF for energy related analytics from OAM

| Information | Source | Description |
| --- | --- | --- |
| Time stamp | OAM | A time stamp associated to the data provided by the OAM. |
| Energy Consumption at 5GC, NF, Network Slice, gNB or NG-RAN level. | OAM | Energy consumption (EC) at various granularities, according to output analytics to be derived based on consumer request. The EC can be collected from OAM or EMF (if available). For the Energy Consumption parameters collected from OAM: - 5GC Energy Consumption is obtained by summing up the Energy Consumption of all the Network Functions (ECNF) that compose the 5G core network as defined in clause 6.7.3.2 of TS 28.554 [6]. - NF energy EC is the sum of the energy consumption of PNF(s) and/or VNF(s) which compose the NF, as defined in clause 6.7.3.1 of TS 28.554 [6]. - Network Slice Energy Consumption is obtained by summing up the Energy Consumption of all the Network Functions (ECNF) that compose the network slice as defined in clause 6.7.3.3 of TS 28.554 [6]. - The Energy Consumption (EC) of the gNB is obtained by summing up the Energy Consumption of all the Network Functions (NF) that constitute the gNB, as defined in clause 6.7.3.4.2 of TS 23.554. - Energy Consumption of the NG-RAN is obtained by summing up the Energy Consumption of all the gNB that constitute the NG-RAN, as defined in clause 6.7.3.4 of TS 28.554 [6]. |
| Energy efficiency at 5GC, NG-RAN or network slice levels | OAM | Energy Efficiency (EE) at various granularities, according to output analytics to be derived based on consumer request. For the Energy Efficiency parameters collected from OAM: - The energy efficiency of 5GC (EE5GC,UO,UP,DV) as defined in clause 6.7.4 of TS 28.554 [6]. - The energy efficiency of NG-RAN (EEMN,DV) as defined in clause 6.7.1.1 of TS 28.554 [6]. - The energy efficiency of network slice as defined in clause 6.7.2 of TS 28.554 [6]. |
| Datacenter energy efficiency ratio | OAM | OAM or Datacenter energy efficiency, e.g. it can be Site Energy Efficiency (SEE) in ETSI ES 203 228 V1.4.1 (2022-04) or the PUE of the datacenter hosting the NF(s) (see ETSI TS 105 174-2-2). |

The NWDAF may also collect data from MDAF. For example, the EnergyEfficiencyProblemType provided by MDAF can help NWDAF with identifying and verifying the derived prediction or statistics of the energy efficiency, i.e. at NF level.

Table 6.12.2.2-2: Input data collected from OAM (MDAF)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | MDAF | A time stamp associated with the collected information. |
| EnergyEfficiencyProblemType | MDAF | Indication of type of the energy efficiency issues. The allowed value is one of the enumerated values: HighEnergyConsumption, LowEenergyEfficiency, Other, Unknown, as defined in clause 8.4.4 of TS 28.104 [16]. |
| EnergyEfficiencyProblematicObject | MDAF | Indication of NR cells or NFs where the energy efficiency issues occurred or potentially occur, as defined in clause 8.4.4 of TS 28.104 [16]. |

In order to determine the EC and EE, in particular the EC and EE at relatively finer granularities (i.e. UE level, PDU session, and QoS flow level), the NWDAF also needs to collect data from other NFs. For example, based on the principle of computing the EC of UE, PDU session or QoS level, the NWDAF needs to determine or estimate the percentage of resources used by the UE, PDU session or QoS level among the overall usage. For example, by acknowledging the number of UEs for a SMF or a slice and the EC of the SMF or slice, the NWDAF is able to determine the average UE level EC. By acknowledging the traffic volume of the UE and that of the associated SMF, and the EC of the SMF, the NWDAF is able to determine the EC of this specific UE. Using similar methods, the NWDAF is able to derive the prediction and statistics of the EC and EE at finer granularity by collecting the informative data.

Table 6.12.2.1-3: Data collected by NWDAF for energy related analytics from NFs

| Information | Source | Description |
| --- | --- | --- |
| Time stamp | UPF, SMF, or AF | A time stamp associated to the data provided by the NFs. |
| UL/DL/overall data volume | UPF, SMF, or AF | UL/DL/overall (UL and DL) data volume of a slice, a UE, a PDU session, Qos flow, UPF, RAN node, 5GC, according to the consumer request. Could be Volume Measurement and/or Throughput Measurement. |
| NF Load information | NRF | Load per NF |
| NF status | NRF | The status of a specific NF instance(s) (registered, suspended, undiscoverable) as defined per TS 29.510 [15]. |
| NF resource usage | OAM | The usage of assigned virtual resources currently in use for specific NF instance(s) (mean usage of virtual CPU, memory, disk) as defined in clause 5.7 of TS 28.552 [10]. |
| NF resource configuration | OAM | The life cycle changes of specific NF resources (e.g. NF operational or interrupted during virtual/physical resources reconfiguration) |
| NF energy state | NRF or NF | Current NF energy state or NF energy state schedule/planning. |
| NF energy consumption estimate | OAM | NF energy consumption estimation (including IT and non-IT resources). This can be a combination of the estimated energy consumption of the NF instance(s) (considering only IT equipment) with the energy efficiency ratio (e.g. PUE) of the datacenter hosting the NF instance(s). The NF energy consumption (considering only IT equipment) can be estimated based on one of the proposed methods in clause 4.1 of TR 28.913 or clauses 6.7.3.1.4, 6.7.3.1.5, 6.7.3.1.6 and 6.7.3.1.7 of TS 28.554 [6]. (NOTE 1) |
| number of QoS flows | OAM, SMF, UPF | The Max/mean number of Qos flows. The measurement can be split into per S-NSSAI at the SMF as defined in clause 5.3.2.1 of TS 28.552 [10]. Or the number of the QoS flow per PDU session at the SMF collected from SMF. |
| number of PDU sessions | OAM, SMF, UPF | The Max/mean number of PDU sessions established by SMF. The measurement can be split into per S-NSSAI as defined in clause 5.3.1 of TS 28.552 [10]. Or the number (active) PDU sessions collected from SMF. |
| Number of UEs | OAM, AMF | The Max/mean number of UEs register to AMF collected from AMF or the number of subscribers per slice as defined in clause 5.2.1 of TS 28.552 [10] collected from OAM. |

In order to derive the output analytics of the renewable energy related information, the NWDAF needs collected the ratio of renewable energy in the total energy supply and the Coefficient of carbon from OAM.

Table 6.12.2.1-4: Input service data related to renewable energy

| Information | Source | Description |
| --- | --- | --- |
| ratio of renew energy in the total energy supply | OAM | The ratio of renew energy in the total energy supply |
| Coefficient of carbon or carbon emission | OAM | Coefficient of carbon is Used to derive the carbon emission, i.e. the statistically observed grams per mill watthour for any use of energy in the country or region where the service is provided. |
| Datacenter Carbon Emission Effectiveness KPI (KPICEE) | OAM | Ratio of CO2eq to the energy consumption by the datacenter hosting the NF, e.g. it can be calculated based on ETSI GS OEU 020 V1.1.1, clause 4.1.2. |

Editor's note:	Coordination with SA WG5 is needed regarding network energy related data collection from OAM.

Editor's note:	Data collected by NWDAF for energy related analytics needs further justification.

********** Next Change **********

#### 6.12.2.3	Energy Related Output Analytics

According to the consumer request, the NWDAF determines the prediction and statistics of energy consumption and energy efficiency along with other corresponding information at different granularities using the input data.

Table 6.12.2.3-1: Statistics of the output analytics

| Information | Description |
| --- | --- |
| List of energy-related information(1…max) | Observed statistics during the Analytics target period in the consumer request. The energy-related information includes statistics of maximum, minimum, variance or average value of energy consumption and energy efficiency at the consumer required granularity over the time slot(s). The energy consumption includes the EC of renewable energy. |
| > Time slot entry (1..max) | List of time slots within the Analytics target period. The time slots of within which the statistics of energy-related information is provided. |
| >>Time slot start | Time slot start within the Analytics target period. |
| >>Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| >> Identifiers (i.e. S-NSSAI/ NSI ID, NF ID/ NF Set ID, gNB ID, , PDU session ID, QFI, UE ID(s) etc.) | Identifiers of slice or slice instance/NF instance or NF set/ RAN node/5GC/PDU session/QoS flow/UE that correspond to the EE, EC and the Carbon emission derived by the NWDAF, based on consumer request. |
| >> energy consumption at required granularities (NOTE 1) | The maximum/ minimum / average / variance value of EC at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, or of a requested AOI. This information can be a value (e.g. in Wh) or a class (e.g. low, medium, high). (NOTE 2) |
| >> energy consumption of renewable energy at required granularities (NOTE 1) | The maximum/ minimum / average / variance value of renewable energy consumption at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, or of a requested AOI. |
| >> Carbon emission at required granularities (NOTE 1) | The maximum/ minimum / average / variance of Carbon emission at the consumer required granularity over a period of time, of a requested AOI, or of a datacentre hosting the NF. |
| >> energy efficiency at required granularities (NOTE 1) | The maximum/ minimum / average/ variance value of energy efficiency at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, of a requested AOI, or of a datacentre hosting the NF. |
| >> NF energy state history | Indicates the statistics of energy state of relevant NF(s). The information might also include information on the time/schedule when the NF instance was observed to be in a specific energy state. |
| >> traffic volume (NOTE 1) | The traffic volume used to derive the statistics of the energy related information at the required granularity, i.e. the energy consumption or energy efficiency. |
| >> traffic/packet rate (NOTE 1) | The traffic/packet rate associated to the statistics of the energy related information at the required granularity or the traffic/packet rate used to derive the EE and EC. |
| >> traffic latency/ packet delay (NOTE 1) | The traffic latency/ packet delay associated to the statistics of the energy related information at the required granularity or the traffic/packet rate used to derive the EE and EC. |
| >> Datacenter Carbon Emission Effectiveness KPI (KPICEE) | Indicates the statistics of ratio of CO2eq to the energy consumption by the datacenter hosting the NF. |
| >> Datacenter energy efficiency ratio (1..max) | Indicates the statistics of Datacenter energy efficiency, e.g. it can be the PUE of the datacenter hosting the NF(s) (see ETSI TS 105 174-2-2). |
| >> Analytics energy consumption (NOTE 1) | Indicates the observed energy consumptions derived for this analytics request/subscription. |
| >> Spatial Validity | Area where the analytics applies. If a Spatial granularity size was provided in the request or subscription, the number of elements of the list is smaller than or equal to the Spatial granularity size. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 2: The operator can define the range of energy consumption corresponding to each class of energy. |  |

Table 6.12.2.3-2: Predictions of the output analytics

| Information | Description |
| --- | --- |
| List of energy-related information(1…max) | Predictions during the Analytics target period in the consumer request. The energy-related information includes predictions of maximum, minimum, variance or average value of energy consumption and energy efficiency at the consumer required granularity over the time slot(s). The energy consumption includes the EC of renewable energy. |
| > Time slot entry (1..max) | List of time slots within the Analytics target period. The time slots of within which the predictions of energy-related information is provided. |
| >>Time slot start | Time slot start within the Analytics target period. |
| >>Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| >> Identifiers (i.e. S-NSSAI/ NSI ID, NF ID/ NF Set ID, gNB ID, PLMN ID, PDU session ID, QFI, UE ID(s) etc.) | Identifiers of slice or slice instance/NF instance or NF set/ RAN node/5GC/PDU session/QoS flow/UE that correspond to the EE, EC and the Carbon emission derived by the NWDAF, based on consumer request. |
| >> energy consumption at required granularities (NOTE 1) | The maximum/ minimum / average / variance value of EC at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, or of a requested AOI. This information can be a value (e.g. in Wh) or a class (e.g. low, medium, high). (NOTE 2) |
| >> energy consumption of renewable energy at required granularities (NOTE 1) | The maximum/ minimum / average / variance value of renewable energy consumption at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, or of a requested AOI. |
| >> Carbon emission at required granularities (NOTE 1) | The maximum/ minimum / average / variance of Carbon emission at the consumer required granularity over a period of time, of a requested AOI, or of a datacentre hosting the NF. |
| >> energy efficiency at required granularities (NOTE 1) | The maximum/ minimum / average/ variance value of energy efficiency at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, of a requested AOI, or of a datacentre hosting the NF. |
| >> NF energy state planning | Indicates the prediction of energy state of relevant NF(s). The information might also include information on the time/schedule when the NF instance is predicted to be in a specific energy state. |
| >> traffic volume (NOTE 1) | The traffic volume used to derive the predictions of the energy related information at the required granularity, e.g. the energy consumption or energy efficiency. |
| >> traffic/packet rate (NOTE 1) | The traffic/packet rate associated to the predictions of the energy related information at the required granularity or the traffic/packet rate used to derive the EE and EC. |
| >> traffic latency/ packet delay (NOTE 1) | The traffic latency/ packet delay associated to the predictions of the energy related information at the required granularity or the traffic/packet rate used to derive the EE and EC. |
| >> Datacenter Carbon Emission Effectiveness KPI (KPICEE) | Indicates the prediction of ratio of CO2eq to the energy consumption by the datacenter hosting the NF. |
| >> Datacenter energy efficiency ratio (1..max) | Indicates the prediction of Datacenter energy efficiency, e.g. it can be the PUE of the datacenter hosting the NF(s) (see ETSI TS 105 174-2-2). |
| >> Analytics energy consumption (NOTE 1) | Indicates the estimated energy consumptions derived for this analytics request/subscription. |
| >> Spatial Validity | Area where the analytics applies. If a Spatial granularity size was provided in the request or subscription, the number of elements of the list is smaller than or equal to the Spatial granularity size. |
| Confidence | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 2: The operator can define the range of energy consumption corresponding to each class of energy. |  |

Editor's note:	Whether the analytics of energy consumption, energy efficiency and Carbon emission at NF level and or slice level or those of a datacentre will be provided by NWDAF or OAM is FFS and may require coordination with SA WG5.

Based on energy consumption information,

-	If the consumer is AF, the AF may decide the energy related limits for a service.

-	If the consumer is PCF, the PCF may make policy decision and adjustments.

-	If the consumer is OAM, the OAM may configure the NF to enter an energy-saving mode.

-	If the consumer is AMF, the AMF may perform cell-based paging and provides a list of recommended cells.

-	If the consumer is SMF, the SMF may select a suitable UPF for the UE based on the energy consumption information.

-	If the consumer is CHF, the CHF may record the energy consumption information for charging.

-	The energy consumption of generating analytics can be used by the consumer NF to determine whether to continue subscribing the analytics.

-	The consumer of NWDAF based Energy analytics can use the analytics to define time-varying reporting periodicity (e.g. for event exposure), if supported by the producer NF.

Editor's note:	The details of whether and how the analytics of EE can be provided at NF service level is FFS.

Editor's note:	What the consumer will do upon reception of energy related analytics provided by NWDAF is FFS.

********** End of Changes **********