3GPP TSG-SA WG2#162	S2-2404339

Changsha, CN, April 15 – 19, 2024	(Revision of S2-240xxxx)

Source:	Sony

Title:	KI#2, New Sol.: AIoT Registration/Onboarding

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This contribution proposed a solution for AIoT registration/onboarding.

# 1.	Discussion

This contribution proposed a solution for AIoT registration/Onboarding.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-13:

# >>>>BEGINNING OF CHANGES<<<<

## 6.X	Solution #X: AIoT Registration/Onboarding

### 6.X.1	Description

This solution address KI#2. The basic principle in 5GS is that the 5GS triggers the registration/onboarding of the Ambient IoT and provision the Ambient IoT with parameters needed to act as an Ambient IoT in the network.

The solution is based on the following assumptions:

-	The AIoT device is not capable to initiate the registration procedure. The Network will trigger AIoT devices to register.

-	The AIoT reader (UE or Base station) periodically or on-demand broadcasts a message, Registration poll message, that triggers nearby unregistered AIoT device(s) to respond. The Registration poll message is assumed to be non-AIoT device specific to allow all unregistered AIoT device(s) to respond in an area.

-	An AIoT device does not respond to the Registration poll message if it is already registered in the network or the PLMN is not allowed.

-	The AIoT device responds with a Registration Request that includes e.g., device owner ID and concealed device ID. This will allow the CN NF to retrieve further information to proceed with the registration/onboarding of the AIoT device. If the CN NF was not able to retrieve the required information or the AIoT device was not possible to authenticate and authorize, the AIoT device will be rejected to register in the PLMN and shall retain from trying to register again in this PLMN.

-	During the registration procedure the CN NF will provision the AIoT device with e.g., security parameters, initial TempID or parameters to derive the initial Temp ID i.e., parameters for the Temp ID generation algorithm as discussed in clause 6.1.

### 6.X.2	Procedures

CN NF is used to name the network function that performs the necessary task to register/onboard an AIoT device to the network. The CN NF will be defined together with the system architecture design to support the Ambient IoT in 5GC.

Figure 6.x.2-1: Registration/onbording procedure for an AIoT device.

- It is assumed that the AIoT device is pre-provisioned by the device owner with information to be used when register/onboarding to a network. The information includes a device unique identifier (unique at owner level, not necessary globally unique), device owner ID and security IE. The AIoT may also have a list of preferred PLMN ID and/or list of non-allowed PLMN ID.

It is assumed that the device owner and the MNO has an SLA. A device owner AF may have provisioned the network with information needed to onboard an AIoT device(s) or a URL or FQDN to establishing IP connection to an AF that holds the device unique onboarding information.

- The AIoT reader (UE or base station) broadcasts a Registration poll message that includes e.g. the PLMN ID. The broadcast can either be an immediate broadcast triggered by CN NF or a scheduled/periodic broadcast configured by the CN NF.

Editor’s Note: The format of the Registration poll message needs to be defined in cooperation with RAN WG.

- An AIoT device that is not registered in the network responds with a Registration Request (device owner ID, concealed device ID). Before sending the Registration Request, the AIoT device may check the PLMN ID to a list of preferred PLMN IDs stored in the device. The Reader forwards the Registration request message to the CN NF.

Editor’s Note: How the concealed device unique ID is created will be defined by SA3.

- The CN NF may hold AIoT device owner information or can retrieve AIoT device owner information from the UDM/UDR using the received device owner ID. It is assumed at this stage that the CN NF may not be able to retrieve device unique information to complete the registration/onboarding.

- The CN NF establishes a secure IP connection with the AF to retrieve the device unique onboarding information by providing the concealed device ID to the AF. The AF must be able to un-conceal the device ID and verify that the device ID is part of the owner’s inventory list. The AF provides the CN NF with authentication/authorization result and necessary device security information.

- In case the AF have provisioned the network with device unique onboarding information in step 0, then the CN NF uses the retrieved information from UDM/UDR to authenticate and authorize the AIoT device.

- The CN NF sends a Registration response (Accept/Reject, security parameters). The AIoT device uses the security parameters together with the pre-provisioned security IE to e.g., derive an initial Temp ID. Additional AIoT configuration can be provision at this step.

.

### 6.X.3	Impacts on services, entities and interfaces

Impacts on existing entities:

CN NF:

- Configure/trigger the Reader to broadcast Registration poll message.

- Retrieve device owner information and device unique onboarding information.

- Accept or reject the AIoT device registration/onboarding request.

AIoT device:

- Receive a Registration poll message.

- Transmit a Registration Request message to the Reader.

- Receive the Registration accept message with configuration parameters.

AIoT Reader (UE or Base station):

- Broadcast Registration poll (on-demand or periodically configured).

- Transparently forward any messages between the AIoT device and the CN NF.

UDM/UDR

- Data management of device owner information.

- Be provisioned with device owner information.

# >>>>END OF CHANGES<<<<